3GPP TSG-RAN WG4 Meeting #115			                                R4-2505751

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.14.4.2

Source:	Xiaomi

Title:	Discussion on TRP and TRS requirements for Size 2 UEs

Document for:	Discussion

- Introduction

RAN4 has discussed the TRP/TRS requirements for size 2 UEs for several meetings, and there is no progress yet. In this contribution, we provide our views on these remaining issues.

Specify TRP TRS requirements: - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): [30] - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-19. - Method of limits derivation: per-band Data driven approach, working procedure in Rel-18 on how to specify the requirement is baseline. - The value at [80%] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion - For a band supporting both PC2 and PC3, specify PC3 requirement based on finalized PC2 requirements, for TDD bands 3dB offset, for FDD bands TBD. - How to specify PDA hand related TRP/TRS requirements (Size2) in legacy band of Rel-18: FFS.

- Discussion

In last RAN4 meeting, we have had extensive discussion on TRP/TRS requirement for size 2 UEs during 1st round and 2nd round adhoc discussion. And the tentative agreements were to adopt 0dB offset between Size 1 and Size 2 device for TRp/TRS requirements for the same band as a general rule for release 19 and forwards.

Tentative Agreements for 2nd ad-hoc： RAN4 adopt 0dB offset between Size 1 and Size 2 device TRP/TRS requirements for the same band as a general rule, for release 19 and forwards. [FFS whether defined requirements in Rel-18 can apply.]

Regarding the FFS part, whether the defined requirements in Rel-18 can apply or not. Based on our internal evaluation, we do not see big scale delta in TRP/TRS performance between Size 1 and Size 2 devices for the same band. Thus, we think the defined requirements in Rel-18 can also apply the 0dB offset.

Proposal 1: RAN4 adopt 0dB offset between Size 1 and Size 2 device TRP/TRS requirements for the same band as a general rule.

3	Conclusions

In this paper, we provided our views on TRP/TRS requirements for size 2 UEs and our proposal as follows:

Proposal 1: RAN4 adopt 0dB offset between Size 1 and Size 2 device TRP/TRS requirements for the same band as a general rule.

4 References

[1] R4-2504763, WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3, Vivo