3GPP TSG-RAN WG4 Meeting #115																			R4-2505753

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on AIoT system parameters

Agenda item:	7.26.2

Document for:	Discussion

- Introduction

For the system parameters, there’re good progress in RAN4#114b [1]. D2R BW and channel raster are still open. This contribution provides our further analysis for these two issues.

- Discussion

  - D2R bandwidth

For D2R BW, the understandings in RAN4 are still not aligned. The followings should be considered according to WF [1],

- Consider SFO impact

- Consider small frequency shift defined in RAN1

- D2R transmission bandwidth (2 times of 1SB), consider bit rate defined in RAN1

Figure 1: D2R CBW copied from WF [1]

For the aspects related to D2R CBW, it seems at least the TBW understanding is aligned in companies. In TR 38.769, the definition is as following,

Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device.

In RAN1, the following is the common understanding [2],

For D2R transmission, the following parameters and relationships are considered:

- Tb: Time duration corresponding to a bit, where the bit is that after FEC if applied.

- Tc: Chip duration, where Tb = Tc×2R.

- R: SFS factor, where the small frequency shift location is determined by ± (R/Tb = 1/2/Tc) Hz.

- Btx,D2R: D2R transmission bandwidth, where Btx,D2R = 4/Tb = 2/(R*Tc).

So at least D2R transmission bandwidth can be defined in RAN4 to clarify the resources used to transmit D2R signal.

Proposal 1: Define D2R transmission BW in RAN4 as following,

Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device.

For the values of Btx,D2R, according to the equation Btx,D2R = 4/Tb and the potential values agreed in RAN1#120b [3],

Agreement

The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table:

- FFS further down-selections of Tchip, Tb, and R to be supported

- Note: Detailed indication signaling of D2R scheduling information is discussed in AI 9.4.4.

|  |  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
|  | 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
|  | 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
|  | 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
|  | 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
|  | 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
|  | 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
|  | 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
|  | 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
|  | 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
|  | 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
|  | 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
|  | 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
|  | 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
|  | 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |
| Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Then the potential D2R TBWs are as following,

Table 1: Potential D2R TBWs

| Tb (us) | 266.67 | 133.33 | 66.67 | 33.33 | 22.22 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBW (KHz) | 15 | 30 | 60 | 120 | 180 | 240 | 360 | 480 | 720 | 960 | 1440 | 1920 | 2880 | 3840 |

Proposal 2: D2R TBW are defined as the following values pending RAN1 further down selection,

15, 30,	60, 120, 180, 240, 360, 480, 720, 960, 1440, 1920, 2880, 3840 kHz.

When the TBW is defined, if CBW needs to be defined need some discussion. The essential issue is how to understand the CBW concept for AIoT device. There were two understandings according to last RAN4 meeting input. One is that CBW is only related to TBW, the other is that SFS should also be taken into account. The later one is captured for information in the WF. The first understanding may miss the SFS factor, but SFS factor is implemented by repeating and the frequency resources inside the frequency shift still can be used if they’re not occupied by the TBW. So one possible way is that Tb, Tchip and R relationship is captured to RAN4 specification to show the SFS factor. If the CBW concept is not defined clearly, it brings too much misleading if FDMA is used.

There is another aspect that SFO brings some uncertainty to the occupied BW, if/how it should be captured in the spec. One way is that if SFO requirement is defined in spec, then the SFO impact to the SFS and occupied BW is implicitly included. So the network can schedule the devices in FDMA mode considering SFO performance. Therefore, in our understanding, there may not need to capture the SFO impact to the BW discussion if no CBW is defined in spec.

Proposal 3: RAN4 do not define CBW for AIoT device and reader side. Capture the Tb, Tchip and R relationship to RAN4 specification to show the SFS factor. SFO impact is implicitly included in SFO requirement.

  - Channel raster

The channel raster may need to analysis D2R and R2D separately then to make the final decision. For R2D, it’s simple that the frequency resource is 1, 2, 3, 4 PRB, then the decision is straightforward that NR enhanced channel raster is reused to have the flexibility. For D2R, it’s not necessary to have orthogonality consideration because the D2R signal is simple time domain signal based on chip, so in theory any choice doesn’t matter. However, considering the spectrum usage, align with NR 10kHz raster can make the channel center frequency more flexible. So we support the 10kHz channel raster option.

Proposal 4: 10 KHz channel raster is used for AIoT system.

- Conclusion

This contribution provides our views on the remaining issues of AIoT system parameters. We have the following proposals and observations.

For D2R bandwidth,

Proposal 1: Define D2R transmission BW in RAN4 as following,

Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device.

Proposal 2: D2R TBW are defined as the following values pending RAN1 further down selection,

15, 30,	60, 120, 180, 240, 360, 480, 720, 960, 1440, 1920, 2880, 3840 kHz.

Proposal 3: RAN4 do not define CBW for AIoT device and reader side. Capture the Tb, Tchip and R relationship to RAN4 specification to show the SFS factor. SFO impact is implicitly included in SFO requirement.

For channel raster,

Proposal 4: 10 KHz channel raster is used for AIoT system.

- References

[1] R4-2505230, “WF on A-IoT device requirements”, CMCC, RAN4#114b.

[2] R1-2503021, “Summary #2 for coding aspects of physical channel design”, Moderator (CMCC), RAN1#120b

[3] R1-2503112, “Session notes for 9.4 Solutions for Ambient IoT in NR”, Ad-Hoc Chair (Huawei), RAN1#120b