3GPP TSG-RAN WG4 Meeting #115				                                                                            R4-2505757

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on ModifiedMPR-Behavior and spec

Agenda item:	4.5.2

Document for:	Discussion

- Introduction

ModifiedMPR-Behavior was discussed in the previous two meetings, but it seems there’s no common understandings between companies. This contribution provides our understanding of ModifiedMPR-Behavior and how to handle the spec. We provide some general rules and the proposals to clean the spec.

- Discussion

  - General approach

The ModifiedMPR-Behavior concept was originally introduced when some new regulation needs to be met then new emission requirements must be met. However, although regulation in theory should be met in a timeline, but new AMPR requirement impact the product under developing. So the ModifiedMPR-Behavior was invented that some time delay is allowed for the earlier release that the corresponding bit MAY be set to zero, but the spec under working in the latest release is defined as SHALL be set to 1. In theory, all of the ModifiedMPR-Behavior bits shall be set to 1 finally. However, it seems this concept is extended and the situation is complicated now.

After reviewing all of the bits in TS 38.101-1-j10 Annex L, there’re several kinds of ModifiedMPR-Behavior setting. For example, legacy requirements are replaced by new requirements, different form factors use different requirements, etc. Some of the requirements were changed twice, thus more misleading was introduced.

Observation 1: In the current spec, ModifiedMPR-Behavior bits are used due to two reasons, i.e. legacy requirements are replaced by new requirements, different form factors use different requirements.

For the case of new requirements replacing legacy requirements, our understanding is that new requirements should be mandatory for the UE from the release in which the new requirements are defined because usually the requirements are from regulation or the performance is improved.

Proposal 1: For the same NS, the latest requirement should be mandatory for the UE from the release in which the requirements are defined.

Some of the requirements are changed twice, but different approach is used. Some band uses separate bits, for example,

| n96 | 0 (leftmost bit) | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 | This bit may be set to 1 by a Rel-16 UE to indicate support of all network signalling labels for n96 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 |
| --- | --- | --- | --- |
|  | 1 | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 | This bit may be set to 1 by a Rel-16 and Rel-17 UE to indicate support of all network signalling labels for n96 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 |
| n102 | 0 (leftmost bit) | Support of all band n102 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 | This bit may be set to 1 by a Rel-16 UE to indicate support of all network signalling labels for n102 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 |
|  | 1 | Support of all band n102 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 | This bit may be set to 1 by a Rel-16 and Rel-17 UE to indicate support of all network signalling labels for n102 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 |

Some band uses the same bit but the bit has different meaning for different release.

| n104 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| --- | --- | --- | --- |

Observation 2: Some of the requirements were changed twice but with different methodologies for ModifiedMPR-Behavior bit.

Our understanding is that a separate bit for the second time requirement change is clearer.

Proposal 2: For the requirements which are changed the second time, a separate ModifiedMPR-Behavior bit is used for the second time change.

For the requirements for different formfactors, current spec uses the same bit for FWA and handheld UE. Taking n104 as the example, the different formfactors use the same bit, which makes the spec not easy to be understood especially the handheld UE requirements are also changed. So we think different formfactor can use a separate bit.

Proposal 3: For the requirement for a different formfactor, a separate ModifiedMPR-Behavior bit is used.

  - Spec handling

For the spec handling, if there will be some agreement that cleanup is needed, we may need to be careful to avoid the impact on the product under development. The good approach may be only the latest spec is cleaned.

Proposal 4: For the existing modifiedMPR-Behaviour bits, R19 spec is cleaned and earlier specs are not touched.

There may need some discussion for the separate bits approach, if new separate bit can be introduced for R19 spec. From spec perspective, it may bring benefit, but from product perspective, the views from UE vendors may be needed. We think it’s ok for R19 spec, if new approach is approved.

Proposal 5: If the separate bit general rules are accepted by the group, R19 spec can be changed to add new bits if it’s needed.

- Conclusion

This contribution provides our understanding of ModifiedMPR-Behavior and how to handle the spec. We provide some general rules and the proposals to clean the spec.

For general approach,

Observation 1: In the current spec, ModifiedMPR-Behavior bits are used due to two reasons, i.e legacy requirements are replaced by new requirements, different form factors use different requirements.

Proposal 1: For the same NS, the latest requirement should be mandatory for the UE from the release in which the requirements are defined.

Observation 2: Some of the requirements were changed twice but with different methodologies for ModifiedMPR-Behavior bit.

Proposal 2: For the requirements which are changed the second time, a separate ModifiedMPR-Behavior bit is used for the second time change.

Proposal 3: For the requirement for a different formfactor, a separate ModifiedMPR-Behavior bit is used.

For spec handling,

Proposal 4: For the existing modifiedMPR-Behaviour bits, R19 spec is cleaned and earlier specs are not touched.

Proposal 5: If the separate bit general rules are accepted by the group, R19 spec can be changed to add new bits if it’s needed.

- References

[1] R4-2504558, “(PHS_System_NRonly) On the requirements of modifiedMPR-Behavior”, Huawei, HiSilicon, RAN4#114bis

[2] R4-2504569, “A-MPR regions for NS_05/NS_05U and optionality in earlier releases by modified MPR behaviour”, Ericsson, RAN4#114bis

[3] R4-2502236, “On the requirements of modifiedMPR-Behavior”, Huawei, HiSilicon, RAN4#114

[4] R4-2500360, “CR 38.101-1 R17 Correction for ModifiedMPR table [Modified_MPR] ”, Nokia, RAN4#114