3GPP TSG-RAN WG4 Meeting #115																			R4-2505759

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on PC2 RedCap RSD

Agenda item:	7.6.2

Document for:	Discussion

- Introduction

In the previous discussion for PC2 RedCap RSD discussion, 1Rx RSD is still open. Companies would like to find a general approach to define PC2 RedCap 1Rx RSD to save the future efforts. This contribution provides our thinking on this, a candidate approach is proposed.

- Discussion

Firstly, we reviewed the PC2 2Rx RSD compared with PC3 2Rx REFSENS. Table 1 is copied from TS 38.101-1, but only kept the RedCap CBW up to 20MHz.

Table 1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 3 | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz |
|  | (dB) | (dB) | (dB) | (dB) | (dB) |
| n1 |  | 0 | 0 | 0 | 0 |
| n2 |  | 0.8 | 0.9 | 1.1 | 1.2 |
| n3 |  | 0.5 | 0.5 | 0.5 | 0.5 |
| n7 |  | 0.5 | 0.5 | 0.5 | 0.5 |
| n8 |  | 0.5 | 0.7 | 0.8 | 2.3 |
| n13 |  | 0.8 | 0.9 |  |  |
| n14 |  | 0.6 | 0.8 |  |  |
| n25 |  | 0.8 | 0.8 | 0.9 | 1.1 |
| n28 | 0.6 | 0.6 | 0.7 | 0.8 | 1.3 |
| n66 |  | 0 | 0 | 0 | 0 |
| n71 |  | 0.5 | 0.9 | 0.9 | 2.2 |
| n70 |  | 0 | 0 | 0 | 0 |
| n85 | 0.6 | 0.6 | 1 | 2.6 |  |

From table 1, we can see some information. For the bands, if Tx noise is low, RSD is zero. For some bands/CBW, the Tx noise is relatively flat in the Rx band, the RSD is close for different CBW. If there’re CIM3/CIM5/CIM7 problem, the RSD is much larger than the flat case.

Observation 1: For the bands, if Tx noise is low, PC2 2Rx RSD is zero. For some bands/CBW, the Tx noise is relatively flat in the Rx band, the RSD is close for different CBW. If there’re CIM3/CIM5/CIM7 problem, the RSD is much larger than the flat case.

We can also check the PC2 REFSENS scaling to 5MHz REFSENS as shown in Table 2.

Table 2: PC2 2Rx REFSENS scaling to 5MHz

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 | 5 | 10 | 15 | 20 |
|  |  | MHz | MHz | MHz | MHz | MHz |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |
| BW (MHz) |  | 3 | 5 | 10 | 15 | 20 |
| BW scale to 5MHz | Scale to 5MHz | -2.2 | 0.0 | 3.0 | 4.8 | 6.0 |
| n1 | 15 |  | -100 | -96.8 | -95 | -93.8 |
|  | Scale to 5MHz REF |  |  | 3.2 | 5 | 6.2 |
| n2 | 15 |  | -97.2 | -93.9 | -91.9 | -90.6 |
|  | Scale to 5MHz REF |  |  | 3.3 | 5.3 | 6.6 |
| n3 | 15 |  | -96.5 | -93.3 | -91.5 | -90.3 |
|  | Scale to 5MHz REF |  |  | 3.2 | 5 | 6.2 |
| n71 | 15 |  | -97.5 | -94.3 | -92.5 | -91.3 |
|  |  |  |  | 3.2 | 5 | 6.2 |
| n8 | 15 |  | -96.5 | -93.1 | -90.6 | -83.5 |
|  |  |  |  | 3.4 | 5.9 | 13 |
| n13 | 15 |  | -96.2 | -92.9 |  |  |
|  |  |  |  | 3.3 |  |  |
| n14 | 15 |  | -96.4 | -93 |  |  |
|  |  |  |  | 3.4 |  |  |
| n25 | 15 |  | -95.7 | -92.5 | -90.6 | -89.2 |
|  |  |  |  | 3.2 | 5.1 | 6.5 |
| n28 | 15 | -99.6 | -97.9 | -94.8 | -92.7 | -89.5 |
|  |  | -1.7 |  | 3.1 | 5.2 | 8.4 |
| n66 | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 |
|  |  |  |  | 3.2 | 5 | 6.2 |
| n70 | 15 |  | -100 | -96.8 | -95 | -93.8 |
|  |  |  |  | 3.2 | 5 | 6.2 |
| n71 | 15 (note 9) |  | -96.7 | -93.1 | -90.7 | -83.8 |
|  |  |  |  | 3.6 | 6 | 12.9 |
| n85 | 15 | -98.6 | -96.4 | -92.8 | -81.4 |  |
|  |  | -2.2 |  | 3.6 | 15 |  |

For the REFSENS scaling is >= 0.5 dB + theory CBW scaling, we marked them in orange. Comparing Table 1 and Table 2, we can find similar information, if the Tx noise is low or flat, then the scaling is near the theory CBW scaling, if there’s other Tx noise due to CIM or cross band ISO, then the scaling is much greater.

It seems PC2 2Rx RSD or the PC2 2Rx REFSENS scaling can be candidates to derive some general rules for PC2 1Rx RSD. After some deep thinking, PC2 2Rx RSD is more appropriate because the flat Tx noise is shown in the RSD but not in the REFSENS scaling.

Observation 2: PC2 2Rx RSD can be the reference to derive 1Rx RSD compared with 2Rx.

Diversity path Rx performance is usually better than primary Rx path, the following is a rough estimate on the MRC REFSENS and PRx/DRx REFSENS calculation.

Table 3: MRC REFSENS and PRx/DRx REFSENS calculation

| PRx RFFSENS | S | S | S | S |
| --- | --- | --- | --- | --- |
| DRx RFFSENS | S | S-1 | S-2 | S-3 |
| MRC RFFSENS | S-3 | S-3.5 | S-4.1 | S-4.8 |

So from the above table, it can be seen if DRx REFSENS is 3 dB better then PRx, the MRC REFSENS is 4.8 dB better than PRx REFSENS. Although in theory, the CIM noise on DRx path can be 10 or 6 dB smaller than PRx path, but usually DRx path implementation such as PCB routing, filter IL are not in high priority compared with PRx, so DRx path REFSENS is better but not at the level of 6 dB. Different products may have different performance. So it’s difficult to have some very accurate analysis for the gain of DRC REFSENS compared with PRx REFSENS. However, generally, if there’s severe Tx noise, PRx implementation is more difficult to control the noise. Therefore, we propose to still use PC2 2Rx RSD to derive PC2 1Rx RSD compared with PC2 2Rx REFSENS. We have the following proposal,

Table 4: PC2 1Rx RSD proposal

| PC2 2Rx RSD X (dB) | PC2 1Rx RSD compared with PC2 2Rx REFSENS (dB) |
| --- | --- |
| X = 0 | 3 |
| X <1 | 3.5 |
| 1≤X<2 | 4 |
| 3≤X<4 | 4.5 |

If the above direction can be agreed, it’s better to define the requirements to capture the final value not the rule. The rule can be captured in the WF.

Proposal: Use the following general rule to derive the PC2 1Rx RSD.

Table 4: PC2 1Rx RSD proposal

| PC2 2Rx RSD X (dB) | PC2 1Rx RSD compared with PC2 2Rx REFSENS (dB) |
| --- | --- |
| X = 0 | 3 |
| X <1 | 3.5 |
| 1≤X<2 | 4 |
| 3≤X<4 | 4.5 |

The requirements are captured as the final value, the rule is captured in the WF.

- Conclusion

This contribution provides our analysis for PC2 1Rx RedCap RSD, we have the following observations and proposals.

Observation 1: For the bands, if Tx noise is low, PC2 2Rx RSD is zero. If the Tx noise is relatively flat in the Rx band, the RSD is close for different CBW. If there’re CIM3/CIM5/CIM7 problem, the RSD is much greater than the flat case.

Observation 2: PC2 2Rx RSD can be the reference to derive 1Rx RSD compared with 2Rx.

Proposal: Use the following general rule to derive the PC2 1Rx RSD.

Table 4: PC2 1Rx RSD proposal

| PC2 2Rx RSD X (dB) | PC2 1Rx RSD compared with PC2 2Rx REFSENS (dB) |
| --- | --- |
| X = 0 | 3 |
| X <1 | 3.5 |
| 1≤X<2 | 4 |
| 3≤X<4 | 4.5 |

The requirements are captured as the final value, the rule is captured in the WF.

- References

[1] R4-2505231, “WF on PC2 RedCap UE RF requirements”, China Telecom, RAN4#114bis