# 3GPP TSG-RAN WG4 Meeting #115	R4-2505760

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:			7.10.3.3

Source: 	Xiaomi

Title: 						RF requirements for IoT-NTN

Document for:	Discussion

- Introduction

Rel-19 NTN agreed to introduce HPUE for NR-NTN/Iot-NTN operation:

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this contribution, views on remaining open issues were provided.

- Discussion

ACLR requirements

In last RAN4 meeting, initial conclusion was reached for co-existence study as following:

Issue 1-1-2: ACLR for eMTC based Iot-NTN for PC1/2; Agreement: o Option 1: Re-use PC3 eMTC based Iot-NTN ACLR 30dBc. o Option 2: 31dBc ACLR

On isolation distance, we prefer to align with previous assumption the deployment scenario and restriction shall be consistency independent to device power classes which belongs to UE capability.

On NTN operation, the critical issue is coverage due to long distance between satellite over the space to terminal. With reasonable ACLR, it will bring benefits to improve uplink coverage with less MPR and or A-MPR.

Based on above consideration, we suggest to specify following ACLR values:

Proposal 1: For PC1 eMTC based Iot-NTN HPUE ACLR can be determine as 31dBc

In TS 36.101, P-MPR isn't exclusive to handheld UEs. It's a power management mechanism ensuring the UE's transmit power meets electromagnetic radiation safety and regulations across various scenarios.

While P-MPR is common in handheld devices, especially when users hold them close to their heads or bodies due to SAR issue, it also applies to other UE types like Vehicle-mounted UEs and Fixed-installed UEs.

Thus, P-MPR is a general power management parameter for various UE devices, not just handheld ones.

Proposal 2: RAN4 should consider P-MPR for both handheld UEs and non-handheld UEs

- RSD (for full duplex eMTC based)

Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based) - Proposals - Reference sensitivity values - 256: - 0 dB (MTK, Samsung, Ericsson) - 1.1dB (Qualcomm) - 255: - 0.8dB (MTK) - 1.2dB (Qualcomm) - No differentiation of handheld and non-handheld for RSD - WF (Ad-hoc agreement): - Reference sensitivity values - 256: [0.5] dB - 255: 1 dB

For the PC2 1Tx RSD (full duplex eMTC based) calculation, we assumed 4dB FE loss, 54dB Rx IIP2 and 55dB Tx isolation. The RSD from PC3 to PC2 will be 1.2dB.

Proposal 3: Apply 1.2dB RSD for band 256 for PC2 1Tx RSD for the full duplex eMTC based IoT-NTN UE.

SAR Related issues

Issue 2-1-2: Whether and how to introduce duty cycle related method. - Proposals (Not necessarily mutually exclusive): - Scheme 1: (vivo) Consider to introduce a scheme that UE voluntarily dropping uplink transmissions by self-assessment to control the SAR. This may be simple enough, can be controlled by network and still useful. - Scheme 2: (oppo) UE indicate whether the following Tx time duration should be counted into the UL duty cycle statistics in addition to current maxUplinkdutycycle capability. How the information is reported is up to RAN2. - Scheme 3: (Samsung) Observation 3-1: Legacy dutycycle method does not fit the purpose of keeping as much power as possible for NTN scenario, as legacy solution requires UE to fall back 3/6 dBs, which is the power of entire power class. Observation 3-2: In Rel-19, NTN would not introduce any beam service ratio in time domain in NTN phase3 WI in parallel. Observation 4-1: To let UE drop certain ratio of uplink opportunities will face the same challenge as other duty cycle method, because there is no NW uplink ratio in time domain in a FDD NTN band. It means if we follow this way, there is no way we could specify or to guide UE behaviors, like how much ratio of uplink should UE drop, and for how long. Observation 4-2: To let UE go into a Tx UL Gap also faces same challenges as above. Proposal 4: To not define any duty cycle based SAR solution, and to use the baseline P-MPR method, considering following: 1) P-MPR had been working well and leave space for implementation; 2) without uplink duty ratio from NW side, current solutions on the table (including legacy tdd duty cycle solution) cannot be optimized and clear enough for UE implementation; 3) There are many non-3GPP technologies already implemented in actual SAR compliance, and it helped to maintain power level well enough. - Scheme 4: (Qualcomm) - A new UE triggered report is specified to report energy headroom to the network in form of “UE can transmit at maximum transmit power for X seconds out of next Y seconds.” - For X and Y, consider value range from 100ms to 5s with 100ms granularity. - Starting time for the indicated values is the transmission time of the report. - Send an LS to RAN2 to specify energy headroom reporting. - Allow but do not mandate UE to reduce it’s maximum output power when Tx duty cycle exceeds 50% in a radio frame for PC2 and 2Tx PC1.5. Capture this allowance into specification. - Further evaluate potential specification impact from filter power handling for PC1. - Scheme 5: (Ericsson, Sony, Nokia) Observation 1: “Duty-cycle” based method is not well designed for FDD bands and is allegedly not used in the field. P-MPR method is used instead, which is on another hand “blind” for the network. Observation 2: “Duty-cycle” based and the P-MPR based methods in combination with reporting of the actual power-capability state of a UE could be considered for NTN HPUEs, similar to Rel-18 enhancements introduced for TN. Proposal 1: Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value. Proposal 2: If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details: (Ericsson, Sony) - If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance: - DPC = ‘00’: the criteria to report DPC is not met - DPC = ‘01’: ΔPPowerClass = 0 dB - DPC = ‘10’: ΔPPowerClass = 3 dB - DPC = ‘11’: ΔPPowerClass = 6 dB - If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c): - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB - Scheme 6: (ViaSat, Inmarsat) Observation 4: In a non-terrestrial network, if the UE is using a UL duty cycle significant enough to exceed SAR limits, it is likely at the edge of coverage (using a low MCS with lots of parity), in which case, transmit power backoff is not a preferred option as it will almost invariably lead to radio link failure. Proposal 3: For NTN, extend the existing TN signalling mechanism for UL duty cycle reporting to inform the RAN that it is close to the SAR limit, then the RAN will reduce the number of UL slots allocated to the UE. When the SAR limit has fallen, the UE can signal the improved condition back to the RAN so that the UL slots are picked back up. - WF - Further discuss the proposed schemes related to duty cycle related SAR scheme. - Other schemes or no duty cycle related scheme are not precluded.

The P-MPR scheme remains preferable due to its mature standardization in 3GPP since Rel-15, ensuring direct NTN compatibility. Its implementation simplicity leverages UE-local sensor data (e.g., proximity/temperature) and autonomous power adjustments, eliminating network coordination needs—critical for NTN’s high-latency FDD environments. The framework supports dynamic power optimization (e.g., per-slot backoff) for coverage enhancement. Additionally, existing SAR/MPE testing standards (TS 37.145) already validate P-MPR, avoiding costly redefinition of test cases required by alternative approaches.

Proposal 4: To not define any duty cycle based SAR solution, and to use the baseline P-MPR method.

- Conclusion

In this contribution, views on remaining open issues were provided.

Proposal 1: For PC1 eMTC based Iot-NTN HPUE ACLR can be determine as 31dBc

Proposal 2: RAN4 should consider P-MPR for both handheld UEs and non-handheld UEs

Proposal 3: apply 1.2dB RSD for band 256 for PC2 1Tx RSD for the full duplex eMTC based IoT-NTN UE.

Proposal 4: To not define any duty cycle based SAR solution, and to use the baseline P-MPR method.

Reference

[1] R4-2505114, WF on HPUE UE requirement for IoT-NTN