# 3GPP TSG-RAN WG4 Meeting #115	R4-2505761

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:			7.10.3.2

Source: 	Xiaomi

Title: 						RF requirements for NR-NTN

Document for:	Approval

- Introduction

Rel-19 NTN agreed to introduce HPUE for NR-NTN operation for uplink coverage improvement.

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this contribution, views on remaining open issues were provided.

- Discussion

ACLR requirements

In last RAN4 meeting, initial conclusion reached for co-existence study as following:

Issue 1-7-2: PC1 ACLR Tentative agreement: - PC1 ACLR is [34] dBc - Companies to further evaluate this, and to be confirmed in next meeting.

On isolation distance, we prefer to align with previous assumption the deployment scenario and restriction shall be consistency independent to device power classes which belongs to UE capability.

For TN operation, following ACLR is specified based on co-existence:

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

On NTN operation, the critical issue is coverage due to long distance between satellite over the space to terminal. With reasonable ACLR, it will bring benefits to improve uplink coverage with less MPR and or A-MPR.

However, companies still have concern for the relaxation of ACLR of PC1 UE. And in the last meeting in Wuhan, compromise value of ACLR for 34dB which is the middle value of two original options.

Based on above consideration, we suggest to specify following ACLR values:

Proposal 1: Specify PC1 NR-NTN HPUE ACLR as 34 dBc

Target device types and device type assumption

In general, we prefer to leverage existing TN network design on FDD bands as much as possible with following thinking:

- Harmonize the common design between TN and NTN operation for echo-system

- Shared RF chain between NTN and TN operation on similar frequency range for handheld device for cost and form factor limitations

- Reduce RAN4 workload on leverage the conclusion from TN to NTN

Tentative agreement: - Define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19. - FFS on other cases if time is allowed. - [RAN4 recognizes PC1.5 for handheld NR-NTN UE with 2Tx is feasible.]

With above observations, our preference as following:

For PC1.5 device type and UE architecture assumption, only prioritize non-handheld device types with 2Tx, do not consider PC1.5 for handheld NR-NTN UE with 2Tx on this stage.

RSD:

Ad-hoc agreements: - For n255 PC2: 5MHz; 10MHz; 15 MHz; 20 MHz. RSD@1Tx; [0.8]; [0.8]; [0.9]; [0.9]. RSD@2Tx; [1.6]; [1.6]; [1.7]; [1.7].

For band n255, the -117dBm/Hz PA interference is assumed. At the same time, the 52dB duplexer isolation and 54dB receiver IIP2 are assumed. Based on these assumptions, the RSD evaluations for both 1Tx/2Rx and 2Tx/2Rx RF architectures are provided as below.

|  | Parameters |
| --- | --- |
| transmit power for n255, dBm | 26 |
| Tx band n255 BW(MHz, Lcrb) | 20 |
| RFFE loss, dB | 4 |
| Duplexer isolation at n255 uplink freq，dB | 52 |
| antenna isolation, dB | 10 |
| n255 Rx filter rejection at 1626.5~1660.5MHz, dB | 50 |
| Typical receiver IIP2, dB | 54 |
| front-end loss | 4 |
| noise figure dB | 9 |
| Rx band n255 BW(MHz, NRB) | 18 |
| n255 PA noise PSD at PA output port, dBm/Hz | -117 |
| n255 Tx filter rejection at 1525~1559MHz, dB | 50 |
| SNR requirement for QPSK | -1 |
| Implementation Margin, dB | 2 |
| RSD For 1 Tx | 1.44 |
| RSD For 2 Tx | 2.51 |

Proposal 3: To determine the 1 Tx RSD for band n255 as 1.44dB and 1 Tx RSD for band n255 as 2.5dB.

- Conclusion

Proposal 1: Specify PC1 NR-NTN HPUE ACLR as 34 dBc

Proposal 2: For PC1.5 device type and UE architecture assumption, only prioritize non-handheld device types with 2Tx, do not consider PC1.5 for handheld NR-NTN UE with 2Tx on this stage.

Proposal 3: To determine the 1 Tx RSD for band n255 as 1.44dB and 1 Tx RSD for band n255 as 2.5dB.

Reference

[1] R4-2505113 WF on NR NTN HPUE, Samsung