3GPP TSG-RAN WG4 Meeting #115                    	R4-2505762

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.11.3

Source:	Xiaomi

Title:	Temporary calibration results and simulation results for the Ku-band NTN coexistence study

Document for:	Discussion

Background

The General requirements of the Rel-19 NR NTN Ku band work item had been discussed in RAN4#114 and 114bis. The simulation scenarios will focus on the following scenarios.

| VSAT model SAN | 45 x 45 Phased Array (1st step for coexistence) | 60 cm Parabolic (1st step for coexistence) | 78 x 78 Phased Array |
| --- | --- | --- | --- |
| GEO | X | X | X |
| LEO 600km | X | X |  |

Meanwhile, basic on the analysis, the cases of each scenario narrow down as below.

| No | Aggressor | Victim | Frequency Band | Notes on the Ku Band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1 | VSAT UL | UE UL | 14 GHz | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 2 | gNodeB DL | VSAT UL | 14 GHz | gNodeB downlink interfering with Ku Band Satellite receiver. | The ACLR of the gNodeB is fixed. The ACS of the Satellite to be varied/defined. |
| 3 | gNodeB DL | Satellite DL | 11 GHz | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |
| 4 | Satellite DL | gNodeB DL | 11 GHz | Ku Band Satellite downlink interfering with UE receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the UE is fixed. |

In this document, Xiaomi provided the calibration results and the temporary simulation results for the above cases.

Calibration results NTN Ku band Coexistence study

The calibration provided couplingloss and SINR of the TN and NTN downlink and uplink.

Table 1: Calibration summary for NTN DL

| Calibration metrics | DL Coupling Loss |  |  | DL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CDF percentile | @5% | @50% | @95% | @5% | @50% | @95% |
| LEO-600(90deg Parabolic) | 93.17 | 101.62 | 109.98 | 29.20 | 37.61 | 46.05 |
| GEO(90deg Parabolic) | 125.38 | 133.59 | 142.87 | 27.86 | 37.15 | 45.33 |
| LEO-600(35deg Parabolic) | 97.48 | 108.57 | 147.91 | -8.67 | 30.63 | 41.74 |
| GEO(35deg Parabolic) | 125.17 | 135.82 | 174.60 | -3.88 | 34.92 | 45.57 |
| LEO-600(90deg 45*45PhasedArray) | 91.49 | 99.12 | 107.84 | 31.36 | 40.13 | 47.75 |
| GEO(90deg 45*45PhasedArray) | 122.75 | 131.21 | 140.45 | 30.27 | 39.53 | 47.99 |
| LEO-600(35deg 45*45PhasedArray) | 100.01 | 110.26 | 149.48 | -10.33 | 28.95 | 39.22 |
| GEO(35deg 45*45PhasedArray) | 127.04 | 137.74 | 177.08 | -6.46 | 32.99 | 43.65 |

Table 2: Calibration summary for NTN UL

| Calibration metrics | UL Coupling Loss |  |  | UL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CDF percentile | @5% | @50% | @95% | @5% | @50% | @95% |
| LEO-600(90deg Parabolic) | 95.13 | 104.81 | 118.30 | 14.82 | 14.82 | 14.82 |
| GEO(90deg Parabolic) | 125.96 | 137.09 | 157.11 | -16.16 | 3.85 | 14.82 |
| LEO-600(35deg Parabolic) | 98.50 | 109.58 | 147.94 | -7.07 | 14.82 | 14.82 |
| GEO(35deg Parabolic) | 125.93 | 137.71 | 176.49 | -35.56 | 3.20 | 14.82 |
| LEO-600(90deg 45*45PhasedArray) | 94.46 | 104.24 | 118.36 | 14.82 | 14.82 | 14.82 |
| GEO(90deg 45*45PhasedArray) | 126.33 | 137.86 | 158.88 | -17.99 | 3.04 | 14.57 |
| LEO-600(35deg 45*45PhasedArray) | 102.24 | 113.27 | 152.04 | -11.13 | 14.82 | 14.82 |
| GEO(35deg 45*45PhasedArray) | 130.12 | 141.13 | 180.22 | -39.35 | -0.24 | 10.78 |

Table 3: Calibration summary for TN DL

| Calibration metrics | DL Coupling Loss |  |  | DL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CDF percentile | @5% | @50% | @95% | @5% | @50% | @95% |
| Macro Urban | 50.63 | 75.61 | 95.54 | 12.41 | 32.77 | 55.26 |

Table 4: Calibration summary for TN UL

| Calibration metrics | UL Coupling Loss |  |  | UL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CDF percentile | @5% | @50% | @95% | @5% | @50% | @95% |
| Macro Urban | 50.72 | 75.82 | 95.44 | 10.20 | 14.75 | 14.81 |

Proposal 1: RAN4 can discuss the necessity of the calibration and consider the calibration work for the Ku-band NTN.

NTN Ku band Coexistence study based on ITU regulations

The parameters and assumptions of the simulation study attached in the attachment of this document. The throughput loss – ACIR figure and the ACIR requirements of each cases shown as below.

GEO 60 cm Parabolic cases

Figure 2.1-1: Throughput loss and ACIR figures

Observation 1: Basic on the simulation results, the ACIR requirement for case 1 the GEO 60 cm Parabolic should be 9.6 dB.

Figure 2.1-2: Throughput loss and ACIR figures

Observation 2: Basic on the simulation results, the ACIR requirement for case 2 the GEO 60 cm Parabolic should be 9.9 dB.

Figure 2.1-3: Throughput loss and ACIR figures

Observation 3: Basic on the simulation results, the ACIR requirement for case 3 the GEO 60 cm Parabolic should be 25 dB.

Figure 2.1-4: Throughput loss and ACIR figures

Observation 4: Basic on the simulation results, the ACIR requirement for case 4 the GEO 60 cm Parabolic should be 0 dB.

LEO-600 60 cm Parabolic cases

Figure 2.2-1: Throughput loss and ACIR figures

Observation 5: Basic on the simulation results, the ACIR requirement for case 1 the LEO-600 60 cm Parabolic should be 2.1 dB.

Figure 2.2-2: Throughput loss and ACIR figures

Observation 6: Basic on the simulation results, the ACIR requirement for case 2 the LEO-600 60 cm Parabolic should be 37.7 dB.

Figure 2.2-3: Throughput loss and ACIR figures

Observation 7: Basic on the simulation results, the ACIR requirement for case 3 the LEO-600 60 cm Parabolic should be 26.2 dB.

Figure 2.3-4: Throughput loss and ACIR figures

Observation 8: Basic on the simulation results, the ACIR requirement for case 4 the LEO-600 60 cm Parabolic should be 0 dB.

GEO 45 x 45 Phased Array case

Figure 2.3-1: Throughput loss and ACIR figures

Observation 9: Basic on the simulation results, the ACIR requirement for case 1 the GEO 45 x 45 Phased Array case should be 3.6 dB.

Figure 2.3-2: Throughput loss and ACIR figures

Observation 10: Basic on the simulation results, the ACIR requirement for case 2 the GEO 45 x 45 Phased Array case should be 10.3dB.

Figure 2.3-3: Throughput loss and ACIR figures

Observation 11: Basic on the simulation results, the ACIR requirement for case 3 the GEO 45 x 45 Phased Array case should be 18.3dB.

Figure 2.3-4: Throughput loss and ACIR figures

Observation 12: Basic on the simulation results, the ACIR requirement for case 4 the GEO 45 x 45 Phased Array case should be 0 dB.

LEO-600 45 x 45 Phased Array case

Figure 2.4-1: Throughput loss and ACIR figures

Observation 13: Basic on the simulation results, the ACIR requirement for case 1 the LEO-600 45 x 45 Phased Array case should be 0 dB.

Figure 2.4-2: Throughput loss and ACIR figures

Observation 14: Basic on the simulation results, the ACIR requirement for case 2 the LEO-600 45 x 45 Phased Array case should be 39.5dB.

Figure 2.4-3: Throughput loss and ACIR figures

Observation 15: Basic on the simulation results, the ACIR requirement for case 3 the LEO-600 45 x 45 Phased Array case should be 19.3 dB.

Figure 2.4-4: Throughput loss and ACIR figures

Observation 16: Basic on the simulation results, the ACIR requirement for case 4 the LEO-600 45 x 45 Phased Array case should be 0 dB.

Summary of the study

According to current simulation study results, we can have the following ACIR requirements in order to achieve the 5% throughput loss criteria.

|  | SAN GEO | SAN LEO-600 | VSAT - 45 x 45 Phased Array | VSAT - 60 cm Parabolic |
| --- | --- | --- | --- | --- |
| ACIR requirements to determine ACLR | 0 | 0 | 3.6 | 9.6 |
| ACIR requirements to determine ACS | 10.3 | 39.5 | 19.3 | 26.2 |

Proposal 2: RAN4 can consider the simulation results as a starting point of ACIR requirement discussion.

Conclusion

Proposal 1: RAN4 can discuss the necessity of the calibration and consider the calibration work for the Ku-band NTN.

Observation 1: Basic on the simulation results, the ACIR requirement for case 1 the GEO 60 cm Parabolic should be 9.6 dB.

Observation 2: Basic on the simulation results, the ACIR requirement for case 2 the GEO 60 cm Parabolic should be 9.9 dB.

Observation 3: Basic on the simulation results, the ACIR requirement for case 3 the GEO 60 cm Parabolic should be 25 dB.

Observation 4: Basic on the simulation results, the ACIR requirement for case 4 the GEO 60 cm Parabolic should be 0 dB.

Observation 5: Basic on the simulation results, the ACIR requirement for case 1 the LEO-600 60 cm Parabolic should be 2.1 dB.

Observation 6: Basic on the simulation results, the ACIR requirement for case 2 the LEO-600 60 cm Parabolic should be 37.7 dB.

Observation 7: Basic on the simulation results, the ACIR requirement for case 3 the LEO-600 60 cm Parabolic should be 26.2 dB.

Observation 8: Basic on the simulation results, the ACIR requirement for case 4 the LEO-600 60 cm Parabolic should be 0 dB.

Observation 9: Basic on the simulation results, the ACIR requirement for case 1 the GEO 45 x 45 Phased Array case should be 3.6 dB.

Observation 10: Basic on the simulation results, the ACIR requirement for case 2 the GEO 45 x 45 Phased Array case should be 10.3dB.

Observation 11: Basic on the simulation results, the ACIR requirement for case 3 the GEO 45 x 45 Phased Array case should be 18.3dB.

Observation 12: Basic on the simulation results, the ACIR requirement for case 4 the GEO 45 x 45 Phased Array case should be 0 dB.

Observation 13: Basic on the simulation results, the ACIR requirement for case 1 the LEO-600 45 x 45 Phased Array case should be 0 dB.

Observation 14: Basic on the simulation results, the ACIR requirement for case 2 the LEO-600 45 x 45 Phased Array case should be 39.5dB.

Observation 15: Basic on the simulation results, the ACIR requirement for case 3 the LEO-600 45 x 45 Phased Array case should be 19.3 dB.

Observation 16: Basic on the simulation results, the ACIR requirement for case 4 the LEO-600 45 x 45 Phased Array case should be 0 dB.

|  | SAN GEO | SAN LEO-600 | VSAT - 45 x 45 Phased Array | VSAT - 60 cm Parabolic |
| --- | --- | --- | --- | --- |
| ACIR requirement to determine ACLR | 0 | 0 | 3.6 | 9.6 |
| ACIR requirement to determine ACS | 10.3 | 39.5 | 19.3 | 26.2 |

Proposal 2: RAN4 can consider the simulation results as a starting point of ACIR requirement discussion.

Referecne

- RP-243314, “Revised WID: Introduction of Ku bands for NR NTN”, 3GPP RAN#105

- R4-2502315 Co-existence simulation assumptions for NR NTN Ku band (Running document)

- R4-2504723 Way Forward for [114bis][313] NR_NTN_Ku_Band_Coexistence

- TR 38.863

Attachment

6a.1	Co-existence simulation scenario for 11/14GHz

The frequency and bandwidth are listed in Table 6a.1-3.

Table 6a.1-3: Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Urban macro | 11/14 GHz | 100MHz | TDD | 1 |
| GEO | 11GHz (DL)/14GHz(UL) | 100MHz per beam | FDD | 2 |
| LEO | 11GHz (DL)/14GHz(UL) | 100MHz per beam | FDD | 2 |

6a.2	Co-existence simulation assumption for 11/14GHz

6a.2.1	Network layout model

6a.2.1.1	Co-existence between NTN satellite and TN

Cellular cell structure is considered for both NTN and TN network layout.

Referring to TR 38.811 Section 6.3 and Annex A, a 3D global coordinate system is considered (Earth-Centred Earth Fixed) for simulating NTN beams direction and location on the earth surface. It means the NTN beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z)

Deployment of NTN satellite and TN cells and UEs for co-existence study is listed in Table 6a.2.1.1-1.

Table 6a.2.1.1-1: Network and UE deployment

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1a | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters. | TN randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
| 2a | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider an active rate of 20% for TN | Only the active TN cells in central NTN beam |
| 3a | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | All active TN clusters which contain NTN UE(s) |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
| 4a | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| 5a | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Only the active TN clusters which contain the NTN UE(s) |
| 6a | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO, interested companies can bring analysis and results for other values) NTN UE: NTN UEs randomly dropped in TN clusters | TN clusters randomly placed in this NTN beam |  |
| 7a | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
| 8a | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider the active rate of 20% for TN | All active TN cells in central NTN beam |

6a.2.2	System parameters

6a.2.2.1	Satellite parameters

Set-1 satellite parameters are listed in Table 6a.2.2.1-2 according to TR 38.821 [6].

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 6a.2.2.1-1: NRB configuration per BandWidth size and SCS

| Configuration Ku-band | NRB (100MHz BW) |
| --- | --- |
| SCS 30 kHz | 273 |

Table 6a.2.2.1-2: Satellite parameters for co-existence study

| Satellite orbit |  |  | GEO | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 600 km |
| Satellite antenna pattern |  |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) |  | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  |  | 43 dBi | 38.5 dBi |
| 3dB beamwidth |  |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  |  | 250 km | 19 km |
|  | Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) |  | Ku-band (i.e. 14 GHz for UL) | 4 m | 0.714 m |
| G/T |  |  | 16.5 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  |  | 44 dBi | 38.5 dBi |
|  | NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |

Table 6a.2.2.1-3: Other parameters for NTN satellite

| Parameters | NTN satellite | Remark |
| --- | --- | --- |
| Carrier frequency | 14GHz(UL) / 11GHz(DL) |  |
| The number of active UE (UL) | 10 UEs and 27RBs per UE for GEO and LEO |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 6a.2.6.2 |  |
| NTN satellite Noise Figure in dB | See Table 6a.2.2.1-4 |  |
| Handover margin | 3dB |  |

Table 6a.2.2.1-4: NTN satellite Noise Figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 30.9 | 10.9 | 10.9 |
| NF (dB) | 3 | 3 | 3 |

6a.2.2.2	NTN UE parameters

The NTN UE antenna characteristics are given in Table 6a.2.2.2-1 and other NTN UE parameters are given in Table 6a.2.2.2-2.

Table 6a.2.2.2-1: Phase Array and Parabolic Antenna Parameters

| 1 | VSAT Antenna Characteristics |  |  |  |
| --- | --- | --- | --- | --- |
| 1.1 | Antenna pattern | TR 38.921, TR 38.922 | TR 38.921, TR 38.922 | Circular Aperture TR 38.863 |
| 1.2 | Element gain (dBi) (Note 1) | 3.5 | 3.5 | N/A |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis | 90° for Y axis 90° for Z axis | N/A |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB | 30 dB | N/A |
| 1.5 | Antenna polarization (Note 5) | Circular (RHCP or LHCP) | Circular (RHCP or LHCP) | Circular (RHCP or LHCP) |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 3) | 45x45 elements | 78x78 elements | 60 cm diameter aperture |
| 1.7 | Number of supported polarizations, P | 1 | 1 | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda | 0.5 lambda | N/A |
| 1.9 | Array Ohmic loss (dB) (Note 1) | 2 | 2 | N/A |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) | 6.9 | 9.1 | N/A |
| 1.11 | Tx Transmit power | 10 Watts for GEO and for LEO | 50 Watt for GEO | 10 Watts for GEO and for LEO |
| 1.12 | Maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees | 0~360 degrees | 0~360 degrees |
| 1.13 | Vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees | 0~60 degrees | 0~60 degrees |
| 1.14 | Normal direction | Toward X+ axis | Toward X+ axis | Toward X+ axis |
| 1.15 | Max EIRP Value | 76.6 dBm | 88.2 dBm | 76.6 dBm |
| 1.16 | Noise Figure | 6 dB | 6.0 dB for GEO | 2.5 dB for both LEO and GEO |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: Antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization evaluations are conducted by converting to linear polarization |  |  |  |  |

Table 6a.2.2.2-2: Other NTN UE parameters

| Characteristics | VSAT (Parabolic) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | NTN VSAT Type 4 and 5: 2.5 dB (LEO and GEO) and 6 dB (LEO); Other options not precluded |
| Tx transmit power | 50 W for GSO and 10 W (40 dBm) for NGSO |
| Tx antenna gain | Option 1 - 36.6 dBi, Option 2 – 36.2 dBi |
| Antenna elevation angle | 35˚ and 90˚ |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. |  |

The altitudes of NTN UE for different use case are defined in Table 6a.2.2.2-2.

Table 6a.2.2.2-3: NTN UE altitude

| Use case | M-ESIM | A-ESIM | L-ESIM |
| --- | --- | --- | --- |
| Altitude | 22.5m | 3-14km | 1.5m |
| Note 1: Co-existence evaluation is only limited to L-ESIM. |  |  |  |

6a.2.2.3	TN parameters

TN parameters for co-existence study are given in Table 6a.2.2.3-1 and Table 6a.2.2.3-2.

Table 6a.2.2.3-1: Simulation assumptions of TN NR

| Carrier frequency in GHz | 11 and 14 |
| --- | --- |
| Size of each nominal channel BW in MHz | 100 |
| Transmission bandwidth in MHz | 98.28 |
| Environment | Urban macro |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance in meter | 450m |
| UE number per cell | Same as the number of BS beam |
| System loading and activity | 20% |
| Network location | See Table 6a.2.1.1-2 |
| DL subcarrier spacing | 30kHz |
| UL | OFDMA |
| DL power control | No |
| UL power control | See Section 6a.2.6.2 |
| Frequency reuse | 1 |
| Number of scheduled UE per cell (DL) | Same as the number of BS beam |
| Number of scheduled UE per cell (UL) | Same as the number of BS beam |
| BS height | 25 |
| UE antenna height in meter | 1.5 |
| UE TX power in dBm | -40 to 23 |
| Building penetration loss | In path loss model, TR 38.901 |
| Cell selection margin in dB | 3 |
| BS-MS min distance in meters | 35 |
| BS noise figure in dB | 10 |
| UE noise figure in dB | 10 |
| BS-UE path-loss model | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |
| Link-level performance model | See Section 6a.2.9 |
| UE distribution | Uniform |
| Evaluation metrics | See Section 6a.2.8 |

Table 6a.2.2.3-2: ACLR/ACS for TN (11 GHz)

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 37 dB |
|  | ACS | 40 dB |
| UE | ACLR | 24 dB |
|  | ACS | 31 dB |

Table 6a.2.2.3-3: ACLR/ACS for TN (14 GHz)

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 30 dB |
|  | ACS | 30 dB |
| UE | ACLR | 24 dB |
|  | ACS | 24 dB |

6a.2.3	Antenna and beam forming pattern modelling

6a.2.3.1	Satellite parameters

In Rel-18, antenna pattern in section 6.4.1 of TR38.811 is reused for NTN SAN and NTN UE. Use of phased array antenna model needs further discussion.

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered.

1                for θ=0

4J1kasinθπ180kasinθπ1802        for 0<θ≤90°

For UE, 4J1kasinπ2kasinπ22          for θ>90°

where:

-	J1(x) is the Bessel function of the first kind and first order with argument ‘x’;

-	ais the radius of the antenna's circular aperture;

-	k = 2f/c is the wave number;

-	f is the frequency of operation;

-	c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency. And the sin() function is in radian.

The beam layout definition for a single satellite simulation in Ka-Band is defined in Table 6a.2.3.1-1.

Table 6a.2.3.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Same with Table 6.2.3.1-1 |
| --- | --- |
| Number of beams | Same with Table 6.2.3.1-1 |
| UV plane illustration (extracted from [19]) | Same with Table 6.2.3.1-1 |
| UV plane convention | Same with Table 6.2.3.1-1 |
| Adjacent beam spacing on UV plane | Same with Table 6.2.3.1-1 |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: 25° for GEO and LEO |
| Frequency-reuse factor | FRF=2 Do not consider interference leakage from adjacent beams as starting point for co-ex study. For example for the figure above, do not consider the interference leakage from green, blue and purple beams when study SINR for the red beam. |
| Polarization re-use | Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |
| UEs outdoor/indoor distribution | Same with Table 6.2.3.1-1 |
| UE distribution | Same with Table 6.2.3.1-1 |
| UE configuration | Mobile VSAT for GSO and NGSO |
| UE orientation | Random Ideal Tracking serving beam |
| UE attachment | RSRP |

6a.2.3.2	TN BS antenna and beam forming pattern modelling

The parameters in Table 6a.2.3.2-1 are used for BS antenna pattern in the coexistence study.

Table 6a.2.3.2-1: TN BS antenna modelling for Urban macro scenario for 11 GHz

| Parameters | Values for 11 GHz |
| --- | --- |
| Scenario | Urban macro |
| Antenna pattern | AAS model see below (from TR 38.922) |
| Element gain GE,max (dBi) | 6.4 |
| Horizontal 3dB /vertical 3dB beam width of single element | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) (Note 4) | 8x16 |
| Horizontal/Vertical radiating sub-array or element spacing (Note 5) | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength for V |
| Pre-set sub-array down-tilt (degrees) (Note 6) | 3 |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 |
| Mechanical downtilt (degrees) | 6 |

Table 8.1.1-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan | dBi |

| Parameters | Values for 14GHz |
| --- | --- |
| Scenario | Urban macro |
| Antenna pattern | Extended AAS model see below (from TR 38.922) |
| Element gain GE,max (dBi) | 6.4 |
| Horizontal 3dB /vertical 3dB beam width of single element | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V |
| Side lobe suppression SLAv (dB) | 30 |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 24 elements |
| Horizontal/Vertical radiating element spacing | H = 0.5 λ V = 2.8λ |
| Number of element rows in sub-array | 4 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 17.14 |
| Base station horizontal coverage range (degrees) | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 |
| Mechanical down-tilt (degrees) | 6 |

Table 6.1.2.3.2-2: Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ |

6a.2.3.3	TN UE antenna model

The UE is expected to have a conducted interface assuming an isotropic radiation pattern antenna and without beamforming.

6a.2.4	ACIR model

ACLR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942[8] is used as baseline. The number of RBs refers to Table 6a.2.2.1-3 and Table 6a.2.2.4-1 respectively.

6a.2.5	Propagation model

Propagation models between NTN and TN components used for the coexistence study are listed in Table 6a.2.5-1.

Table 6a.2.5-1: Propogation models between NTN and TN

| Link | Propagation model |
| --- | --- |
| TN BS to Fixed VSAT on roof | Free space path loss |
| TN BS to L-ESIM at 1.5 m | UMa as in 3GPP TR 38.803 |
| TN BS to TN UE | UMa as in 3GPP TR 38.803 |
| TN UE to Fixed VSAT on roof | UMa as in 3GPP TR 38.803 (BS is to be replaced with VSAT) |
| TN UE to L-ESIM | Umi |
| Satellite to TN BS/UE | 3GPP TR 38.821 |
| Satellite to VSAT/ESIM | 3GPP TR 38.821 |
| TN BS to Satellite | 3GPP TR 38.821 |
| Note1: For the propagation models which use the 3GPP TR 38.821 [3], to use same assumptions as in [3] and consider the atmospheric losses and the scintillation losses. Note2: TN BS height is 25m |  |

6a.2.6	Transmission power control model

6a.2.6.1	UL TPC

Following UL power control model for both NTN and TN are used as starting point.

Where,

-	Pmax = maximum UE Tx power, i.e. 23 dBm for TN UE;

-	Rmin = minimum power reduction ratio, i.e. -63 dB for TN UE, and [-63 dB by assuming NTN UE min Tx power as -30dBm as starting point] for NTN UE;

-	CLx-ile and γ are set as following:

-	γ = 1;

-	CLx-ile = –SNR_target + Pmax – Thermal Noise – BS/SAN_NoiseFigure - 10*log10(BW), considering SNR_target is 15dB and BW is actual UL BW.

6a.2.6.2	DL TPC

For downlink scenario, no power control scheme is applied.

6a.2.7	Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

-	RX_PWR is the received power

-	TX_PWR is the transmitted power

-	G_TX is the transmitter antenna gain (directional array gain)

-	G_RX is the receiver antenna gain (directional array gain).

6a.2.8	Performance metric

For NR, the average throughput loss and 5%-ile throughput loss should be less than 5%.

For NTN, the average throughput loss and 5%-ile throughput loss should be less than 5%.

6a.2.9	Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics

6a.3	Co-existence simulation methodology for 11/14GHz

6a.3.1	Simulation procedure

See Section 6.3.1 of TR 38.863, noting the FRF and network deployment for NTN and TN in above 10GHz are specified accordingly.

Following approaches for the deployment of NTN UE and satellite are used:

-	The satellite should be generated in the visible area/sky of NTN UE;

-	NTN UEs point to the satellite accurately;

-	The position of the satellite should guarantee that NTN UE vertical angle towards the satellite;

-	For calibration, use 30 degree and 90 degree instead of the range;

-	SAN elevation angle value, 35, & 90 degrees (computed from the centre of the beam), for co-existence -simulation.

-	Horizontal angle of NTN UEs should be calculated based on the satellite position;

-	Use 35m as minimum distance assumed between VSAT and TN BS for co-ex study.