3GPP TSG-RAN WG4 Meeting # 115	 R4-2505763

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on general aspects of AI/ML testability and interoperability

Agenda Item:	7.21.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss about general aspects of AI testability and interoperability.

# 2 Discussion

In last meeting, there are WF about deactivation and activation:

Delay requirements for activation/switching/fallback Agreement: - RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2: - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference - functionality deactivation refers to the process of network deactivating an active functionality Agreement: - FFS on whether the deactivation requirements will be introduced - The starting point is when the UE receives the signalling to commence deactivation - FFS when the endpoint for deactivation is - FFS whether fallback delay is the same as deactivation delay or has some other components Agreement: - For beam management use case - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistant reporting based on AI/ML - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML - FFS on the definition of first reporting

AI beam reporting functionality activation procedure is agreed in RAN1#119:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

RAN2 # 129 agree related signalling:

Agreements - Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). - The full inference configuration is sent in CSI-ReportConfig. - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

- Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities FFS option B Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

The existing CSI-ReportConfig framework will be reused for AI/ML-based beam management (BM) inference reporting.

Observation 1: From RAN1 agreements, legacy CSI-ReportConfig framework will be used for BM inference reporting.

Observation 2: Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

- Semi-persistent reporting is activated by MAC CE/DCI.

- Aperiodic CSI reporting is activated by DCI.

Observation 3: For periodic CSI reporting, if option A is configured in Step 3, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4.

Deactivation Delay Requirements

The current legacy CSI framework provides well-established deactivation mechanisms:

- For periodic CSI reporting, deactivation occurs through RRC reconfiguration

- For semi-persistent CSI reporting, deactivation is achieved via MAC CE or DCI signaling

- Aperiodic CSI reporting automatically terminates after single-shot transmission

In legacy, no de-activation delay is defined for CSI reporting in RAN4. We suggest no additional deactivation delay requirements is defined for AI/ML-based CSI reporting. Deactivating AI/ML functions doesn't have any special or different requirements. It’s similar to how we deactivated functions in the past, so we don’t need to change when and how we turn them off.

Observation 4: In legacy, no de-activation delay is defined for CSI reporting in RAN4.

Proposal 1: For AI BM, legacy CSI-ReportConfig framework will be used, don’t need to define deactivation delay.

Fall back

Fallback involves reverting to a previous state or configuration to ensure system stability and functionality, which has distinct purposes and operational mechanisms compared to deactivation that simply stops an active function. Given the unique nature of fallback, it is essential to define its requirements separately to avoid confusion and ensure clear implementation guidelines, rather than lumping it together with deactivation requirements.

Proposal 2: Define fallback requirements independently from deactivation.

Activation Delay Requirements for First Reporting

The timing of first report transmission involves several considerations:

- For periodic and semi-persistent reporting:

- Current framework uses n0 + reportSlotOffset calculation

- AI/ML processing may introduce additional latency for:

  - Model initialization

  - Measurement set switching

  - Inference computation

- For aperiodic reporting:

- Legacy timing based on DCI processing delay

- AI/ML may require additional processing time

- The interaction between traditional and AI-based measurements needs clarification

We suggest waiting for more progress from RAN1 regarding both the definition of the activation delay and the first report in the context of AI - based BM. RAN1's further development on the overall AI - based BM reporting structure will clarify whether and how the reportSlotOffset will be adjusted, which impacts the activation delay, and also provide more clarity on what constitutes the first report considering new AI - related operations and requirements.

Proposal 3: For definition of first report and activation delay, wait for more RAN1 progress.

Issue 1-4-4: Relation to legacy RAN4#112bis WF: Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management - Proposals - Option 1: Keep same side conditions for legacy and AI/ML (Apple proposal 18, CATT proposal 1 option 1, CMCC proposal 1) - Option 2: Measurement error may have an impact on whether AI can exceed “legacy” requirements (Xiaomi proposal 1) - Option 3: Allow larger margin (presumably more relaxed requirement) for higher overhead reduction (CATT proposal 1 option 2) - For CSI compression, better than legacy with the same overhead, and no worse than legacy with reduced overhead (Huawei proposal 1) - For positioning, re-use legacy requirement for case 2a/3a if AI and non-AI cannot be distinguished (Huawei proposal 3) - for CSI prediction, it is proposed that AI/ML based performance requirement should not be worse than the legacy PMI requirements for 'typeII-doppler-r18 (CMCC proposal 2) - Option 4: Legacy performance is the baseline; allow margin for scenarios in which legacy is not defined compared to the baseline (ZTE proposals 3, 4). - Option 5: When a test is defined for AI for which there is no legacy test, need to decide whether to (i) Investigate non-AI performance in that scenario and set AI requirement greater or (ii) set an absolute requirement on AI (without considering non-AI) (Huawei proposal 5) - Recommended WF Discuss the following: - Same side conditions as legacy - What does this mean in practice ? When no existing requirement exists, or some other types of condition should be tested - Does measurement error impact comparison to non-AI - If no non-AI requirement exists for some functionality/condition, is it necessary to consider non-AI performance for comparison to AI, or sufficient just to set an absolute requirement for AI ? - When there is a non-AI requirement, if the AI functionality operates with lower overhead, should the same requirement be met or a more relaxed requirement ?

For AI based beam management, currently, there are several candidate test metrics:

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

From our understanding, whether the AI requirements exceed “legacy” requirements depends on whether the same metric and side condition is defined for AI and non-AI legacy requirement.

For example, if test metric is beam ID prediction accuracy, in legacy requirement, there is no such metric. We prefer just to set a specific requirement for AI.

Proposal 4: For beam ID prediction accuracy, there is no performance requirement in legacy. Set a specific requirement for AI.

If test metric is absolute RSRP prediction accuracy for the same beam. In legacy non-AI, there is also similar metric. For this metric, predicted RSRP or legacy measured RSRP will be compared with ground truth. RSRP delta is the accuracy requirement.

However, currently, the impact of measurement error is studied. The AI input is measured L1-RSRP with error, it will degrade the RSRP prediction performance. It depends on how the AI model is trained. there are different implementations:

- AI input is measured RSRP without error, label is ground RSRP without error

- AI input is measured RSRP with error, label is measured RSRP with error

- AI input is measured RSRP with error, label is measured RSRP without error

Whether the accuracy of prediction RSRP can still outperform the accuracy of legacy measured RSRP is an issue which needs to be studied.

Proposal 5: For RSRP prediction accuracy, measurement error may have impact on whether AI requirements can exceed “legacy” requirements.

# 3 Conclusion

In this contribution, we provide the following proposals:

Observation 1: From RAN1 agreements, legacy CSI-ReportConfig framework will be used for BM inference reporting.

Observation 2: Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

- Semi-persistent reporting is activated by MAC CE/DCI.

- Aperiodic CSI reporting is activated by DCI.

Observation 3: For periodic CSI reporting, if option A is configured in Step 3, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4.

Observation 4: In legacy, no de-activation delay is defined for CSI reporting in RAN4.

Proposal 1: For AI BM, legacy CSI-ReportConfig framework will be used, don’t need to define deactivation delay.

Proposal 2: Define fallback requirements independently from deactivation.

Proposal 3: For definition of first report and activation delay, wait for more RAN1 progress.

Proposal 4: For beam ID prediction accuracy, there is no performance requirement in legacy. Set a specific requirement for AI.

Proposal 5: For RSRP prediction accuracy, measurement error may have impact on whether AI requirements can exceed “legacy” requirements.

# 4 Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interfa