3GPP TSG-RAN WG4 Meeting # 115	R4-2505767

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on impacts on RAN4 requirement for measurement event prediction in AI mobility

Agenda Item:	7.23.3.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss RRM impact on AI mobility for measurement event prediction.

# 2 Discussion

2.1 Core requirement

As agreed in TR, there are two methods to predict measurement event, namely indirect and direct measurement event prediction.

In legacy, RAN4 defined event triggered L3 reporting delay in core part and define corresponding test case to verify whether UE will correctly detect the event by L3 reporting.

Observation 1: In legacy, RAN4 defined event triggered L3 reporting delay in core part.

Similarly, for measurement event prediction, it may have impact on predicated event triggered L3 reporting delay. RAN4 also needs to verify UE behaviour whether event can be correctly predicted. It’s FFS how to define requirement for predicated event triggered L3 reporting, RAN4 needs to wait for more RAN2 progress.

Proposal 1: Measurement event prediction may have impact on predicated event triggered L3 reporting delay.

2.2 Performance requirement

- 2.2.1 RSRP predication accuracy requirement

For legacy, non-AI-based event-triggered reporting, each report contains only one RSRP value for each cell. The accuracy of RSRP predictions is evaluated independently for each report. If the difference between the predicted RSRP and the ground truth RSRP is within a specified threshold, the prediction is considered valid. To ensure a high accuracy rate, at least 90% of the reports must be valid.

For case A, UE will predict several RSRPs in future and will report the predicted RSRPs in advance. For each report, it can include single or multiple RSRPs.

Figure 5.2.1-1: Example of intra-frequency temporal domain case A

If each report may include multiple predicted RSRP values for different future time instances, it introduces a new challenge in defining the RSRP accuracy metric, as the accuracy of predictions for multiple RSRPs in a single report may vary.

From RAN2 simulation, the time interval from measurement to prediction increase, the prediction accuracy decreases. For example, the predicted RSRP accuracy of the first point in PW is quite different from that of the last point in PW due to the fact that T2 is larger than T1, as shown below:

Fig.1 different L3-RSRP predication accuracy in TTT

Observation 2: The time interval from measurement to prediction increase, the prediction accuracy decreases. The prediction accuracy of individual measurement results in a time window could be quite different.

Additionally, event reporting mechanisms are based on the differences between a series of predicted RSRP values and a threshold over a continuous period of time. To trigger an event report, all predicted RSRP values for multiple continuous time instances must meet the threshold requirement.

Given these considerations, we propose two possible metrics for evaluating RSRP accuracy in AI-based event-triggered reporting:

Metric 1: Independent Evaluation of Each RSRP Prediction

- Each of the N*M predicted RSRP values is independently evaluated against the true values.

- If the difference between a predicted RSRP value and its true value is less than the threshold X, it is marked as correct (1).

- If at least 90% of the N*M predictions are correct, the test is deemed successful.

Metric 2: Continuous Evaluation Scheme for a Time Duration in the Report

- In one report, there are M RSRPs. If the difference between all M predicted RSRP values and their corresponding true values is within a threshold of X dB, the prediction for that report is considered valid.

- If at least 90% of the N reports are valid, the test is deemed successful.

We suggest to adopt metric 2. Event reporting mechanisms are based on the differences between a series of predicted RSRP values and a threshold over a continuous period of time. To trigger an event report, it is essential that all predicted RSRP values for multiple continuous time instance shall meet the threshold requirement.

For metric 1, it’s similar as legacy. However, independent Point Evaluation fails to ensure continuity. If the prediction at one time instance exceeds the threshold, it can lead to false triggering of event reports, which can negatively impact mobility performance.

For metric 2: Continuous Evaluation over multiple time instances ensures that the predicted RSRP values meet the threshold requirements over the entire period, thus maintaining the continuity and reliability of the event reporting mechanism.

Therefore, it is necessary to evaluate the accuracy of multiple time instances within a single report, rather than independently assessing each individual point.

Proposal 2: For RSRP accuracy requirement for event triggered reporting, if there are multiple predicted RSRPs in one report, we propose to evaluate RSRP accuracies continuously over multiple time instances to enhance the performance of event reporting.

Relative accuracy

We suggest to discuss RSRP relative accuracy in measurement event prediction since relative RSRP is used to trigger event 2.

Similarly, when there are multiple RSRPs for different time instances in one report, we suggest to evaluate relative RSRP accuracies continuously over multiple time instances to enhance the performance of event reporting.

Proposal 3: For event triggered reporting, RAN4 to define relative accuracy requirement. If there are multiple predicted RSRPs in one report, evaluate relative RSRP accuracies continuously over multiple time instances.

2.2.2 Event accuracy

For AI based indirect event prediction, performance metric F1 score is defined in RAN2

F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3) For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: - Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa - Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] - Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD] The ETD i.e. timing difference between ground-truth event and predicted event is illustrated in Figure 5.3.1-1: Figure 5.3.1-1: illustration of ETD As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected.

From RAN2, performance metric F1 score is defined, which considered both precision and recall.

Besides, ETD is also defined in RAN2, which is a timing difference between predicated event and ground truth event.

- max ETD also be interpreted as the event prediction time margin in the test.

- max ETD can be interpreted as timing accuracy requirement for event prediction.

Event prediction time accuracy = predicated event time – ground truth event time

If timing difference is smaller than max ETD, UE has correctly predicted the event.

RAN4 needs to discuss how to consider the impact of ETD.

Observation 3: In RAN2, performance metric F1 score is defined for event prediction accuracy, which considered both precision and recall.

Observation 4: In RAN2, a timing difference between predicated event and ground truth event is defined.

While in legacy RAN4, in test for verify whether event is correctly detected, only precision = 90% is used. Precision considers the correct reporting ratio for all triggered events, which is shown below:

##### A.4.6.1.1.3 Test Requirements The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

Observation 5: In legacy RAN4, in test to verify event prediction accuracy, only precision = 90% is used and no timing difference between predicated event and ground truth event is considered.

Proposal 4: RAN4 to only consider precision and not consider recall in event prediction accuracy KPI.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: In legacy, RAN4 defined event triggered L3 reporting delay in core part.

Proposal 1: Measurement event prediction may have impact on predicated event triggered L3 reporting.

Observation 2: The time interval from measurement to prediction increase, the prediction accuracy decreases. The prediction accuracy of individual measurement results in a time window could be quite different.

Proposal 2: For RSRP accuracy requirement for event triggered reporting, if there are multiple predicted RSRPs in one report, we propose to evaluate RSRP accuracies continuously over multiple time instances to enhance the performance of event reporting.

Proposal 3: For event triggered reporting, RAN4 to define relative accuracy requirement. If there are multiple predicted RSRPs in one report, evaluate relative RSRP accuracies continuously over multiple time instances.

Observation 3: In RAN2, performance metric F1 score is defined for event prediction accuracy, which considered both precision and recall.

Observation 4: In RAN2, a timing difference between predicated event and ground truth event is defined.

Observation 5: In legacy RAN4, in test for verify event prediction accuracy, only precision = 90% is used and no timing difference between predicated event and ground truth event is considered.

Proposal 4: RAN4 to only consider precision and not consider recall in event prediction accuracy KPI.