3GPP TSG-RAN WG4 Meeting #115				                                R4-2505782

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.26.4

Title: 	Discussion RRM requirements for Ambient IoT

Source: 	Xiaomi

Document for:		Discussion

- Introduction

The work item of Ambient IoT was approved in [1, RP-243326]. And in the las meeting, the corresponding RRM discussion was started and some agreements achieved [2, R4-2504905]. In this paper we provide further discussion on the open issues below.

- IoT random access procedure

- timing

- Discussion

- Timing

According to TR agreements [3, TR38.769], the following parameters for the timing relationship between the device and reader will be defined for AIoT.

Observation 1: the key parameters for timing relationship parameters are under RAN1’s discussion, such as [TR2D_max, TR2D_TR2D_min, TD2R_D2R_min]

And on the other hand, from RAN4 itself, there are timing requirements for UE transmit timing error (7.1), UE timer accuracy (7.2) , TA adjustment accuracy (7.3), MRTD/MTTD (7.5, 7.6). In our view, for A-IoT device, TA and CA functions will not be supported. That is,

Observation 2: RAN4 needs consider potential timing requirements for AIoT device of

- UE initial transmit timing error (7.1),

- UE timer accuracy (7.2)

Proposal 1: RAN4 to discuss whether there is RRM impact on D2R transmit timing requirements based on RAN1 final conclusion on timing relationship design.

- Random access procedure

Issue 1-3: A-IoT random access procedure Agreement: - RAN4 to include RRM requirements for correct A-IoT device behaviour during CBRA and CFRA. FFS the details

In the last meeting, RAN4 agreed to FFS on the RRM requirements for AIoT random access procedure. According to the legacy random access requirements defined in TS38.133 6.2.2.2, the correct UE behaviour during random access shall be specified.

However, the detailed AIoT random access procedure is still under RAN2’s discussion. For an instance, in the last RAN2#129bis meeting, the agreements on the random access response message were included below, which would directly impact the 6.2.2.2.1.3 in TS38.133 at least.

Agreements on NACK

- For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

- For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access.    No need for a timer.   FFS whether subsequent R2D message is trigger message or paging

- For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve

- FFS on end of procedure

Observation 3: the detailed AIoT random access procedure is still open in RAN2.

Thus, we believe that RAN4 spec needs still wait further RAN2 progress.

Proposal 2: How to include procedural requirements for AIoT random access needs wait RAN2 further progress.

- Summary

This paper provided our consideration on RRM impacts for AIoT.

Observation 1: the key parameters for timing relationship parameters are under RAN1’s discussion, such as [TR2D_max, TR2D_TR2D_min, TD2R_D2R_min]

Observation 2: RAN4 needs consider potential timing requirements for AIoT device of

- UE initial transmit timing error (7.1),

- UE timer accuracy (7.2)

Proposal 1: RAN4 to discuss whether there is RRM impact on D2R transmit timing requirements based on RAN1 final conclusion on timing relationship design.

Observation 3: the detailed AIoT random access procedure is still open in RAN2.

Proposal 2: How to include procedural requirements for AIoT random access needs wait RAN2 further progress.

- References

- RP-243326

- R4-2504905

- 3GPP TR38.769

- 3GPP TS 38.133 v18.7.0