# 3GPP TSG-RAN WG4 Meeting #115	R4-250xxxx

# Malta, MT, 19-23 May, 2025

Agenda Item:	7.5.3

Title: 	TP to TR 38.755 on RF requirements evaluation

Document for:	APPROVAL

- Introduction

In last RAN4 meeting, TR 38.755 FR1 Fragmented carriers version 0.2.0 was approved in [1]. This contribution provides draft text proposal on section 7.x based our companion contribution [2].

- Text proposal

---------------------------Start of TP------------------------------------------

7.X	RF requirements evaluation from company C (Xiaomi)

The major impact on REFSENS requirements was Tx leakage due to reduced attenuation on UL carrier frequency with wideband LPF under “single Rx chain mode”.  The performance impact is diverse pending on the configuration (DL and UL frequency separation, operating frequency) and UE implementation (ADC performance, and NF increase due to AGC adjustment, LPF performance).

Figure 7.X-1: Per-carrier LBF vs single wideband LBF for CA_n3(2A)

Initial evaluation results based on experienced performance from products were provided in below table for band n3 with configuration as CA_n3(2A), 5MHz-65MHz-5MHz (PCC-Wgap-SCC).

Table 7.X-1: Link-budget for Self-interference of CA_n3(2A)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -97 | -92.3 | / | / |
| FE loss | / | 4 | 4 | / | / |
| Duplexer output | -23 | -101 | -96.3 | / | / |
| LPF attenuation | 12 | 1 | 1 | / | / |
| LPF output (dBm) | -36.4 | -102 | -97.3 | / | / |
| Noise floor for basic REFSENS | / | / | / | -101 | / |
| NF increase due to AGC adjustment | / | / | / | 10 | / |
| Noise floor | / | / | / | -91 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -96.3 | / | / |
| ADC output (dBm) | / | / | / | / | -104.4 |
| SNR after ADC (dB) | / | / | / | / | -14.0 (PCC) -14.6 (SCC) |

REFSENS performance degradation (ΔRIBNC ) is around 13 dB for PCC and 13.6dB for SCC for CA_n3(2A) with 5MHz-65MHz-5MHz (PCC-Wgap-SCC).

For in-gap interference, we evaluated whether relaxation on ACS/IBB is needed or not:

- For ACS case 1, both wanted carrier power and interference power are configured relative to REFSENS with fixed offset value(s). We assume the input power for both wanted signal and interference signal can be adjusted based on REFSENS relaxation with ΔRIBNC.

- For ACS case 2, both wanted carrier power and interference power are fixed as -56.5dBm/-25dBm. The received power of wanted carrier is much higher than Tx leakage power and the impact due to Tx leakage is negligible considering duplex can provide ~50dBc attenuation.

- For IBB1/IBB2, wanted signal power is configured relative to REFSENS with fixed offset value (6dBc). We assume the input power for wanted signal can be adjusted based on REFSENS relaxation with ΔRIBNC.  The interference signal power is fixed value as -56dBm (IBB case 1)/-44dBm (IBB case 2).

Figure 7.X-2: ACS/IBB requirements

Two scenarios were evaluated with results summarized in table 7.X-2 and 7.X-3:

- Scenario 1: In-gap blocker w/o image overlapped with wanted carrier(s)

- Scenario 2: Image of in-gap interference overlapped with wanted carriers(s)

Scenario 1: In-gap blocker w/o image overlapping with wanted carrier(s)

Table 7.X-2: Link-budget for in-gap blocker w/o image overlapping of CA_n3(2A)

| Duplexer isolation (dB) | 50 |  |
| --- | --- | --- |
| LPF attenuation (dB) | 12 |  |
| NF increase due to AGC adjustment (dB) | 10 |  |
| ΔRIBNC adjustment due to self-interference | 13dB for PCC | 13.6 dB for SCC |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | -1.2 | -1 |
| ACS case 2 | -1 | -1 |
| IBB Case 1 | 4.2 | 5 |
| IBB Case 2 | 4.2 | 5 |

There is no impact to ACS case 2 requirements due to Tx leakage. If same relaxation on REFSENS can be applied for ACS1/IBB1/IBB2 test cases, then the Tx leakage impact to ACS/IBB requirements is marginal.

Scenario 2: Image of in-gap interference overlap with wanted CC

With in-gap interference overlapping with wanted CC, we assume UE receiver has 25dBc image rejection ratio capability.

Table 7.X-3: Link-budget for in-gap blocked with image overlapping of CA_n3(2A)

| Duplexer isolation (dB) | 50 |  |
| --- | --- | --- |
| LPF attenuation (dB) | 12 |  |
| Image rejection ratio (dB) | 25 |  |
| NF increase due to AGC adjustment (dB) | 10 |  |
| ΔRIBNC adjustment due to self-interference | 13dB for PCC | 13.6 dB for SCC |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | -6.5 | -6 |
| ACS case 2 | -6.5 | -6.5 |
| IBB Case 1 | 0.6 | 1.2 |
| IBB Case 2 | -9.2 | -8.6 |

For the case in gap interference overlapped with wanted carriers, performance degradation will be observed once power ration between In-gap interference and wanted carriers is over than UE image rejection capability.

To avoid the impact to ACS/IBB requirements with image overlapped with wanted carrier(s), restriction with power difference between wanted carrier and interference in gap less than 20dB can solve the issue.

--------------------------End of TP----------------------------------

Reference

[1] R4-2505158,	draft update TR 38.755 FR1 Fragmented carriers_rm, MTK

[2] R4-250xxx, 	Impacts on UE RF requirements and DL performance, Xiaomi