# 3GPP TSG-RAN WG4 Meeting #115	R4-250xxxx

# Malta, MT, 19-23 May, 2025

Agenda Item:	7.5.4

Source: 	Xiaomi

Title: 						Impacts on UE RF requirements and DL performance

Document for:	Discussion

- Introduction

Rel-19 agreeded a study item to study the feasibility and requirements impact on using shared Rx chain to support mutilple fragemented carriers in single band under intra-band NC CA.

In previous RAN4 meeting, the reference archirecture and target scope has been clarified and agreed as following:

#### Sub-topic 1-2: Study scope clarification Agreement: - Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz as illustrated below: #### Merge issue 2-0/2-1 discussion: Agreement: The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823)

In previous RAN4 meeting, following agreements were reached for evaulation of RF requirements impact:

RAN4#113 #### Issue 3-3: Discussion of impacts on DL performance for UE enabling fragmented carrier mode Companies are encouraged to bring analysis with their own assumptions following the table format #### Issue 3-4: Discussion on in-gap requirements RAN4 evaluate performance impact on the cases: - image of in-gap interference overlaps with wanted carrier - For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC - image of in-gap blocking interference does not overlap with wanted carrier RAN4#114 #### Issue 3-1: Discussion on maximum input level for fragmented carriers - Maximum power level including all RF signals and interference at the UE input does not exceed -25 dBm #### Issue 3-2: Discussion of impacts on DL performance for UE enabling one Rx RF chain mode - Relaxation on REFSENS for both PCC and SCC will be evaluated and captured in the study outcome report. - DL performance degradation can be considered based on configurations - Configurations leading to no relaxation are not precluded pending on the evaluation results. The exact values and applicable conditions shall be decided in the WI phase if there is corresponding WI - Maintain the table format agreed in previous RAN4 meeting. Detailed evaluation assumptions such as imbalanced PRx/DRx noise level and other assumptions are encouraged to be provided from companies’ evaluation TPs #### Issue 3-3: Discussion on in-gap interference requirements - For image of in-gap interference not overlap with wanted CC: Evaluate whether relaxation on ACS/IBB is needed based on evaluated ΔRIBNC on PCC and SCC. - For image of in-gap interference(ACS/IBB) overlap with wanted CC: companies are encouraged to provide the impact of interference to wanted signal power ratio of [15-26]dB to the ΔRIBNC for PCC and SCC. #### Issue 3-4: Discussion on DL fragmented carriers in TDD bands - Option 2: Separate LOs are assumed for UL and DL for TDD band RAN4#114bis #### Issue 2-4: Discussion on applicability for PCell and SCell A more detailed proposal based on the tentative agreement in online discussion: - To address the potential impacts on the DL performance for PCell, RAN4 agrees to further discuss the following aspects: - To limit the use of this feature to CA configurations where the impact is limited - In the CA configurations where there is limited impact, how to further mitigate the impact by means including restricting the PCC and SCC frequency location, or restricting UE locations (e.g., the feature is not enabled when the UE is located at cell edge where wanted signal level is close to receiver sensitivity)

In this contribution, views on receiver requirements impact were provided.

- Discussion

  - Issue 3-2 Impact to REFSENS requirements due to Tx leakage

As evaluated by companies in previous RAN4 meeting, major impact on REFSENS requirements was Tx leakage due to reduced attenuation on UL carrier frequency with wideband LPF under “single Rx chain mode”.  The performance impact can be diverse pending on the configuration (DL and UL frequency separation, operating frequency) and UE implementation (ADC performance, and NF increase due to AGC adjustment, LPF performance).

Initial evaluation based on collected experienced performance from products were provided in below tables for band n3 with configuration as CA_n3(2A), 5MHz-65MHz-5MHz (PCC-Wgap-SCC):

Figure 1: Link-budget for Self-interference of CA_n3(2A)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -97 | -92.3 | / | / |
| FE loss | / | 4 | 4 | / | / |
| Duplexer output | -23 | -101 | -96.3 | / | / |
| LPF attenuation | 12 | 1 | 1 | / | / |
| LPF output (dBm) | -36.4 | -102 | -97.3 | / | / |
| Noise floor for basic REFSENS | / | / | / | -101 | / |
| NF increase due to AGC adjustment | / | / | / | 10 | / |
| Noise floor | / | / | / | -91 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -96.3 | / | / |
| ADC output (dBm) | / | / | / | / | -104.4 |
| SNR after ADC (dB) | / | / | / | / | -14.0 (PCC) -14.6 (SCC) |

Observation 1: With initial evaluation based on experienced product performance, REFSENS performance degradation (ΔRIBNC ) can be 13 dB for PCC and 13.6dB for SCC for CA_n3(2A), 5MHz-65MHz-5MHz (PCC-Wgap-SCC).

  - Issue 3-3 Impact to in gap requirements - Scenario 1: For image of in-gap interference not overlap with wanted CC

For in-gap interference, we need to evaluate whether relaxation on ACS/IBB is needed.

- For ACS case 2, the wanted carrier power is fixed value as -56.5dBm/-25dBm for wanted carrier and interference carrier. The received power wanted carrier is much higher than Tx leakage power and the impact due to Tx leakage is negligible considering duplex can provide ~50dBc attenuation.

- For ACS case 1, both wanted carrier power and interference power are configured relative to REFSENS with fixed offset value(s). We assume the input power for both wanted signal and interference signal will be adjusted based on REFSENS relaxation with ΔRIBNC.

- For IBB1/IBB2, wanted signal power is configured relative to REFSENS with fixed offset value (6dBc). We assume the input power for wanted signal will be adjusted based on REFSENS relaxation with ΔRIBNC.  The interference signal power is fixed value as -56dB/-44dBm.

For ACS case 1 and IBB testing, the wanted carrier power is relative value compared to REFSENS.

Based on initial analysis, if wanted signal power can be set based on adjust REFSENS then the impact to ACS and IBB requirements is marginal.

Figure 2: Link-budget for in-gap blocked w/o image overlapping of CA_n3(2A)

| Duplexer isolation (dB) | 50 |  |
| --- | --- | --- |
| LPF attenuation (dB) | 12 |  |
| NF increase due to AGC adjustment (dB) | 10 |  |
| ΔRIBNC adjustment due to self-interference | 13dB for PCC | 13.6 dB for SCC |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | -1.2 | -1 |
| ACS case 2 | -1 | -1 |
| IBB Case 1 | 4.2 | 5 |
| IBB Case 2 | 4.2 | 5 |

Observation 2: There is no impact to ACS case 2 requirements due to Tx leakage.

Observation 3: If same relaxation on REFSENS can be applied for ACS1/IBB1/IBB2 test cases, then the impact to ACS/IBB requirements is marginal.

Proposal 1: At least same relaxation for REFSENS shall be applied for ACS/IBB test cases on “Single Rx mode” for the case w/o image overlapping.

  - Issue 3-3 Impact to in gap requirements - Scenario 2: For image of in-gap interference overlap with wanted CC

With in-gap interference overlapping with wanted CC, by assuming UE receiver still have capability with 25dBc image rejection capability.

Figure 3: Link-budget for in-gap blocked with image overlapping of CA_n3(2A)

| Duplexer isolation (dB) | 50 |  |
| --- | --- | --- |
| LPF attenuation (dB) | 12 |  |
| Image rejection ratio (dB) | 25 |  |
| NF increase due to AGC adjustment (dB) | 10 |  |
| ΔRIBNC adjustment due to self-interference | 13dB for PCC | 13.6 dB for SCC |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | -6.5 | -6 |
| ACS case 2 | -6.5 | -6.5 |
| IBB Case 1 | 0.6 | 1.2 |
| IBB Case 2 | -9.2 | -8.6 |

Above calculation is also aligned with theoretical analysis.  The potential impact to ACS/IBB requirements with image of in-gap interference overlapped with wanted CC is pending on the power ration between in gap interference and wanted signal power.

Observation 4: For the case in gap interference overlapped with wanted carriers, performance degradation will be observed once power ration between In-gap interference and wanted carriers is over than UE image rejection capability.

To avoid the impact to ACS/IBB , restriction with power difference between wanted carrier and interference in gap less than 20dB can solve the issue.

Proposal 2: Restrict the received power difference between wanted carrier and interference in gap shall be less than [20dB] as one condition for “single Rx operation mode”.

  - Issue 2-4: Discussion on applicability for PCell and SCell

Based on evaluation results from companies, the DL performance degradation due to Tx leakage or image of in-gap blocker are varied pending on configuration (DL-UL separation, Wgap length, CHBW) , UE implementation performance on AGC adjustment, ADC design and image rejection capability  and UE mobility state (uplink Tx power).

When RAN4 specify Rx requirements including REFSENS, ACS and in-band blocking requirements, we always assumed worst case which can be different compared to real network scheduling.

It’s difficult to set-up a hard rule for requirements relaxation and DL performance impact on PCC and SCC which make restriction to deploy this feature if DL performance can be manageable during real network deployment with proper network scheduling.

Observation 5: DL performance degradation due to Tx leakage or image of in-gap blocker are varied pending on configuration (DL-UL separation, Wgap length, CHBW) , UE implementation performance on AGC adjustment, ADC design and image rejection capability  and UE mobility state (uplink Tx power).

Proposal 3: During WI phase if any, separate REFSENS and ACS and IBB requirements for “single Rx chain mode” shall be introduced (with extra ΔRIBNC ) as per band per configuration basis

  - The allowed relaxation shall be discussed and decided case by case manner per operators’ request on target band combinations with associated configuration.

Proposal 4:  RAN4 shall consider mechanism for UE to monitor DL performance impact and exit from “single Rx mode” if the large performance degradation observed which can be indicated to NW.

- Conclusion

In this contribution, views on receiver requirements impact were provided.

Observation 1: With initial evaluation based on experienced product performance, REFSENS performance degradation (ΔRIBNC ) can be 13 dB for PCC and 13.6dB for SCC for CA_n3(2A), 5MHz-65MHz-5MHz (PCC-Wgap-SCC).

Observation 2: There is no impact to ACS case 2 requirements due to Tx leakage.

Observation 3: If same relaxation on REFSENS can be applied for ACS1/IBB1/IBB2 test cases, then the impact to ACS/IBB requirements is marginal if image of in-gap interference not overlap with wanted CC

Proposal 1: At least same relaxation for REFSENS shall be applied for ACS/IBB test cases on “Single Rx mode” for the case w/o image overlapping.

Observation 4: For the case in gap interference overlapped with wanted carriers, performance degradation will be observed once power ration between In-gap interference and wanted carriers is over than UE image rejection capability.

Proposal 2: Restrict the received power difference between wanted carrier and interference in gap shall be less than [20dB] as one condition for “single Rx operation mode”.

Observation 5: DL performance degradation due to Tx leakage or image of in-gap blocker are varied pending on configuration (DL-UL separation, Wgap length, CHBW) , UE implementation performance on AGC adjustment, ADC design and image rejection capability  and UE mobility state (uplink Tx power).

Proposal 3: During WI phase if any, separate REFSENS and ACS and IBB requirements for “single Rx chain mode” shall be introduced (with extra ΔRIBNC ) as per band per configuration basis

  - The allowed relaxation shall be discussed and decided case by case manner per operators’ request on target band combinations with associated configuration.

Proposal 4:  RAN4 shall consider mechanism for UE to monitor DL performance impact and exit from “single Rx mode” if the large performance degradation observed which can be indicated to NW.

Reference

[1] R4-2420337, WF on NR FR1 DL fragmented carriers study, MTK

[2] R4-2502873, WF on NR FR1 DL fragmented carriers study, MTK