# 3GPP TSG-RAN WG4 Meeting #115   	R4-2505790

# Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on Study of testability and interoperability for AI/ML mobility

Agenda Item:	7.23.4

Document for:	Discussion

# 1Introduction

Discussion on testability and interoperability for AI/ML mobility was still going on. Here below we attach the RRM agreements on testability and interoperability in previous RAN4 meeting:

###### Agreements on testability and interoperability in RAN4#112-bis Issue 3-1-1: New testability aspects - Agreement - According to the SID, RAN4 to leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - RAN4 will need to consider at least the following testability aspects for AI/ML-enabled mobility SI: - Testing in FR1 - Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction (for scenario 3) - Testing cell-level L3-RSRP measurements - Consideration of serving and neighbor cells - Issue 3-2-1: Testing in FR1 - What way of testing to consider for FR1: - Candidate Options: - Option 1: OTA testing - Option 2: Conducted testing - Option 3: Both OTA and Conducted - Agreement: - RAN4 to consider Option 2 as a baseline option for testing in FR1. - Option 2: Conducted testing ###### Agreements on testability and interoperability in RAN4#113 Issue 3-1-1: Testing setup - Agreement - RAN4 to further discuss: - Whether and how legacy testing setup for L3 measurements can be reused for FR1 and FR2 Issue 3-2-1: Testing goal - Agreement - As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved. Issue 4-1-2: Prediction consistency in time domain Agreement: - To ensure prediction consistency in time domain, RAN4 to discuss how to model different time-varying characteristics per cell/site due to moving UE trajectories. - Discuss how to incorporate controlled randomness and the extent of time-domain variation and correlation - RAN4 to prioritize discussion on FR1. Issue 4-1-3: Multiple cells in the testing setup Agreement: - RAN4 to consider two cells: serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction. - FFS: RAN4 to study if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

Based on the achieved agreements, we would like to share our further consideration on the testability and interoperability issues for AI/ML mobility and provide our proposals.

- Discussion

RAN2 has discussed the RRM measurement prediction for several meetings, and has agreed to focus on L3 filtered cell level measurement, while deprioritize L3 filtered beam level prediction cases.

3 sub-use cases listed below are considered for cell level RRM measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated

- Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)

- Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

In RAN4#112bis meeting, it was agreed to focus on the three high priority scenarios as ranked by RAN2.

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

To verify the accuracy of the predicted L3 filtered cell-level measurement result, sufficient randomness and variation in time and spatial domain of Tx beam has to be configured in the test and emulated by the test equipment. Similar as the discussion in NR_AIML_Air WI, the channel model assumption for RRM measurement prediction need to be agreed first. There are two channel model assumptions: AWGN channel and Fading channel. The AWGN channel is a deterministic channel, where signal power will be constant for both spatial and time domain. While, the Fading channel is a random channel, where variation in spatial and/or time domain can be included. It’s hard to verify the prediction functionality introduced by AI using AWGN, since there is no any uncertainty in both spatial and time domain. Just as the progress achieved in Channel model assumption for AI based BM, we think the fading channel should be assumed in the test. For both the scenarios of FR1 to FR1 inter-frequency intra-cell temporal domain case B and FR1 to FR1 inter-frequency inter-cell frequency domain, we understand the variation in time domain would be enough for FR1 scenario, so the TDL channel model is preferred.

Proposal 1: For testing in FR1 RRM measurement prediction, the configuration of 1 Tx and TDL channel model can be considered.

Furthermore, RAN4 agree to discuss the prediction consistency in time domain considering the impact on UE trajectories in last meeting. AI model is expected to be trained with real-world data, which is collected under dynamic UE mobility conditions. However, traditional test methods are executed under certain SNR level with velocity-induced doppler shifts. We understand the UE trajectory would introduce large-scaled propagation parameters impact for test setup. In TR 38.843 section 6.3.1, 3 options of UE trajectory model are considered. We think RAN4 should take these three models as starting point.

Proposal 2: Starting from FR1 RRM measurement prediction, RAN4 to select one reference UE trajectory model to further study the prediction consistency impact.

In previous meeting, RAN4 has concluded that conductive testing method could be used for AI/ML mobility test in FR1. We understand that in conductive test, the UE and TE are connected directly with RF cable. The TE can adjust the transmission power to mimic the cell to be measured and the cell to be predicted similar to legacy approach. So, we agree to assume that the actual ground truth is available at the TE. However, we see some potential issues. As we support to use fading channel, the channel conditional is random in time domain. UE and TE need to align the time point when UE is supposed to conduct measurement. Otherwise, it’s hard to get the actual ground truth. We believe this issue will be figured out in L3 RSRP prediction accuracy discussion.

Proposal 3: For testing in FR1 RRM measurement prediction, the way the ground truth is extracted has impact on the testing procedure:

- If the ground truth of L3-RSRP is calculated by TE, new test procedure on how TE determine the ideal L3-RSRP need to be further studied.

- If the ground truth of L3-RSRP is reported by UE, the legacy test procedure can be taken as baseline.

Specifically, for the FR1 to FR1 inter-frequency inter-cell frequency domain scenario, the multi-cell configuration need to be pay more attention. To verify the accuracy of the predicted L3 filtered cell-level measurement result in this scenario, multi-cell configuration in the test need to be further studied. We think the configuration in this case depends on the number of cells to be predicted, which needs to wait for RAN2 decision. If we only verify that case one cell to be measured and one cell to be predicted, we understand that the legacy test setup can be used as baseline. If we need to verify the case one cell to be measured and multiple cells to be predicted, we think the test setup need to be further studied.

Proposal 4: For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, the legacy test procedure can be taken as baseline for single cell prediction. FFS for multi-cells prediction.

For FR2 RRM measurement prediction, we understand that we face the similar testability issues as in NR_AIML_Air WI. We think the conclusions on testability achieved in NR_AIML_Air WI can be taken as baseline.

Proposal 5: For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline.

- Conclusion

Proposal 1: For testing in FR1 RRM measurement prediction, the configuration of 1 Tx and TDL channel model can be considered.

Proposal 2: Starting from FR1 RRM measurement prediction, RAN4 to select one reference UE trajectory model to further study the prediction consistency impact.

Proposal 3: For testing in FR1 RRM measurement prediction, the way the ground truth is extracted has impact on the testing procedure:

- If the ground truth of L3-RSRP is calculated by TE, new test procedure on how TE determine the ideal L3-RSRP need to be further studied.

- If the ground truth of L3-RSRP is reported by UE, the legacy test procedure can be taken as baseline.

Proposal 4: For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, the legacy test procedure can be taken as baseline for single cell prediction. FFS for multi-cells prediction.

Proposal 5: For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline.

- Reference

- R4-2504901, WF on RRM requirements for FS_NR_AIML_Mob, Nokia