# 3GPP TSG-RAN WG4 Meeting #115   	R4-2505792

# Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on RRM core requirements for LP-WUS/WUR

Agenda Item:	7.28.5.2

Document for:	Discussion

# 1Introduction

In RAN4#114 meeting, the RRM impacts on low-power wake-up signal and receiver for NR (LP-WUS/WUR) was discussed and a WF was approved [1]. In this contribution, we provide our further analysis on RRM impacts of LP-WUS/WUR features.

- Discussion

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Based on the discussion in previous meetings, RRM measurement relaxation case#1 and case#3 has already be agreed. In case 1, i.e., fully offloading case, MR stops perform any RRM measurement and LP-WUR performs serving cell measurement. There is still FFS on the condition and details for the MR neighboring cell measurement. From our perspective, this is the case where UE could perform the greatest level of relaxation measurement. We prefer not to consider any neighbor cell measurement in this case.

Proposal 1: RAN4 not to consider any neighbor cell measurement in case 1, except when higher priority measurement configured.

In case 2, MR has RRM relaxation on serving cell measurement (no neighbour cell measurement is needed) and LP-WUR performs serving cell measurement. We think this is the case UE in the cell center, when UE does not need to measure neighbor cell, as well as no higher priority inter-f or inter-RAT configured. From our perspective, this is the typical case for starting LP-WUR monitoring and no need to start MR measurement. So, we prefer to de-prioritize this case.

Proposal 2: RAN4 to de-prioritize the discussion on case 2.

Issue 1-1-2: Threshold for switch between different cases

Issue 1-1-5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

As we understand that the LP-WUR measurement is initially introduced to guarantee the performance of LP-WUS monitoring, we think the LR measurement is not expected to start later than the LP-WUS monitoring. Also, we see little advantage in early starting of the LR measurement. If UE still use MR for paging reception, it is reasonable to use MR for RRM measurements as MR also need measurement for T/F synchronization. In this case, the LR measurement results seems useless. So, we prefer to define the same entry/exit condition of LP-WUR measurement and LP-WUS monitoring. For the entry/exist conditions for LP-WUS monitoring, we understand that this issue is under discussion in RAN1 and RAN2. We think RAN4 should wait for RAN1/2 progress. RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered by other working group(s) if needed.

RAN4 also discuss the entry/exit conditions for MR RRM measurement relaxation. Based on our analysis in the previous section, we think it is more reasonable that the MR RRM relaxation is executed when LR is used for serving cell measurement. If the LR is not doing measurement, the evaluation of measurement relaxation can only depend on serving cell measurement in MR, which is aligned with Rel-16 power saving functionality. If the LR is doing RRM measurement while MR is also doing normal RRM measurement, there seems little benefit but more power consumption. So, from our perspective, the entry/exit conditions for MR RRM measurement relaxation should also be the same as LP-WUR serving cell measurement. However, we understand RAN2 is currently working on the criteria design, it is suggest RAN4 to focus on the relaxation cases.

Proposal 3: RAN4 to concentrate on requirements discussion while the entry/exit criteria (including threshold) for fully offloading/MR RRM measurement relaxation/ LP-WUS monitoring should be based on RAN2 decision.

Issue 1-1-10: Considerations on higher priority frequency layer

In accordance with the legacy measurement procedure for MR only UE, measurements for neighbor cell could be divided into two parts. One is for intra-frequency measurements and NR inter-frequency or inter-RAT frequencies measurements with equal or lower priority, and the decision for MR to conduct these measurements hinges on the results from the serving cell measurements. When the signal quality of the serving cell reduce to certain threshold, the intra-frequency measurements and NR inter-frequency or inter-RAT frequencies measurements with equal or lower priority will be triggered. The other one is for NR inter-frequency or inter-RAT frequencies with higher priority, and the decision for MR to conduct the measurements depend on NW configuration. In this case, as long as NW configuration indicates the priority information, UE needs to perform measurements of higher priority NR inter-frequency or inter-RAT frequencies. We think the handling on NR inter-frequency or inter-RAT frequencies measurements with higher priority should also follow the legacy.

In last meeting, RAN4 agreed to support relaxed higher priority frequency layer search at case 1. The next issue is about the relaxation factor. During last meeting discussion, there was a tentative agreement:

Tentative agreement:

- For case 1, when the serving cell is above threshold for inter-frequency or inter-RAT neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search.

  - Note: K2 is 60

We are fine with the relaxation factor K2=60, which is the same number as the relaxation factor introduced for higher priority frequency search in R16 RRM relaxation. However, we think there is no need to further restrict the threshold of the higher priority relaxation from RAN4 side. Because we understand that UE is not expected to perform neighbor cell measurement in case1 if no higher priority measurement configured by NW, which means that the serving cell measurement results from MR should be at least no less than the threshold for NR inter-frequency and inter-RAT measurements, i.e. SnonIntraSearchP and SnonIntraSearchQ, and the threshold for intra-frequency measurements, i.e. SIntraSearchP and SIntraSearchQ. So, we prefer that in RAN4 perspective, we only define the relaxed measurement behavior other than the enter condition.

- Proposal 4: In case 1, when NR inter-frequency or inter-RAT frequencies measurements with higher priority are configured by NW, MR is expected to perform relaxed higher priority frequency layer search with the relaxation factor of K2=60.

- Detailed LP-WUR requirements

Issue 1-2-2: Periodicity for LP-WUR RRM requirements Agreement: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. Issue 1-2-4-1: On LR measurement filtering requirement Agreement: - The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR.

The above attached agreements on detailed LP-WUR requirements were achieved in last RAN4#114 meeting. We will further discuss the open issues below.

During previous discussion, the measurement delay requirement in IDLE/Inactive mode for LP-WUR was divided into two cases. One is for LP-WUR based on LP-SS, the other one is for LP-WUR based on SSB. For the former case, agreement was achieved that measurement delay requirements for LP-WUR based on LP-SS shall be defined based on LP-SS periodicity. For the later case, i.e. LP-WUR based on SSB, it was agreed that the measurement delay requirements are defined based on LO periodicity.

For the case LP-SS is not configured, we understand only OFDM-based LP-WUR can work in this case. The OFDM-based LP-WUR obtains synchronization based on SSB. Basically, we think it is quite similar as MR. Also, the current agreement, i.e. based on LO periodicity, also aligns with measurement delay requirements for MR. As no upper bound has been defined for MR, we prefer not to define upper bound for LR either.

Proposal 5: RAN4 not to define upper bound for the measurement delay requirements for LP-WUR when LP-SS is not configured.

For the case LP-SS is configured, the agreement was achieved that measurement delay requirements for LP-WUR based on LP-SS shall be defined based on LP-SS periodicity. However, based on the latest RAN1 progress, we can see that the range of LP-SS periodicity is still open. There is possibility that the LP-SS periodicity can be configured to 80ms or 160ms, which is comparable to the periodicity of SSB. We prefer to set a lower limit for the measurement delay requirements in case the periodicity of LP-SS can be configured to small value like 80ms or 160ms. Or, if majority companies can accept, we also support to define the measurement delay requirements based on max(LP-SS, LO periodicity).

Proposal 6: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms, RAN4 to

- Option 1: define the lower limit for measurement delay requirements;

- Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity).

Based on the RAN4 discussion progress, RAN4 agreed to define single set of LR evaluation requirement for the transition between different cases, while the applicability of LR evaluation requirements is still under discussion.

The agreements on entry and exit conditions reached under current RAN2 discussions are summarized below.

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | For serving cell RRM relaxation: - MR threshold and optional LR threshold - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation | FFS |

Taken legacy case into consideration, the transition between cases including:

- from RRM measurement fully offloading case to legacy case

- from RRM measurement fully offloading case to RRM measurement relaxation case

- from RRM measurement relaxation case to legacy case

- from legacy case to RRM measurement relaxation case

- from legacy case to RRM measurement fully offloading case

- from RRM measurement relaxation case to RRM measurement fully offloading case

- from LP-WUR monitoring case to legacy case

- from legacy case to LP-WUR monitoring case

We think it is straightforward that the LR evaluation requirements should be applicable for ①, ②, ⑦, as the corresponding exit condition is based on LR threshold.

For ③ and ⑥, RAN2 has not decided the corresponding exit condition and the corresponding entry condition can optionally include LR threshold. Based on this current discussion status, we think at least the LR evaluation requirements should be applicable when NW configure LR threshold.

For ④, ⑤ and ⑧, We understand LR evaluation requirements applicability is related to the LR status. In previous meeting, RAN4 achieved the agreements that when and how to turn on LR for serving cell measurement is up to UE implementation. We understand the LR evaluation requirement should be applicable for the case UE evaluate whether the LR measurement result is above the configured threshold. And the LP-WUR status has to be “on” to obtain the LR measurement result. If RAN4 agreed not to restrict when and how UE to turn on LR, we think it is reasonable that the LR evaluation requirement is not applicable for the case LR is “ON” at legacy state.

Proposal 7: The LR evaluation requirement is applicable to the following transition evaluation:

- from RRM measurement fully offloading case to legacy case

- from RRM measurement fully offloading case to RRM measurement relaxation case

- from RRM measurement relaxation case to legacy case

- from RRM measurement relaxation case to RRM measurement fully offloading case

- from LP-WUR monitoring case to legacy case

Proposal 8: The LR evaluation requirement is not applicable to the following transition evaluation:

- from legacy case to RRM measurement relaxation case;

- from legacy case to RRM measurement fully offloading case;

- from legacy case to LP-WUR monitoring case.

- On MR wake up delay related requirements for LP-WUR

The wake up delay related requirements has been discussed for several meetings. Based on our understanding, RAN1 has agreed to define the wake-up delay. Based on RAN1’s agreement, the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. It is clear that the wake-up delay define in RAN1 considered the MR synchronization time. So, we think RAN4 does not need to define the wake-up delay requirement.

Proposal 9: RAN4 no need to define additional requirements for MR wake up delay, since RAN1 has decided the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR and RAN1 LS R1-2501624 Issue 1-2-6-1: On the number of SSBs Agreement: Aim to conclude the issue by Tuesday in RAN4 #115. Issue 1-2-6-2: On SSBs number for different SSB periodicities Agreement Same SSB number for different SSB periodicity Agreement When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay. Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity

In last meeting, RAN1 sends a LS on the wake-up delay and is asking RAN4 to check on the assumption related to the calculation on wake-up delay. During discussion in last meeting, RAN4 reached consensus on using the same number of SSBs for MR synchronization for different SSB periodicities. While the issue on the number of SSBs for MR synchronization was still open. Here below, we further share our views on this open issue.

Agreement in RAN1#118bis Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state. - FFS the values - Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective Agreement in RAN1#120 For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities Agreement in RAN1#120bis On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, consider the following alternatives for possible down-selection in RAN1#121. - Alt 1: Do not report for SSB periodicities other than 20ms - Note: LP-WUS is not supported for SSB periodicities larger than 20ms - Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns): SSB periodicity (ms); Wake-up delay (ms) UE capability 1; Wake-up delay (ms) UE capability 2; Wake-up delay (ms) UE capability 3. [5]; [x]; [x]; [x]. [10]; [x]; [x]; [x]. 20; [70]; [500]; [900]. [40]; [x]; [x]; [x]. [80]; [x]; [x]; [x]. [160]; [x]; [x]; [x]. - Alt 3: It is up to RAN4

Based on RAN1 agreed definition on wake-up delay, we understand the delay should include two parts of time: the time for MR ramping up and the time for MR synchronization. For the former one, we think it is related to the MR sleep mode. RAN1 support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state. From our side, it is reasonable to take different values for MR ramping up considering different UE implementation.

For the later one, the time for MR synchronization is related to the SSB periodicity, as UE is expected to accomplish synchronization based on the SSB. During discussion in last meeting, a discussion point recommended is on whether LR can provide rough time. For our side, it is related to the UE implementation, and should not be restricted from RAN4 requirement perspective. Normally, RAN4 define the requirement considering the worst case, so we prefer to define the requirement with the assumption that LR can not provide rough time. Another point is about the side condition for UE to execute the MR synchronization. As RAN4 is currently evaluate LR accuracy under side condition of -3 dB, we think it is reasonable to take the same side condition for MR synchronization. In this way, considering the components for synchronization, we think it needs to account for time required for AGC settings, SSB detection and the fine time-frequency tracking. We propose to use 5 number of SSBs for MR synchronization, not differentiating the MR sleep state.

Proposal 10: RAN4 to use 5 number of SSBs for MR synchronization, not differentiating the MR sleep state.

- Accuracy

Issue 1-2-1: General on LR accuracy requirement Agreement: No dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin relative to the threshold in the core requirement. Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements Agreement: For FR1: - [3.5] dB can be used for core requirements for LP-RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot [x, x = -3] dB - [3.5] dB can be used for core requirements of SSB based RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot [y, y = -3] dB - Note: [2 or 2.5] dB RF impairment margin is assumed

RAN4 discussed the LR accuracy and side conditions requirements in last meeting and the above agreement was achieved. For the specific numbers of LR accuracy used for core requirements were in the square brackets. In our understanding, the main issue here is the number of RF impairment margin.

In RAN4#114 meeting, RAN4 RF has reached agreement related to the NF:

Issue 2-1-1: SNR, NF and IM value for FR1 LP-WUR Agreement: - For the FR1 requirements targeting at bands <2.5GHz - The IM+NF values are - Set 1: 18dB - Set 2: 13.5dB - Send an LS to RAN1 with RAN4 decision of two sets of : NF (assume IM is 2.5dB)

Considering the MR NF is assumed as 9.0 dB, we can see that the LR NF is extremely larger than the MR NF. In our understanding, the RF impairment margin for MR is assumed as 2.5 dB. We prefer to assume a larger number for the RF impairment margin of LR to account for the LR NF deterioration.

Proposal 11: RAN4 to assume the 3.5 dB RF impairment margin for LR.

- LP-WUR requirements at RRC_CONNECTED mode

In RRC_CONNECTED mode, as clearly described in the objectives, UE RRM/RLM/BFD/CSI measurements are performed by MR. So, we think there should be no RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode expected. However, we are fine not to jumps to conclusions in this early stage.

Proposal 12: Postponed the discussion on RAN4 impact of LP-WUS/WUR feature in RRC_CONNECTED mode until more progress in other groups.

- Conclusion

Proposal 1: RAN4 not to consider any neighbor cell measurement in case 1, except when higher priority measurement configured.

Proposal 2: RAN4 to de-prioritize the discussion on case 2.

Proposal 3: RAN4 to concentrate on requirements discussion while the entry/exit criteria (including threshold) for fully offloading/MR RRM measurement relaxation/ LP-WUS monitoring should be based on RAN2 decision.

- Proposal 4: In case 1, when NR inter-frequency or inter-RAT frequencies measurements with higher priority are configured by NW, MR is expected to perform relaxed higher priority frequency layer search with the relaxation factor of K2=60.

Proposal 5: RAN4 not to define upper bound for the measurement delay requirements for LP-WUR when LP-SS is not configured.

Proposal 6: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms, RAN4 to

- Option 1: define the lower limit for measurement delay requirements;

- Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity).

Proposal 7: The LR evaluation requirement is applicable to the following transition evaluation:

- from RRM measurement fully offloading case to legacy case

- from RRM measurement fully offloading case to RRM measurement relaxation case

- from RRM measurement relaxation case to legacy case

- from RRM measurement relaxation case to RRM measurement fully offloading case

- from LP-WUR monitoring case to legacy case

Proposal 8: The LR evaluation requirement is not applicable to the following transition evaluation:

- from legacy case to RRM measurement relaxation case;

- from legacy case to RRM measurement fully offloading case;

- from legacy case to LP-WUR monitoring case.

Proposal 9: RAN4 no need to define additional requirements for MR wake up delay, since RAN1 has decided the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

Proposal 10: RAN4 to use 5 number of SSBs for MR synchronization, not differentiating the MR sleep

state.

Proposal 11: RAN4 to assume the 3.5 dB RF impairment margin for LR.

Proposal 12: Postponed the discussion on RAN4 impact of LP-WUS/WUR feature in RRC_CONNECTED mode until more progress in other groups.

- Reference

- R4-2504908, WF on RRM requirements for NR_LPWUS, Vivo

- R1-2501624, LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode, RAN1