# 3GPP TSG-RAN WG4 Meeting #115	R4-2505799

# Malta, Malta, 19th – 23rd May, 2025

Title: 	PC1.5 MPR evaluations for intra-band CA

Agenda item:	7.1.2.1.3

Source:	Huawei, HiSilicon

Document for:	Discussion

# Introduction

In the last meeting the WF [1] was agreed with tentative MPR values for intra-band contiguous ULCA with non-contiguous RB allocation (with TxD). Moreover, it was suggested to make evaluations for intra-band non-contiguous ULCA (with dualPA) for this meeting.

## 1.2 MPR values for intra-band contiguous ULCA with non-contiguous RB allocation (with TxD) - The MPR values for PC3(1Tx) and PC2(2Tx) listed in the below table is just for reference Modulation; PC3 (1Tx); PC2 (2Tx); PC1.5 (2Tx). Handheld; Handheld; Handheld. inner; Outer12; Outer23; inner; Outer12; Outer23; inner; Outer12; Outer23. DFT-s-OFDM; PI/2 BPSK; 2.5; 6; 11.5; 41; 8.5; 14.5; [4.5-5]; [9-9.5]; [15.5-17]. QPSK; 2.5; 6; 11.5; 41; 8.5; 14.5; [4.5-5]; [9-9.5]; [15.5-17]. 16QAM; 3; 6; 11.5; 41; 8.5; 14.5; [4.5-5]; [9-9.5]; [15.5-17]. 64QAM; 5; 6; 11.5; 6; 8.5; 14.5; [6.5-7]; [9-9.5]; [15.5-17]. 256QAM; 6.5; 6.5; 11.5; 7.5; 8.5; 14.5; [8-8.5]; [9-9.5]; [15.5-17]. CP-OFDM; QPSK; 3.5; 7; 12; 4.51; 9; 15.5; [5-5.5]; [9.5-10]; [16.5-17]. 16QAM; 3.5; 7; 12; 4.51; 9; 15.5; [5-5.5]; [9.5-10]; [16.5-17]. 64QAM; 5; 7; 12; 6; 9; 15.5; [6.5-7]; [9.5-10]; [16.5-17]. 256QAM; 7.5; 7.5; 12; 8.5; 9; 15.5; [9-9.5]; [9.5-10]; [16.5-17]. NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported. Agreement: - Further discussion in upcoming meetings based on the tentative values in the above table. - The MPR contributions from companies is captured in the spreadsheet below: ## 1.3 MPR values for intra-band non-contiguous ULCA (with dualPA) Agreement: - Further discussion in upcoming meetings

In this discussion paper, we will address the MPR values for the issue 1.2, then the importance of properly modelling Reverse- inter modulation (R-IMD) in simulations is discussed.

# Discussion

## MPR values for intra-band contiguous ULCA with non-contiguous RB allocation:

In the last meeting our MPR evaluations were presented in [2]. Moreover, ranges of MPR values were agreed in the last meeting. In Table 1 we summarized the MPR values to be considered for intra-band contiguous ULCA with non-contiguous RB allocation, after further internal evaluations.

Proposal 1: MPR values for intra-band contiguous ULCA with non-contiguous RB allocation

Table 1 Non-contiguous RB allocation for Power Class 1.5 with dual Tx

| Modulation |  | PC1.5 (2Tx) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | FWA |  |  |
|  |  | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 |
| DFT-s-OFDM | QPSK | 4.5 | 9.5 | 17 | 4 | 9 | 16.5 |
|  | 16QAM | 4.5 | 9.5 | 17 | 4 | 9 | 16.5 |
|  | 64QAM | 6.5 | 9.5 | 17 | 6 | 9 | 16.5 |
|  | 256QAM | 8.5 | 9.5 | 17 | 8 | 9 | 16.5 |
| CP-OFDM | QPSK | 5 | 10 | 17.5 | 4.5 | 9.5 | 17 |
|  | 16QAM | 5 | 10 | 17.5 | 4.5 | 9.5 | 17 |
|  | 64QAM | 7 | 10 | 17.5 | 6.5 | 9.5 | 17 |
|  | 256QAM | 9.5 | 10 | 17.5 | 9 | 9.5 | 17 |
| NOTE 1: Void NOTE 2: Outer 1 MPR for QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |

## Reverse-IMD for intra-band non-contiguous ULCA (with dualPA)

The Reverse IMD happens, because the diversity PA output signal passes through the output port of the main PA and makes an inter-modulation product with the main signal. This phenomenon is different than PA forward mixing, where the diversity PA output signal, through PCB, gets inside the main PA input port and creates an inter modulation product. The R-IMD is an important factor. As in [3] Figure 4 shows that with 50 dB isolation, @4dB power back off,  the R-IMD level difference with respect to the main signal is 68 dB. It should be mentioned that in RAN4, we assume 10 and 20dB isolations for handheld and FWA UEs, respectively. Therefore, one can expect R-IMD level difference compared to the main signal to be smaller than 68dB with RAN4 assumptions, which shows why the R-IMD is important and cannot be modelled as PA forward mixing.

Fig. 4 [3]Reverse IMD of 10 watt GaN SSPA (coupling 50 dB).

Observation 1: For MPR evaluations with DualPA, the reverse IMD should not be modelled in a same way as an IMD due to PA forward mixing. They are different phenomena.

Proposal 2:  Proponents of MPR simulations with DualPA are invited to describe the model that they used to include R-IMD.

# Conclusion

Proposal 1: MPR values for intra-band contiguous ULCA with non-contiguous RB allocation

Table 1 Non-contiguous RB allocation for Power Class 1.5 with dual Tx

| Modulation |  | PC1.5 (2Tx) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | FWA |  |  |
|  |  | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 |
| DFT-s-OFDM | QPSK | 4.5 | 9.5 | 17 | 4 | 9 | 16.5 |
|  | 16QAM | 4.5 | 9.5 | 17 | 4 | 9 | 16.5 |
|  | 64QAM | 6.5 | 9.5 | 17 | 6 | 9 | 16.5 |
|  | 256QAM | 8.5 | 9.5 | 17 | 8 | 9 | 16.5 |
| CP-OFDM | QPSK | 5 | 10 | 17.5 | 4.5 | 9.5 | 17 |
|  | 16QAM | 5 | 10 | 17.5 | 4.5 | 9.5 | 17 |
|  | 64QAM | 7 | 10 | 17.5 | 6.5 | 9.5 | 17 |
|  | 256QAM | 9.5 | 10 | 17.5 | 9 | 9.5 | 17 |
| NOTE 1: Void NOTE 2: Outer 1 MPR for QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |

Observation 1: For MPR evaluations with DualPA, the reverse IMD should not be modelled in a same way as an IMD due to PA forward mixing. They are different phenomena.

Proposal 2: Proponents of MPR simulations with DualPA are invited to describe the model that they used to include R-IMD.

# Reference

[1]	“R4-2505100, WF on HPUE RF requirements, Qualcomm, Mediatek, Skyworks, LGE.”

[2]	“R4-2503279, MPR evaluations for contiguous intra-band CA with NC allocation for PC1.5 TxDiv, Huawei.”

[3]	A. Katz, D. McGee, C. Brinton, and J. Qiu, “Sensitivity and mitigation of Reverse IMD in power amplifiers,” in 2011 IEEE Topical Conference on Power Amplifiers for Wireless and Radio Applications, Phoenix, AZ, USA: IEEE, Jan. 2011, pp. 53–56. doi: 10.1109/PAWR.2011.5725374.