3GPP TSG-RAN WG4 Meeting #115		R4-2505823

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.10.5.2

Source:		Samsung

Title:	Discussion on RRM impacts by using NGSO in the test

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#106 meeting. The objectives of performance part of NTN testing for NGSO are described in [1] as:

The objectives of performance part for enhanced requirements and test methodology for NR and IoT NTN include: NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

From this meeting, RAN4 will start the discussion on RRM performance part of NTN testing for NGSO. In this contribution, we provide our view of the RRM performance part of NTN testing for NGSO.

- Discussion

In existing core requirements of uplink timing for Rel-17 NTN are specified in 7.1C UE transmit timing for Satellite Access.

### 7.1C.1 Introduction The UE shall have capability to follow the frame timing change of the reference cell in RRC_CONNECTED state. The uplink frame transmission takes place NTA+NTA offset+NTA,adjcommon+NTA,adjUE×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements. ### 7.1C.2 Requirements The UE initial transmission timing error shall be less than or equal to Te_NTN where the timing error limit value Te_NTN: - is specified in table 7.1C.2-1 for FR1-NTN. #### 7.1C.2.1 Gradual timing adjustment When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE shall adjust the timing such that timing error is to within Te_NTN. The reference timing shall be NTA+NTA offset+NTA,adjcommon+NTA,adjUE×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules: Then the definition and requirements for Tp and Tq

For IoT-NTN, the core requirements use similar UE pre-compenstion:

## 7.20A UE transmit timing for NB-IoT for Satellite Access ### 7.20A.1 Introduction The Category NB1 UE shall have capability to follow the frame timing change of the connected eNode B. The uplink frame transmission takes place NTA+NTA,offset+NTA,adjcommon+NTA,adjUE×Ts before the reception of the first detected path (in time) of the corresponding downlink frame from the reference NB-IoT cell. UE shall use the serving NB-IoT cell as the reference cell for deriving the UE transmit timing. UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are defined in the following requirements. ### 7.20A.2 Requirements The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20A.2-1.

In R17 test case for uplink timing, AWGN is used for test.

In previous meetings, the NGSO channel models have been discussed in BSRF session. The latest agreements are approved in [2].

According to the agreement:

It is up to TE implementation which method is used to derive the varying Doppler shift and propagation delay for NGSO test. - Note: the details models for Kepler’s equation and equation of motion can be referred in Appendix

From the key models for Kepler’s equation, the key factor for RRM impacts is related the doppler shift and the propagation delay generation.

As time varies, the doppler shift and the propagation delay are updated at every new time point.

In the Rel-17 uplink timing requirements, as shown above, the time delay has been already included for UE side estimation and pre-compensation in the NTA,adjUE. The core requirements have considered the time vary delay according to satellite. For the definition and requirements of Tp and Tq for gradual timing adjustment, there is no need to change the rate.

Proposal 1: UE side estimation and pre-compensation for time delay has been included in core requirements for uplink timing. No impacts for core requirements for NGSO channel model testing for both NR NTN and Iot NTN.

Proposal 2: For gradual timing adjustment for Tp and Tq, No impacts for core requirements for NGSO channel model testing.

In the test cases, for NR, there are two TCs for NR-NTN:

#### A.14.3.1.1 NR UE Transmit Timing Test for FR1 ### A.14.3.2 Timing advance for satellite access A.14.3.2.1 SA FR1 timing advance adjustment accuracy

Proposal 3: First, NGSO channel model should be added in these TCs. The corresponding configuration of NGSO channel should be added.

- During the tests, UE position is needed to be added and set by AT command at the beginning of T1. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

For the Transmit timing test, the test equipment verified the timing based on UE estimated value.

Proposal 4: For NR-NTN

- For transmit timing test, no impacts for the test procedure and test requirements.

- For the timing advance test, FFS on whether addition margin should be added in the test requirements.

In Iot-NTN test, five TCs are specified:

## A.13.4 Timing and signalling characteristics for satellite access ### A.13.4.1 UE transmit timing for satellite access A.13.4.1.1 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access #### A.13.4.1.2 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access #### A.13.4.1.3 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access ### A.13.4.2 UE timing advance for satellite access A.13.4.2.1 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access A.13.4.2.2 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access

Proposal 5: For five TC for IoT-NTN, use the same principle as NR-NTN.

- Conclusion

In this contribution, we analyze the potential impact on RRM performance requirements for NTN testing for NGSO and provide our proposals:

Proposal 1: UE side estimation and pre-compensation for time delay has been included in core requirements for uplink timing. No impacts for core requirements for NGSO channel model testing for both NR NTN and Iot NTN.

Proposal 2: For gradual timing adjustment for Tp and Tq, No impacts for core requirements for NGSO channel model testing.

Proposal 3: First, NGSO channel model should be added in these TCs. The corresponding configuration of NGSO channel should be added.

- During the tests, UE position is needed to be added and set by AT command at the beginning of T1. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

Proposal 4: For NR-NTN

- For transmit timing test, no impacts for the test procedure and test requirements.

- For the timing advance test, FFS on whether addition margin should be added in the test requirements.

Proposal 5: For five TC for IoT-NTN, use the same principle as NR-NTN.

- Reference

[1] RP-242901, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN

[2] R4-2504700, Way Forward for [114bis][323] NTN_testing_NGSO_channel_model_and_demod, Samsung