3GPP TSG-RAN WG4 Meeting #115		R4-2505824

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.31.5.1

Source:		Samsung

Title:	Discussion on RRM requirements of RedCap in Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

In previous meetings, we have already discussed all the RRM requirements of RedCap in NTN and latest agreements are included in [1]. There are still some open issues in [1]. In this contribution, we continue to discuss the FFS parts of RRM requirements for (e)RedCap UE with NR NTN operating in FR1-NTN bands.

- Discussion

##### Issue 1-2-1-1a: The proximity distance between two SMTCs <Way Forward>: - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms. - Y=5 ms

In existing spec for handheld NTN UE,

-	the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4 ms.

From the RedCap UE perspective, the relaxed proximity can get better performance for complexity and processing.

Therefore, in last meeting, we discussed the relaxed proximity is based on 4ms + X ms. (4ms is the performance of normal UE). The X exactly depends on UE capability of UE processing. Some companies proposed 1ms to 3s. First, we don’t see different levels are needed. One value of X is enough for RedCap UE supported this WI. Y = 5ms to allow 1ms relaxation for Redcap UE is accepted.

Proposal 1: For RedCap UE in NTN, the proximity distance can be relaxed compared to the legacy NTN UE for 4ms + X (1ms) = 5ms (Y).

#### Issue 3-1-1: General HD-FDD related impact Online agreement Agreement: - The legacy requirements and applicable conditions can be reused as baseline: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - [Minimum requirement for L1 indication for RLM and LR] - Scheduling availability of UE performing intra/inter measurements RAN4 to further discuss: - The legacy requirements and applicable conditions can be reused as baseline for defining requirements of RedCap UE in NTN scenario, and discuss whether there is spec impact for the following requirements by HD-FDD: - FFS: Scheduling availability of UE during L1-RSRP measurement - Other requirement is not precluded - Candidate options for the potential spec impact for each of the above requirements (if needed): - Option a: the requirement is extended, e.g., the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Option b: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Option c: The requirement applies when there is at least one RS within each measuremen interval. - FFS the same or different option for DRX and non-DRX

For the highlighted “Minimum requirement for L1 indication for RLM and LR”, we don’t see the issue to re-use the legacy TN requirements in NTN. The only difference of RedCap UE from normal UE is:

When DRX is not used for HD-FDD RedCap UEs, TIndication_interval,RedCap is max(10 ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in clause 8.1B.2 if the RLM-RS resource is SSB, or TCSI-RS specified in clause 8.1B.3 if the RLM-RS resource is CSI-RS, under the following condition - For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period. In case DRX is used for HD-FDD RedCap UEs, TIndication_interval,RedCap is Max(10ms, 1.5 × DRX_cycle_length, 1.5 × TRLM-RS,M)) if DRX_cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX_cycle_length is greater than 320ms, under the following condition - For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period.

Proposal 2: For Minimum requirement for L1 indication for RLM and LR, legacy requirements in TN RedCap and applicable conditions can be reused.

For the “Scheduling availability of UE during L1-RSRP measurement”, in legacy TN Redcap UE requirements:

#### 9.5B.6.1 Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1 For FD-FDD and TDD RedCap UEs, there are no scheduling restrictions due to L1-RSRP measurement performed on SSB and CSI-RS configured as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1. For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1. #### 9.5B.6.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement. - For FD-FDD and TDD RedCap UEs, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement. - For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1.

These prioritized rules have been introduced for HD-FDD RedCap UE in TN.

Observation 1: In Rel-17 RedCap discussion, it was agreed that CBD evaluation is always prioritized over the UL transmission for RedCap HD-FDD.

The CBD evaluation can be based on SSB or CSI-RS.

For SSB collides with UL transmission, existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN, which DL SSB measurement is prioritized. According to this, the Rel-17 existing requirements can be reused.

For CSI-RS (DL signal) collides with UL transmission, in RAN1#120b, the default priority rule for case 4 for other use cases, DL is also prioritized.

Proposal 3: For Scheduling availability of UE during L1-RSRP measurement, SSB based measurement is prioritized. CSI-RS based measurement for DL is prioritized. So, Rel-17 legacy requirements of CBD evaluation is always prioritized over the UL transmission for RedCap HD-FDD can be reused.

#### Issue 3-2-1: The impact on NR Handover due to HD-FDD Online agreement Agreement: - Option 1: agree on the principle below and further discuss the wording in the CR: - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - One SSB is available during T∆ - One SSB is available during TIU - Option 2: No spec impact

For the impact on NR Handover due to HD-FDD, in general option 1 is accepted. If no spec impact in option 2 which means no requirements, the performance cannot be ensured.

Proposal 4: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted.

#### Issue 4-1-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands? <Way Forward>: - Proposal 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). - Proposal 2: Introduce new capability to declare support over RedCap over NTN. - Proposal 3: RAN4 to not discuss or conclude on UE capability signaling and structure for RedCap NTN, unless specific requests are received from other working groups via LSs.

From the technical view, Proposal 1 and 2 are both fine. However, the only concern of proposal 1 is the release independent issue. If use P1, whether RedCap UEs will support from Rel-17 RedCap UE in NTN from Rel-17 and Rel-18 eRedCap UEs in NTN from Rel-18? It needs to add release independent principle in the spec. If use Proposal 2, it is clear only Rel-19 UEs need to meet the requirements for (e)RedCap UE to NTN-FR1 from Rel-19. It is not so urgent for Rel-17 and Rel-18 UEs to support this feature in the commercial market. From this perspective, we support Proposal 2.

Proposal 5: Introduce new capability to declare support over RedCap over NTN.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for (e)RedCap UE with NR NTN operating in FR1-NTN bands:

Proposal 1: For RedCap UE in NTN, the proximity distance can be relaxed compared to the legacy NTN UE for 4ms + X (1ms) = 5ms (Y).

Proposal 2: For Minimum requirement for L1 indication for RLM and LR, legacy requirements in TN RedCap and applicable conditions can be reused.

Observation 1: In Rel-17 RedCap discussion, it was agreed that CBD evaluation is always prioritized over the UL transmission for RedCap HD-FDD.

Proposal 3: For Scheduling availability of UE during L1-RSRP measurement, SSB based measurement is prioritized. CSI-RS based measurement for DL is prioritized. So, Rel-17 legacy requirements of CBD evaluation is always prioritized over the UL transmission for RedCap HD-FDD can be reused.

Proposal 4: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted.

Proposal 5: Introduce new capability to declare support over RedCap over NTN.

- Reference

[1] R4-2504912, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT