3GPP TSG-RAN WG4 Meeting #115		R4-2505825

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.31.5.2

Source:		Samsung

Title:	Discussion on RRM requirements of other aspects in Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

In last meeting, RAN4 received LS from RAN2 in [1]. In this contribution, we discuss the potential impacts in RAN4 RRM for SMTC.

- Discussion

In the LS from RAN2, RAN2 assume to add further enhancement of SMTC in Rel-19 NTN Phase 3 as:

For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe.

In existing RAN4 requirements for both RRC idle mode and RRC connected mode, the assumption is the periodicity of multiple SMTCs for the same ssb frequency are the same although multiple SMTC can be supported in Rel-17 NTN.

Observation 1: Periodicity of multiple SMTCs for the same ssb frequency are the same although multiple SMTC can be supported in Rel-17 NTN.

In existing requirements in RRC idle mode like cell reselection, the SMTC & DRX are used to define such requirements.

Table 4.2C.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). FR1. 0.32; 1; 11.52 x N1 x M2 (36 x N1 x M2); 1.28 x N1 x M2 (4 x N1 x M2); 5.12 x N1 x M2 (16 x N1 x M2). 0.64; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). NOTE 1: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 4 upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; otherwise M2=1. Where, NSMTC is the number of SMTCs configured by SAN If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. NOTE 2: The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment..

In last meeting, companies discussed the real SSB periodicity and SMTC configure misalignment for different satellites.

In the yellow part, it was introduced by smtc2 in Rel-16 enhancement.

But now, in latest RAN2 spec, the motivation is still the same. From the neighbor cells in the same satellite or different satellite, the SSB periodicity may different. In addition, for different satellites, the propagation delays are also different. By using different SMTCs with different SMTC periodicities is the approach which RAN2 would like to use in Rel-19.

If two different SMTC periodicities are supported, RAN4 existing requirements can be reused if the requirements applied by using the shorter SMTC.

Proposal 1: For idle mode, considering cell reselection requirements, if two different SMTC periodicities are supported, RAN4 existing requirements can be reused if the requirements applied by using the shorter SMTC. It is feasible.

Another question is whether there is any issue if:

RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

The mechanism is quite similar as Rel-17 NTN. Firstly, it is only for idle/inactive mode, in Rel-17, up to 4 SMTCs are supported. If the enhancement is just increasing the maximum of number from 4 to 6. No such difference for RAN4 requirements.

Proposal 2: For idle/inactive mode, if the enhancement is increasing the maximum of number from 4 to 6, No RRM impacts.

However, in existing requirements in RRC connected modes, for the same frequency, RAN4 have so many requirements, and some requirements depends on SMTC like L3 measurement. Some requirements are considering the collision of SMTCs like serving cell L1-RSRP measurements. Some requirements are assuming the SMTC periodicity from serving cell for the same carrier. The rules and updates for each requirement may be different. RAN4 need to check them case by case.

Proposal 3: For connected mode, if configuring two different SMTC periodicities (with different offsets) in one frequency layer, we observe RAN4 have RRM impacts in different requirements. The support and modification for each requirement may be different. RAN4 need to check and discuss them case by case. The workload is not expected to be finished in one meeting.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for DL coverage enhancement:

Observation 1: Periodicity of multiple SMTCs for the same ssb frequency are the same although multiple SMTC can be supported in Rel-17 NTN.

Proposal 1: For idle mode, considering cell reselection requirements, if two different SMTC periodicities are supported, RAN4 existing requirements can be reused if the requirements applied by using the shorter SMTC. It is feasible.

Proposal 2: For idle/inactive mode, if the enhancement is increasing the maximum of number from 4 to 6, No RRM impacts.

Proposal 3: For connected mode, if configuring two different SMTC periodicities (with different offsets) in one frequency layer, we observe RAN4 have RRM impacts in different requirements. The support and modification for each requirement may be different. RAN4 need to check and discuss them case by case. The workload is not expected to be finished in one meeting.

- Reference

[1] R2-2503068, LS on SMTC enhancements, RAN2