3GPP TSG-RAN WG4 Meeting #115                                                                    R4-2505833

Malta, 19th – 23rd May, 2025

Agenda item:         7.21.2

Title:	Discussion on General Aspects for AI/ML Air Interface

WI/SI:	NR_AIML_air-Core

Release:	Rel-19

Source:	Korea Testing Laboratory

Document for:	Discussion

# 1. Introduction

This contribution addresses key challenges in validating AI/ML models after deployment, in alignment with the AI/ML air interface Work Framework. Given the stochastic nature of AI/ML behavior, the increasing reliance on UE-side training and field-based updates, and the diversity of model change types, it is essential to establish validation strategies that balance robustness, signaling efficiency, and test feasibility.

We first distinguish between Fine-tuning and Model Update, and propose that conformance testing requirements be aligned with their respective impact levels. While threshold-based performance monitoring remains under discussion for many AI/ML use cases, it is recognized as a practical approach for enabling controlled validation and reducing unnecessary fallback signaling.

In contrast, Beam Management adopts a non-threshold validation metric based on Top-M vs. Top-K set inclusion, as agreed in RAN1 #120bis. To enhance validation efficiency in this case, we propose supporting multi-M evaluation, enabling the UE to compute multiple accuracy metrics from shared inference results and report them compactly.

Finally, this document highlights the risk of overfitting, even when training is based on field-collected data. In accordance with the agreed RAN1 procedure (AI-Example1), we recommend incorporating explicit validation mechanisms to detect and mitigate overfitting in post-deployment scenarios using independent datasets and structured performance reporting.

2. Discussion

2.1  Post deployment testing

In the last meeting, [R4-2503953, WF on AI/ML air interface study] was summarized as following;

Table 1. Topic 1: Post-deployment testing [WF on AI/ML air interface study, R4-2502953]

|  |  | Option 1 | Option 2 |
| --- | --- | --- | --- |
| 1 | New name | Pre-activation functionality/model update testing | Post deployment management based on LCM |
| 2 | Description | Conduct the validation of a change in AI functionality before its deployment/activation in already deployed UEs • Validation takes into account the UE hardware in which the model is to be deployed/activated. | Using performance monitoring and LCM procedures • Performance monitoring will be designed in other groups • RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. |
| 3 | Sub-options | Possible collection of input data to a model during conformance testing for use later on. |  |
| 4 | Further potential clarifications (not resolved) | The following proposals have been made for option 1 but not discussed. o Whether the testing of a new model is on a device or in a lab. o Whether models that have been updated post conformance testing should be kept in a device. o Whether parallel operating of one model and (in-device) testing of another can be assumed. o Whether signaling from a UE of changes/updates are needed. o Whether there is any relation to GCF procedures. o Whether it is possible to differentiate “major” and “minor” model changes and only apply option 1 to major changes. - Definition of Fine-tuning and Model update - Model update can have a performance impact. - Fine-tuning might have little performance impact. o Hybrid Validation approach - A hybrid approach integrating both pre-activation functionality/model update testing (Option 1) and post-deployment management based on LCM (Option 2) aims to provide a balanced validation strategy. | The following proposals have been made for option 1 but not discussed. - Whether the metric used for performance testing and the metric used for monitoring can be aligned. Whether monitoring used for post-deployment testing should be NW sided only. |
| 5 | Significant issues | Whether the option is reasonable or would cause too much complexity and test time burden when introducing updates. | Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.) Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs. |

- Regarding whether it is possible to differentiate between “major” and “minor” model changes and apply Option 1 only to major changes, the definitions of Fine-tuning and Model Update, along with a Hybrid Validation approach—integrating both pre-activation functionality/model update testing (Option 1) and post-deployment management based on LCM (Option 2)—aim to provide a balanced validation strategy.

In our understanding, the intent behind establishing a trustworthy post-deployment testing procedure was not solely to avoid costly, extensive, and repetitive conformance testing. Rather, the primary objective was to ensure that fine-tuned or updated AI/ML models not only pass the conformance requirements but also consistently maintain their intended performance under real-world operational conditions in the field.

A hybrid validation approach combining Option 1 (Lab. Testing) and Option 2 (performance monitoring based on LCM) has been proposed to ensure the robustness of AI/ML-based functionalities deployed in UEs. However, utilizing Option 1 exclusively for basic functionality checks may introduce several critical issues.

Specifically, the fundamental limitation arises from the fact that verifying only functionality configuration correctness (i.e., whether a model follows specification-based execution on a given UE) does not account for performance requirements, even when the DUT is tested in a controlled conformance testing environment. Since conformance tests define the testing framework, all legacy teste cases should align with these requirements to ensure compliance with predefined functional and minimum performance requirements.

Additionally, reliance on Option 2 necessitates frequent signaling exchanges between UEs and the network, potentially inducing excessive signaling overhead, increased latency in detecting performance degradation, and heightened complexity in distinguishing between model-related issues and external network conditions.  Thus, a more efficient procedure can be achieved by monitoring the function of the LCM in field environments, as controlling the threshold for the range of performance could reduce the time-consuming aspects of the process mentioned in Section 2.2. Moreover, the absence of clearly defined signaling protocols from the UE regarding AI fine tuning or model updates can further exacerbate inconsistencies in the network's ability to identify, diagnose, and promptly respond to performance fluctuations.

Given these challenges in both functionality validation and performance monitoring, it is crucial to differentiate between minor modifications, such as fine-tuning, and more extensive model updates. Their impact on performance and testing requirements differs significantly. Since fine-tuning only involves modifying minor weight values while keeping the overall model structure intact, a functionality check alone may suffice to ensure not only conformance testing but also the stability of field operations.

Observation 1: “Post deployment management based on LCM” requires frequent signaling exchanges between UEs and the network.

Proposal 1: Optimize LCM-based performance monitoring in post-deployment testing to reduce unnecessary signaling.

Table 2 Example of Gradient Descent for Fine tuning and Model update

|  | Fine tuning | Model update |
| --- | --- | --- |
| 1 | Fine-tuning updates a subset or all weights of a pretrained model using gradient descent: The basic model is , if applied the Gradient Descent, | In contrast, a Model update replaces all model parameters, meaning all layers undergo full retraining, The basic model is , if applied the Gradient Descent, |
| 2 |  |  |
| 3 | : Fine-tuned weights : Pretrained weights : Learning rate : Input data : Inference output : Loss function evaluated on new dataset : Gradient of the loss function with respect to model parameters : Binary mask (0 for frozen layers, 1 for updated layers) | : New model weights from scratch or initialized differently. There is no direct dependency on unlike Fine-Tuning. can be a new architecture (e.g., Transformer instead of CNN) : Input data : Inference output |
| Model’s structure | Retaining the pre-trained model’s structure. | Involving full replacement. |
|  | Ref. : Bengio, Y., Courville, A., & Vincent, P. (2012). Representation learning: A review and new perspectives. IEEE Transactions on Pattern Analysis and Machine Intelligence, 35(8), 1798-1828. |  |
| Learning | Adjusting existing weights. | Might be retraining from scratch |
|  | Ref. Goodfellow, I., Bengio, Y., & Courville, A. (2016). Deep Learning. MIT Press. |  |
| Learning Data | Typically using an extended version of the original dataset. | Introducing entirely new datasets |
|  | Ref. He, K., Zhang, X., Ren, S., & Sun, J. (2016). Deep Residual Learning for Image Recognition. Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (CVPR), 770-778. |  |
| Modification | Updating only part of the model. | Possibly, replacing entire architecture. |
|  | Ref. : Vaswani, A., Shazeer, N., Parmar, N., Uszkoreit, J., Jones, L., Gomez, A. N., Kaiser, L., & Polosukhin, I. (2017). Attention is all you need. Advances in Neural Information Processing Systems (NeurIPS), 30. |  |

Table 3 Terms in TR 38.843

[3GPP TR 38.843] 3 Definitions of terms, symbols and abbreviations 3.1 Terms Model update: Process of updating the model parameters and/or model structure of a model.

Given Table 2 and Table 3, we would like to propose the following definitions:

Proposal 2:  Define Fine-tuning as the process of adjusting existing model weights while retaining the pre-trained model’s structure. Fine-Tuning modifies only a subset of model parameters, allowing for targeted adaptation to new data without altering the overall model architecture. Due to the minimal impact on structural integrity, conformance testing requirements for fine-tuned models should be limited to the functionality configuration.

Proposal 3: Define Model Update as a process that involves full replacement of an AI/ML model, requiring retraining from scratch. Model updates may also include architectural modifications, such as transitioning between different deep learning frameworks (e.g., CNN to Transformer). Due to the substantial changes in model behavior, conformance testing must be conducted comprehensively, covering both functional validation and real-world performance testing.

- Regarding whether the metrics used for performance testing and the metric used for monitoring can be aligned.

In AI/ML-based Life Cycle Management (LCM) operations, it is crucial to differentiate between performance testing and continuous performance monitoring to ensure efficient signaling and accurate validation of AI/ML model performance. Performance testing is conducted to validate the effectiveness of a fine-tuned or updated AI/ML model before its full deployment, whereas monitoring is a continuous process within the live network that detects performance degradation and triggers necessary model adaptation actions such as switching, fallback, or deactivation. Using the same signaling mechanisms for both performance testing and monitoring leads to excessive signaling overhead, particularly in UE-to-network communication, thereby increasing network congestion and degrading overall system efficiency.

In post-deployment testing, when an AI/ML model undergoes validation, the UE does not necessarily need to engage in full LCM decision-making processes such as model switching or fallback. However, if the UE continues to operate under normal LCM functions during this period, several issues arise. First, AI/ML model performance exhibits stochastic variations, meaning that transient fluctuations in performance may not necessarily indicate the need for fallback or switching. If the UE prematurely switches to another AI/ML model or falls back to a legacy algorithm before a sufficient validation period, the performance testing process becomes inconclusive, making it impossible to assess the impact of fine-tuning or model updates accurately. Second, unnecessary signaling overhead occurs when the UE continuously sends messages to the network for switching or fallback decisions based on performance fluctuations that are within an acceptable range. This leads to excessive control plane traffic, inefficient use of radio resources, and increased processing burden on both the UE and the network.

To address these issues, it is necessary to establish clear distinctions between performance testing metrics and monitoring metrics to ensure efficient signaling and accurate AI/ML validation. Performance testing metrics should be fine-grained and focused on controlled validation in a structured test environment, whereas monitoring metrics should be aggregated and lightweight to minimize signaling during normal LCM operation. During AI/ML validation testing, switching or fallback actions initiated by the management framework should be temporarily disabled to prevent premature interventions triggered by natural fluctuations in model behavior. This can be achieved by operating the UE in a dedicated test mode, setting validation thresholds that are less sensitive than those used in runtime conditions, and limiting any fallback or switching responses unless the observed degradation is sustained and exceeds a critical threshold.

For example, to further improve flexibility, a field such as reportingLogic can be introduced to allow the network to specify the direction of the reporting condition—whether the UE should report when a performance metric drops below a configured threshold or when it exceeds it. This enables two distinct strategies: in validation scenarios, the UE may report only when the metric falls below the target value, thereby highlighting underperformance while avoiding excessive signaling when the model is functioning well; in contrast, during live operation, the UE may report only when the metric rises above a certain level, such as when confidence in model output becomes sufficiently high to trigger an operational change.

For instance, the network may configure the UE to report only if the model inference accuracy drops below 90% during the validation phase, allowing early detection of weak model performance with minimal signaling during otherwise stable operation. Conversely, in a deployment scenario, the network may configure reporting to occur only when the model confidence exceeds 85%, enabling selective feature activation based on high-confidence AI predictions. These configurations can be expressed through RRC signaling parameters that define the relevant metric, threshold, and reporting logic, thereby enabling adaptive and efficient AI/ML lifecycle control.

By implementing such flexible signaling logic, the validation and deployment phases of AI/ML models can be better aligned, minimizing signaling overhead while supporting robust, real-world testing of model behavior in 5G networks.

Proposal 4: The difference metric shall be used for performance monitoring in post-deployment testing and the metric used for monitoring in normal operation.

③ Regarding whether monitoring used for post-deployment testing should be NW-sided only

If the post-deployment AI/ML model management relies solely on network decision (i.e., network-initiated AI/ML management), the UE must explicitly signal to the network when a model update occurs. This is because the network is responsible for AI/ML lifecycle control, and without clear signaling, it cannot reliably track or validate the updated model state. Implicitly inferring model updates—such as through UE-assigned or pre-defined Model IDs—may introduce ambiguity, particularly in Step C and Step D of the agreed procedure in RAN1#116-bis.

According to RAN1#116-bis, Step C allows for AI/ML models to be developed or updated at the UE side, and Step D requires the UE to report model-related information to the network. However, Step D does not explicitly require the UE to indicate whether the model has been newly trained or updated. Therefore, to ensure robust synchronization and lifecycle management, a dedicated signaling mechanism (e.g., AI-ML-Model-Update-Report) becomes essential—especially when the network solely manages the AI/ML lifecycle (as in Alt.1 or Alt.4 where model ID is assigned by the network or by rule).

Alternatively, if the AI/ML lifecycle management allows for UE-side autonomy (e.g., UE decision, event-triggered behavior as configured by the network), then the need for explicit signaling may be reduced or conditionally optional. In such cases, the UE can autonomously validate updated models and take actions such as fallback, switching, or continued operation based on pre-configured thresholds. The network does not need to be notified of every update, since lifecycle decisions are partially delegated to the UE under controlled conditions.

Importantly, the use of performance monitoring, such as defined in RAN1 Option 2, provides an opportunity to streamline post-deployment validation. If the network can evaluate the UE-reported monitoring metrics—such as Top-M vs. Top-K beam prediction accuracy—as indirect evidence of model behavior, the dependency on event-driven signaling can be reduced. This enables a more scalable validation process, while still preserving synchronization, especially when monitoring is configured to align with inference timing.

In summary, whether or not explicit UE signaling is required following a model update depends directly on the chosen validation and lifecycle management architecture:

- If the network performs validation (NW-sided), explicit signaling is mandatory.

- If validation is UE-driven and event-triggered, signaling may be optional.

- Monitoring metrics can act as a complementary or alternative mechanism to support this decision flow.

Therefore, the treatment of post-deployment monitoring—whether it is strictly NW-sided or also UE-capable—must be clearly aligned with the signaling requirements in the core 3GPP specification.

Table 4 Agreement (RAN1 #116-bis)

Agreement (RAN1 #116-bis) From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

Observation 2: Current agreements (RAN1 #116-bis, Step C/D) do not explicitly mandate signaling of UE-side model updates, creating ambiguity in certain model ID assignment scenarios.

Proposal 5: Clearly define and introduce an explicit UE signaling procedure such as AI-ML-Model-Update-Report within the core 3GPP specifications, which becomes mandatory if post-deployment validation relies solely on network-side validation (NW-sided only).

2.2 Efficient Process for Post deployment testing

The AI/ML, especially those based on probabilistic methods or neural networks, often exhibit stochastic variation in performance compared to deterministic legacy algorithms based on optimization schemes. These variations occur within a certain range where negligible degradation in AI/ML generalization is acceptable. Therefore, AI/ML-based performance requirement should allow for ac controlled range of deviations, making relevant procedures feasible under  Life Cycle Management(LCM)  operations. Additionally, it is necessary to define and discuss what constitutes a negligible degradation to ensure consistency in testing and validation.

From our perspective, we emphasize the importance of LCM functions for both conformance testing and post-deployment evaluation (Option1 and Option2). It is presently understood to test and or validate the minimum performance of AI/ML functionality/feature.

For gNB sided decision for the LCM operation, based on the results of the model monitoring from UE, gNB can decide on model updating/switching or fallback to legacy schemes. There are implementation areas such as the fallback performance thresholds that determine when and how gNB commands the fallback to the UE depending on the performance monitoring.  When configuring the conformance test environment or the validation due to update or fine tuning or addition/removal of models, the test and or validation setup may decide that since gNB manages the instructions for the LCM operation, the results are pass or fail based on the performance monitoring reports.

For UE-side inference, the LCM function provides performance monitoring and supports various actions such as selection, switching, activation, deactivation, and fallback. In the event of fallback, the system can revert to a non-AI/ML function or a legacy algorithm during conformance testing, with the possibility to reactivate the AI/ML functionality/feature (Association ID #1) or switch to another AI/ML physical model implemented in the UE under the scope of the same AI/ML functionality (Association ID #1). This means that both UE-side inference and LCM are running simultaneously.

As we know, AI/ML performance exhibits stochastic variations, as seen in the statement "AI/ML can achieve about 70%~80% beam prediction accuracy" in TR 38.843. This implies that the AI/ML performance results can be defined within a range that satisfies the minimum requirements. In other words, decisions can be made by monitoring average results to determine whether the AI/ML functionality/feature meets or fails to meet the minimum performance standards.

An efficient approach could involve separating the conformance testing processes for AI/ML functionalities/features. This approach can be applied to Option 1 and Option 2 for post-deployment testing. Below, we outline an efficient approach for conformance testing that applies to both Option1 and Option 2 in post-deployment testing. In the following, we present an efficient approach in case of conformance testing.

One mechanism to further enhance the efficiency of validation, particularly in the context of post-deployment testing, is the use of multiple values of M (e.g., M=1, 2, 4) configured by the network in the CSI report configuration. The UE can perform inference and reuse the same measurement results to calculate accuracy metrics for each configured M, and report the results in a single composite message. This structure enables comprehensive performance profiling with minimal signaling overhead and computational reuse. The multi-M evaluation not only supports accurate and flexible validation under different performance thresholds but also facilitates the determination of fallback conditions based on a more nuanced view of AI/ML model behavior across various beam selection granularities. By enabling multiple performance metrics to be computed and embedded within a single report message using shared measurement and inference results, the structure significantly reduces the number of reporting transactions and overall signaling overhead.

To support efficient validation of AI/ML-based beam management functionalities, it is proposed to allow the configuration of multiple values of M (e.g., M = 1, 2, 4) in the CSI report configuration under Option 2. The UE may reuse the same inference output and measurement results to evaluate accuracy for each configured M, and report the results in a single composite message. This enables: Granular profiling of AI/ML model performance across different prediction strictness levels, Support for adaptive fallback threshold determination, Reduction in signaling overhead without additional measurement or inference, Minimal complexity at the UE side due to shared computation.

This mechanism provides a unified structure applicable to both conformance and post-deployment testing, and introduces a novel framework that may serve as a basis for future standard-essential innovations related to AI/ML model validation and lifecycle management.

[RAN1 #120Bis Wuhan, China, April 7th – 11th, 2025] Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling

Firstly, during conformance testing when the LCM management is operating on the UE sided, performance monitoring and reporting are required but the transitions to deactivation and fallback should be avoided. Our understanding is that this can be configured by setting the threshold or the management of the fallback/deactivation function is set to a very low performance requirement (AI/ML test mode), in order to avoid the deactivation and the fallback. For example, based on R2-2407848, the threshold for conformance testing and/or post-deployment testing (Option 1 and/or Option 2) can be configured in Step 3, within the RRCReconfiguration message, alongside inference-related parameters. The UE can then use this configuration to send an Applicable Functionality Report in Step 4 as a response. Alternatively, the threshold for conformance testing and/or post-deployment testing (Option 1 and/or Option 2) can be configured in Step 5, within the RRCReconfiguration message, as part of the inference configuration.

In addition, performance monitoring based on the configuration of monitoring resources—such as CSI-RS or SSB sets—can be used by the UE to derive the performance metric (e.g., Top-M vs Top-K accuracy) as defined under RAN1 Option 2. By aligning the monitoring configuration with the inference timing, and enabling composite evaluation across multiple M values, the monitoring process itself can serve as the basis for validation, eliminating the need for separate procedures. The reporting conditions and triggering criteria may also be linked to the monitoring results, ensuring that validation occurs efficiently and consistently with minimal signaling overhead.

From this approach, both "Network decision, network-initiated AI/ML management" and "UE decision, event-triggered as configured by the network" can achieve efficient post-deployment AI/ML model management by implementing a reporting criteria threshold, which regulates the conditions under which the UE reports AI/ML performance status to the network, thereby minimizing unnecessary signaling overhead. Simultaneously, a distinct deactivation/fallback prevention threshold can be applied to ensure that the AI/ML model remains active throughout the validation phase, preventing premature switching or fallback due to stochastic variations in AI/ML performance.

Observation 3: Most AI/ML use cases are expected to rely on threshold-based performance monitoring for validation. This approach allows for quantifiable assessment of stochastic AI/ML behavior, and validation efficiency may depend on how tolerable thresholds are configured in relation to the intended operation phase.

Observation 4: Beam management adopts a distinct approach to performance monitoring, which is not based on threshold comparisons but on set-inclusion logic—specifically, whether at least one of the Top-M monitored beams is included in the Top-K predicted beams in the inference report. This metric structure was agreed in RAN1 #120bis for UE-assisted monitoring (Option 2).

Proposal 6: Enable flexible configuration of multiple M values in beam management validation. The UE may reuse inference and measurement results to compute Top-M vs. Top-K accuracy for each configured M and report them in a single composite message, supporting multi-granularity evaluation with minimal signaling overhead.

Table 5. [2] R2-2407848, “LS on applicable functionality reporting for beam management UE-sided model”

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

Secondly, the normal operation, which represents the standard threshold state, is when the conformance test evaluates the AI/ML functionality/feature, and the UE’s LCM management can report to the gNB, such as through a “Management Request” or “Management Decision Request” [38.843]. During this signaling procedure, various outcomes can be expected (inference). The test is considered to have passed if no “Management Request” or “Management Decision Request” (such as a fallback request) is triggered during the conformance test. Occasional occurrences of a “Management Request” or “Management Decision Request” (e.g., fallback) may be acceptable. However, if a “Management Request” or “Management Decision Request” is sent to the gNB (Test setup) most of the time during the test, it would be deemed inadequate.

It becomes challenging to determine whether the minimum requirements have been met because AI/ML performance exhibits stochastic variation, especially if signaling procedures are nearly continuous. To address the difficulty in interpreting stochastic triggering behavior, the monitoring function can be used to continuously track model performance through structured metrics (e.g., Top-M vs Top-K accuracy), providing a more stable and quantifiable basis for validation. This reduces reliance on event-driven management signaling and enhances the robustness of the conformance decision-making.

Observation 5: It has different behavior to verify the minimum performance of AI/ML functionality/feature depending on “Signalling procedures for model and functionality LCM” in TR 38.843.

Thirdly, to verify the function of LCM management, a separate or a fake AI/ML inference mimic environment.

In summary, RAN4 can perform the conformance test, as well as Option 1 and Option 2 for post-deployment testing, efficiently by using these methods, if RAN1/RAN2 define the detail fallback and the deactivation mechanism considering on the view or opinion of RAN4’s. It should be discussed within RAN4 group for the way of efficiency process for conformance test and post-deployment testing, by using the mechanism of the fallback / the deactivation if it might not be possible to define the mechanism in detail at RAN1/RAN2 since the reason of the implementation area.

Proposal 7: It should be discussed within the RAN4 group regarding the efficient process for conformance testing and post-deployment testing, using fallback mechanisms or deactivation, especially if it might not be possible to define the mechanism in detail at RAN1/RAN2 since the reason of the implementation area.

Table 6 Signalling procedures for model and functionality life cycle management [38.843]

| For example: Network decision, network-initiated AI/ML management (replace Network to TE) | For example: Network decision, UE-initiated AI/ML management(replace Network to TE) |
| --- | --- |
| For example: UE decision, event-triggered as configured by the network(replace Network to TE) | For example: UE autonomous, decision reported to the network(replace Network to TE) |

- In the following, it provides the overfitting problem and the necessity of the validation process.

Overfitting is a common challenge in machine learning (ML), where a model demonstrates strong performance on training datasets but significantly underperforms on new or unseen data. To mitigate this issue, various techniques such as early stopping, data augmentation, regularization, dropout layers, and batch normalization are utilized. From a RAN4 perspective, data augmentation is especially emphasized as a critical method for configuring controlled test environments, whereas other methods should be considered within the implementation scope of UE vendors. However, it should be clearly noted that the conformance tests defined by RAN4 represent minimum performance requirements and may not be sufficient to completely prevent overfitting.

Although field-collected data typically contains greater diversity and realistic noise, thereby reducing the likelihood of overfitting, it does not entirely eliminate the risk. Even when trained or updated with diverse field datasets, deep learning models can still exhibit overfitting due to factors such as data biases or historical dependency on certain data patterns. Thus, dedicated validation processes remain essential to reliably detect and mitigate residual overfitting risks.

Moreover, the recently agreed-upon RAN1 procedures (Agreement RAN1 #116-bis, AI-Example1) explicitly describe UE-sided data collection, training, updating, and reporting procedures, emphasizing that UE-side ML models can be developed or updated based on field-collected data following network-signaled configurations.

Given this RAN1 procedural framework, it becomes clear that even UE-side trained or updated AI/ML models, despite utilizing field-collected data, require explicit validation procedures post-deployment to verify the absence of overfitting. While the diversity and realistic noise in field data inherently reduce the likelihood of overfitting, these characteristics alone cannot completely guarantee model robustness in all possible field deployment scenarios.

Therefore, it is critical that post-deployment scenarios explicitly incorporate validation procedures to confirm whether deployed UE-side models indeed avoid overfitting. A dedicated validation mechanism is necessary, employing independent validation datasets, distinct from the training datasets, to reliably evaluate and confirm the model’s generalization capability. During this validation phase, the Test Equipment (TE) or the Network (NW) could temporarily suppress or adjust Life Cycle Management (LCM) operations such as fallback, deactivation, or model switching, setting thresholds to prevent premature or unnecessary LCM actions. Simultaneously, UEs should continuously report performance metrics derived from the validation datasets, enabling the Network to detect and diagnose potential overfitting.

In summary, considering the agreed procedures by RAN1, it is strongly recommended that validation procedures to detect overfitting be explicitly defined and standardized for post-deployment scenarios. Such procedures will provide a systematic approach to verifying and ensuring model robustness and reliability, complementing field-based model training or updates.

Observation 6: Although UE-side field-data-based training inherently mitigates overfitting risks, the recently agreed RAN1 procedure (AI-Example1) further emphasizes the need for explicit post-deployment validation mechanisms to confirm generalization performance.

Proposal 8: It is proposed that RAN4 explicitly incorporates overfitting validation mechanisms into post-deployment scenarios. Such mechanisms should define clear procedures for utilizing independent validation datasets, performance monitoring, and corresponding NW-UE signaling to robustly detect and mitigate overfitting risks in field-deployed AI/ML models.

3. Conclusion

In this contribution, we provided our viewpoints.

Observation 1: “Post deployment management based on LCM” requires frequent signaling exchanges between UEs and the network.

Proposal 1: Optimize LCM-based performance monitoring in post-deployment testing to reduce unnecessary signaling.

Proposal 2:  Define Fine-tuning as the process of adjusting existing model weights while retaining the pre-trained model’s structure. Fine-Tuning modifies only a subset of model parameters, allowing for targeted adaptation to new data without altering the overall model architecture. Due to the minimal impact on structural integrity, conformance testing requirements for fine-tuned models should be limited to the functionality configuration.

Proposal 3: Define Model Update as a process that involves full replacement of an AI/ML model, requiring retraining from scratch. Model updates may also include architectural modifications, such as transitioning between different deep learning frameworks (e.g., CNN to Transformer). Due to the substantial changes in model behavior, conformance testing must be conducted comprehensively, covering both functional validation and real-world performance testing.

Proposal 4: The difference metric shall be used for performance monitoring in post-deployment testing and the metric used for monitoring in normal operation.

Observation 2: Current agreements (RAN1 #116-bis, Step C/D) do not explicitly mandate signaling of UE-side model updates, creating ambiguity in certain model ID assignment scenarios.

Proposal 5: Clearly define and introduce an explicit UE signaling procedure such as AI-ML-Model-Update-Report within the core 3GPP specifications, which becomes mandatory if post-deployment validation relies solely on network-side validation (NW-sided only).

Observation 3: Most AI/ML use cases are expected to rely on threshold-based performance monitoring for validation. This approach allows for quantifiable assessment of stochastic AI/ML behavior, and validation efficiency may depend on how tolerable thresholds are configured in relation to the intended operation phase.

Observation 4: Beam management adopts a distinct approach to performance monitoring, which is not based on threshold comparisons but on set-inclusion logic—specifically, whether at least one of the Top-M monitored beams is included in the Top-K predicted beams in the inference report. This metric structure was agreed in RAN1 #120bis for UE-assisted monitoring (Option 2).

Proposal 6: Enable flexible configuration of multiple M values in beam management validation. The UE may reuse inference and measurement results to compute Top-M vs. Top-K accuracy for each configured M and report them in a single composite message, supporting multi-granularity evaluation with minimal signaling overhead.

Observation 5: It has different behavior to verify the minimum performance of AI/ML functionality/feature depending on “Signalling procedures for model and functionality LCM” in TR 38.843.

Proposal 7: It should be discussed within the RAN4 group regarding the efficient process for conformance testing and post-deployment testing, using fallback mechanisms or deactivation, especially if it might not be possible to define the mechanism in detail at RAN1/RAN2 since the reason of the implementation area.

Observation 6: Although UE-side field-data-based training inherently mitigates overfitting risks, the recently agreed RAN1 procedure (AI-Example1) further emphasizes the need for explicit post-deployment validation mechanisms to confirm generalization performance.

Proposal 8: It is proposed that RAN4 explicitly incorporates overfitting validation mechanisms into post-deployment scenarios. Such mechanisms should define clear procedures for utilizing independent validation datasets, performance monitoring, and corresponding NW-UE signaling to robustly detect and mitigate overfitting risks in field-deployed AI/ML models.

4. Reference

- R4-2502953, WF on AI/ML air interface study