3GPP TSG-RAN4 Meeting #115	R4-2505850

# Malta, Malta, 19th - 23rd May, 2025

Agenda item:		7.14.3.1

Source:				Keysight Technologies UK Ltd, Nokia

Title:					On Split TRP/TRS Grids for Devices with Limited Battery Capacity

Document for:		Approval

# Introduction

This contribution proovides additional clarifications of the split TRP/TRS measurement grids primarily tailored to device with limited battery capacity such as XR devices.

# Background of Split TRP/TRS Grid Approach

The concept of the split measurement grids is briefly reviewed first. For devices with sufficient battery capacity to last an entire test, TRP/TRS measurements are commonly performed with all grid points assessed in a single test session, i.e., without changing/charging the battery during the test. This is visualized in Figure 1 where all grid points shown in black are assessed without any interruption to charge and/or replace the battery.

Figure 1: 266 measurement grid point assessed in a single test session

On the other hand, for devices with limited battery capacity, TRP/TRS measurements cannot performed with all grid points assessed in a single session, i.e., multiple battery changes/device charging sessions are required. This is visualized in Figure 2 where all grid points shown in different colors represent the different test sessions and in between changes of grid point colors, the device needed to be re-charged or the battery replaced. In this figure, it is assumed that no positioning errors between test sessions are made, i.e., the location of each grid point in red, blue, magenta or cyan coincides with the grid points in black from Figure 1.

Figure 2: 266 measurement grid point assessed in 4 different test sessions without any positioning error between sessions

The repositioning MU for TRP/TRS measurement approaches that require multiple repositionings, must take multiple device positionings into account in the MU calculation though as the positioning of the device in the fixture and/or the phantom could be slightly off compared to the previous/original positioning. This is further visualized in Figure 3 where grids points for any of the 4 sessions could be off from the test session that does not require any re-positioning, see Figure 1.

Figure 3: 266 measurement grid point assessed in 4 different test sessions small positioning errors between sessions

The goal of the split measurement grid approach is to split the full TRP/TRS grid up into multiple coarser grids and perform each TRP/TRS measurement on the coarse grid without the need for a battery change/charge after seemingly random times during the TRP/TRS test. Splitting the test up into smaller sub-tests would thus allow each sub-test to finish before the battery is depleted completely. One example of splitting the classical TRP grid with 266 fine grid points (Dq=Df=15°) into 4 separate TRP tests with 68 interpolated grid points (Dq=15°, Df=60°) each is illustrated in Figure 4 and Figure 5.

Figure 4: 266 measurement grid point for TRP

Figure 5: Split 4x68 measurement grids

# Empirical Validation of Split TRP/TRS Grid Approach

During the discussion of [1], some concerns were raised that this approach is considering coarse measurement grids and that more clarifications and/or measurement data should be provided to demonstrate the validity of this split measurement approach. In the WF, it was captured that this could be done using measurement data for a smartphone UE [2], i.e.,

Issue 1-3-3: Alternative split measurement grids method for XR to resolve battery issue Agreements: - The measurement data for smartphone will be provided next meeting.

The spreadsheet enclosed in the contribution contains measured data for a smartphone UE for LTE Band 4 with the 3D pattern visualized in Figure 6. The measurement was taken with the classical grid using Dq=Df=15° and due to system/positioner limitations, the measurement at the bottom pole, Dq=180°, was skipped.

Figure 6: 3D pattern of smartphone UE measured in LTE Band 4 with Dq=Df=15°

This measurement was then analyzed using the full TRP grid with Dq=Df=15°, see Figure 4, and the 4 split measurement grids with Dq=15° and Df=60°, see Figure 5. Additionally, split measurement grids with smaller Df were considered as well, see Figure 7 and Figure 8. Both TRP quadatures, based on sin(q) and Clenshaw-Curtis weighting, were analysed. No measurement errors due to device re-positioning were taken into account, i.e., this analysis only focuses on whether the full grid can be subdivided into multiple coarse measurement grids and whether the TRP using the full grid matches the average of the TRPs from the coarse, split grids.

=
++

Figure 7: Split 3x89 measurement grids

=+

Figure 8: Split 2x133 measurement grids

The overview of the analysis is shown in Table 1. Clearly, this data confirms that the TRP of the full measurement grid matches the average of the TRPs of the 2, 3, or 4 split measurement grids for both quadratures.

Table 1: Analysis of Measurement Data with full and split measurement grids

| Grid Type | Dq [°] | Df [°] | TRPsin [dBm] | TRPCC [dBm] |
| --- | --- | --- | --- | --- |
| Full | 15 | 15 | 18.44909 | 18.46679 |
| Split 1 | 15 | 60 | 18.46112 | 18.47876 |
| Split 2 | 15 | 60 | 18.45093 | 18.46868 |
| Split 3 | 15 | 60 | 18.44230 | 18.45999 |
| Split 4 | 15 | 60 | 18.44200 | 18.45970 |
| AVG(Split 1-4) | 15 | 15 | 18.44909 | 18.46679 |
| Split 1 | 15 | 45 | 18.44858 | 18.46626 |
| Split 2 | 15 | 45 | 18.44873 | 18.46640 |
| Split 3 | 15 | 45 | 18.44998 | 18.46770 |
| AVG(Split 1-3) | 15 | 15 | 18.44909 | 18.46679 |
| Split 1 | 15 | 30 | 18.45172 | 18.46938 |
| Split 2 | 15 | 30 | 18.44647 | 18.46419 |
| AVG(Split 1-2) | 15 | 15 | 18.44909 | 18.46679 |

Observation 1: The provided measurement data and the analysis of the full and split measurement grids confirm that the TRP of the full measurement grid matches the average of the TRPs of the 2, 3, or 4 split measurement grids

# Theoretical Validation of Split TRP/TRS Grid Approach

In the following, the split measurement approach is derived theoretically, i.e., that the average of the 4 split measurement grid TRPs equal the full measurement grid TRP.

TRP is defined as

TRP= 14πθ=0πϕ=02πEIRPθθ,ϕ+EIRPϕθ,ϕsinθdϕdθ

Equation 1

The summation form (based on sin(q) weighting is defined as

TRP≈ 14πn=0N-1m=0M-1EIRPθθn,ϕm+EIRPϕθn,ϕm∆θ∆ϕsinθn=14πn=0N-1m=0M-1EIRPθn,ϕm∆θ∆ϕsinθn=14πn=0N14m=0,4,8, …,M-4EIRPθn,ϕm4∆ϕ+m=1,5,9, …,M-3EIRPθn,ϕm4∆ϕ+m=2,6,10, …,M-2EIRPθn,ϕm4∆ϕ+m=0,4,8, …,M-1EIRPθn,ϕm4∆ϕ∆θsinθn=14πn=0N14m=0,4,8, …,M-4EIRPθn,ϕm∆ϕsplit+m=1,5,9, …,M-3EIRPθn,ϕm∆ϕsplit+m=2,6,10, …,M-2EIRPθn,ϕm∆ϕsplit+m=0,4,8, …,M-1EIRPθn,ϕm∆ϕsplit∆θsinθn=14TRPsplit,1+TRPsplit,2+TRPsplit,3+TRPsplit,4

Equation 2

where ∆ϕsplit=4∆ϕ and

TRPsplit,i=14πn=0N     m=i-1,i-1+4,i-1+8, …,M-4+i-1EIRPθn,ϕm∆ϕsplit∆θsinθn

Clearly, Equation 2 matches the equation proposed in [1], i.e.,

TRP=10log10TRP1/10+10TRP2/10+10TRP3/10+10TRP4/104

Observation 2: It can be shown theorectically that the TRP of the full measurement grid matches the average of the TRPs of the 4 split measurement grids

Similarly, the same can be demonstrated for N-split measurement grids.

Proposal 1: Confirm the validity of the split TRP/TRS grid approach

# Positioning Uncertainty

This section is taking a very high-level look at the impact of the positioning uncertainty for three separate cases

- Device has sufficient battery capacity to last for an entire TRP measurement

- Device has limited battery capacity and requires the TRP test to be performed in 4 separate test sessions, i.e., the device after battery replacement or re-charge continues where it left off

- Device has limited battery capacity and is tested with 4 separate split grids and the reduced number of grid points does not completely deplete the battery at the end of the test.

Given the uncertainty of placing the device the same way into the phantom and/or device fixtures, small angular positioning errors are made everytime the device is taken out and put back into the chamber again. This is simulated next for two antenna patterns; one less and one more directive pattern which XR antenna patterns are expected to be. Since no XR antenna pattern has been made available, the simulations here assume an n78 (3.5 GHz) BHR and FS smartphone antenna pattern from [3]. The antenna patterns, once a coordinate system transform is applied is shown in Figure 9 and Figure 10. It should be noted that this analysis does not take into account the positioning error effects of the phantom on the antenna performance; this would require much more time-intensive numerical 3D electromagnetic simulations. However, given the very fine simulated pattern data with Dq=Df=1°, small positioning errors/pattern rotations can be assessed very accurately.

Figure 9: Reference smartphone BHR pattern for the positioning MU analyses (from [3]), left with Dq=Df=1°, right with Dq=Df=15°

Figure 10: Reference smartphone FS pattern for the positioning MU analyses (from [3]), left with Dq=Df=1°, right with Dq=Df=15°

For these simulations, a random angular error placing the device into the phantom/DUT fixture in azimuth and elevation of up to ±{1°, 2°, and 2.5°} is assumed. This is visualized for the three cases investigated here in Figure 11. Clearly, each test session (each session is illustrated with a different color) can have a different angular position error.

Figure 11: Visualization of positioning errors applied to single TRP session without interruption (left), to 4 separate TRP test sessions interrupted due to limited battery capacity of DUT (center), to a split TRP measurement grid with 4 separate coarse grids (right).

A total of 10,000 different random positioning errors were simulated for these three cases and for each angular error, the respective (portion of the) antenna pattern was rotated and the TRP was recorded for each of the 10,000 simulations. The standard deviation of the respective TRPs are tabulated in Table 2.

Table 2: Impact of positioning error on standard deviation of TRP

| Cases Simulated | Standard Deviation of TRP [dB] for 10,000 different random positioning errors |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max Angular Position Error of ±1° |  | Max Angular Position Error of ±2° |  | Max Angular Position Error of ±2.5° |  |
|  | BHR Pattern | FS Pattern | BHR Pattern | FS Pattern | BHR Pattern | FS Pattern |
| Case 1: Single TRP test session | 0.0109 | 0.0193 | 0.0217 | 0.0386 | 0.0273 | 0.0482 |
| Case 2: 4 separate TRP test sessions, each test session continued where the previous sessions finished | 0.0136 | 0.0158 | 0.0273 | 0.0316 | 0.0342 | 0.0395 |
| Case 3: Split TRP measurement grid with 4 coarse measurement grids | 0.0061 | 0.0101 | 0.0124 | 0.0203 | 0.0154 | 0.0253 |

The results in Table 2 suggest that the impact of the split measurement grids (case 3) on the positioning error/MU is smaller when compared to a single test session (case 1). This is likely related to multiple randomly distributed offsets spread out over the 3D surface help to cancel out the overall positioning impact.

Observation 3: The impact of the split measurement grids on the positioning error/MU is smaller when compared to a single test session

Additionally, it can be observed that the split measurement grid (case 3) has a smaller impact on the positioning error when compared to the case where 4 consecutive tests are performed (case 2).

Observation 4: The impact of the split measurement grids on the positioning error/MU is also smaller when compared to a single test session

For multiple sessions run consecutively, the impact of the random positioning errors to each session the in general makes not much of a difference when compared to performing the TRP test in a single session.

Proposal 2: It is proposed for TRP/TRS tests performed with multiple sessions (case 2 and case 3) to have the same positioning uncertainty as defined for the full TRP/TRS tests that are performed in a single session.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: The provided measurement data and the analysis of the full and split measurement grids confirm that the TRP of the full measurement grid matches the average of the TRPs of the 2, 3, or 4 split measurement grids

Observation 2: It can be shown theorectically that the TRP of the full measurement grid matches the average of the TRPs of the 4 split measurement grids

Observation 3: The impact of the split measurement grids on the positioning error/MU is smaller when compared to a single test session

Observation 4: The impact of the split measurement grids on the positioning error/MU is also smaller when compared to a single test session

Proposal 1: Confirm the validity of the split TRP/TRS grid approach

Proposal 2: It is proposed for TRP/TRS tests performed with multiple sessions (case 2 and case 3) to have the same positioning uncertainty as defined for the full TRP/TRS tests that are performed in a single session.

# References

- R4-2504425, DraftCR for split TRP/TRS measurement grids method, Nokia, Keysight Technologies, 3GPP TSG-RAN4 Meeting #114-bis, April 2025

- R4-2504763, WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3, vivo, 3GPP TSG-RAN WG4 Meeting #114bis, April 2025

- R4-2218848, Antenna patterns for TRP and TRS measurement grid analysis, vivo, 3GPP TSG-RAN4 Meeting #105, November 2022