3GPP TSG-RAN WG4 Meeting #115	 					     							R4-2505860

Malta, Malta, 19-23 May 2025

Agenda item:			7.4.2

Source:	Apple

Title:	On UE feature list for intra-band non-collocated CA

Document for:	Discussion

# Introduction

This contribution presents feature list for CSI-RS based L1 measurement.

# Discussion

- NonCol_intraB_ENDC_NR_CA_Ph2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NonCol_intraB_ENDC_NR_CA_Ph2 | 47-1 | Support of intra-band non-collocated NR CA operation | Indicates the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table x.x.x.x/Table x.x.x.x in 38.133 and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1. And the UE also supports TDD-TDD intra-band NR-CA operation with MRTD according to Table x.x.x.x in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1. | 33-1 (RAN4 R18 feature) | yes |  | The feature cannot be supported. | Per BC | N/A | FR1 only for Band n77/n78 |  | The UE supports up to maximum 8 layer/CC in collocated condition and maximum 4 layer/CC in non-collocated condition. | Optional with capability signaling |
|  | 47-2 | Support of intra-band non-collocated EN-DC operation | Indicates the UE supports TDD-TDD inter-band non-collocated EN-DC operation with MTTD/MRTD requirements according to Table x.x.x.x/Table x.x.x.x in 38.133 and UE RF requirements for inter-band non-collocated EN-DC including 7.10B in 38.101-3. And the UE also supports TDD-TDD inter-band EN-DC operation with MRTD according to Table x.x.x.x in 38.133 and UE RF requirements for inter-band EN-DC except for 7.10B in 38.101-3. | 33-2 (RAN4 R18 feature) | yes |  | The feature cannot be supported. | Per BC | N/A | FR1 only for Band n77/n78 |  | The UE supports up to maximum 6/8 layer/CC in collocated condition and maximum 4 layer/CC in non-collocated condition. | Optional with capability signaling |