3GPP TSG-RAN WG4 Meeting #115	 					     							R4-2505865

Malta, Malta, 19-23 May 2025

Agenda item:			7.29.3.2

Source:	Apple

Title:	On open issue for CSI-RS based L1 measurement

Document for:	Discussion

# Introduction

During the past meetings, there have been extensively discussions on CSI-RS based L1 measurement for NR mobility enhancements WI [1-4]. WF in [4] captured all the conclusions and open issues for CSI-RS based L1 measurement in RAN4#114bis meeting for further discussion.

This contribution will give further analysis on the remaining open issues and present our proposals.

# Discussion

## Measurement procedure

The following is concluded from RAN4#114bis meeting.

#### Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2

Agreement:

- Whether step 2 can be skipped for neighbour cell or not based on new UE capability reporting.

- Further discuss whether step 2 can be skipped for serving cell or not.

This issue is motivated by the fact that LTM candidate cell (s) can also be current serving cell (s). We slightly prefer not to change the measurement procedure for candidate serving cell. E.g. the CSI-RS based L1 measurement is done assuming either SSB based L1 measurement is done beforehand, or CSI-RS resource is set with repetition on depending on network work configuration. But we are also open for further discussion pending input from other companies.

Proposal 1: It is proposed to keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM.

## Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement

The following options remain from RAN4#114bis meeting.

#### Issue 4-4: CSI-RS based intra-frequency/inter-frequency measurement definition

Candidate options:

- Option 1: Apple, CMCC

A measurement is defined as a CSI-RS based intra-frequency measurement provided that:

    - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and

    - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and

      - It is applied for SCS = 60KHz

    - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured).

- Option 2:  MediaTek, ZTE, Ericsson, Nokia, Qualcomm

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

    - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

    - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

      - It is applied for SCS = 60KHz

    - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

    - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

- Option 3: Huawei

If L1 CSI-RS RSRP measurement with gap is not supported, either option of intra-frequency CSI-RS based L1 RSRP measurement definition has no big difference.

If CSI-RS measurement with gap is supported, to define measurement requirements, the “CSI-RS frequency layer” concept will be touched. Then the intra-frequency definition based on center frequency is more preferred.

- Option 4: vivo

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

    - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

    - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP.

    - at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

We think either option 1 or option 2 is workable. The benefits of option 1 is that it completely followed the logic as that for legacy CSI-RS based L3 measurement and will be more friendly to classification of intra-frequency and inter-frequency. And option 1 can make it easier to introduce frequency layer definition. While in the last meeting, it was agreed not to define measurement requirement with measurement GAP and hence no need to introduce frequency layer definition. In this sense we think either option is fine and we are ok to move forward with option 2 if it is majority view.

Proposal 2: Either option 1 or option 2 can be used for intra-frequency definition.

## Measurement restriction

The following was concluded from RAN4#114 bis meeting.

#### Issue 4-8: Restriction on CSI-RS resource configuration

Candidate options:

- Option 1: Ericsson

In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms.

The proposal from Ericsson is fine for us. It is meaningful to limit the CSI-RS resources aligned with L3 measurement restriction.

Proposal 3: Agree the measurement restriction for CSI-RS based L1 measurement.

- In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms.

## Number of CSI-RS resources to be measured

In the last meeting it was agreed not to introduce frequency layers for CSI-RS based L1 measurement for neighbour cell for LTM. It is seen as intra-frequency measurement as long as the CSI-RS resources to be measured are within the UE active BWP. While due to different UE processing capability, the number of CSI-RS resources a UE could handle may be different depending on UE capability. It is proposed that Number of CSI-RS resources for L1-RSRP measurement within a slot and Number of total CSI-RS resources to be measured are based on UE capability reporting. The measurement requirement is only applicable within UE’s capability. If the number of CSI-RS resources exceeds the UE capability, it is up to UE implementation on how to choose cells/CSI-RS to measure. Since now all CSI-RS resources are intra-frequency, there is no need to limit the number of cells for requirement applicability.

Proposal 4: It is proposed that Number of CSI-RS resources for L1-RSRP measurement within a slot and Number of total CSI-RS resources to be measured are based on UE capability reporting. The measurement requirement is only applicable within UE’s capability. If the number of CSI-RS resources exceeds the UE capability, it is up to UE implementation on how to choose cells/CSI-RS to measure.

## CSI-RS measurement accuracy

#### Issue 4-13: CSI-RS accuracy requirements

Candidate options:

Option 1: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline. (ZTE, Apple, Nokia)

The following was concluded for further discussion.

It was agreed in the last meeting to reuse legacy side condition of SINR=-3dB from serving cell L1-RSRP measurement. And it was also agreed to resue the same CSI-RS resource configuration and time domain configuration restriction, we think it’s natural the lagacy accuracy requirements for serving cell L1-RSRP are also applicable for neighbour cell.

Proposal 5: CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell.

# Conclusion

This contribution proposed our consideration on open issue for CSI-RS based L1 measurement. The following proposals are concluded.

Proposal 1: It is proposed to keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM.

Proposal 2: Either option 1 or option 2 can be used for intra-frequency definition.

Proposal 3: Agree the measurement restriction for CSI-RS based L1 measurement.

- In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms.

Proposal 4: It is proposed that Number of CSI-RS resources for L1-RSRP measurement within a slot and Number of total CSI-RS resources to be measured are based on UE capability reporting. The measurement requirement is only applicable within UE’s capability. If the number of CSI-RS resources exceeds the UE capability, it is up to UE implementation on how to choose cells/CSI-RS to measure

Proposal 5: CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell.

# Reference

[1] RP-234036, New WID: NR mobility enhancements Phase 4, Apple, RAN#102

[2] R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#112bis

[3] R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, RAN4#113

[4] R4-2504049, WF on RRM requirements for NR_Mob_Ph4_Part1, RAN4#114bis