3GPP TSG-RAN WG4 Meeting #115	R4-2505866

Malta, Malta, 19th – 23th May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on CSI-RS based L1 measurement requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CSI-RS based L1-RSRP measurement requirements need to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CSI-RS based L1-RSRP measurement requirements is defined in 9.14a.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement for CSI-RS based L1-RSRP measurement requirements will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 9.14a.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 9.14a	CSI-RS based Intra-frequency L1-RSRP measurements for neighbor cell

### 9.14a.1	Introduction

### 9.14a.2	Requirements Applicability

### 9.14a.3	Measurement Reporting Requirements

### 9.14a.4	Number of CSI-RS resources, number of cells and number of SSBs

### 9.14a.5	CSI-RS based L1-RSRP measurement requirements without measurement gaps

The requirements specified in this clause are only applicable when

-	maximum RTD between cells are within CP.

-	highSpeedMeasFlag-r16 is not configured, and

-	highSpeedMeasFlagFR2-r17 is not configured, and

-	highSpeedMeasCA-Scell-r17 is not configured.

-	The SSB from the neighbor cell completely contained in the active BWP of the UE

- 	At least 48 RBs of the CSI-RS configured for measurement is confined within the active BWP of the UE.

If a neighbor cell is known according 9.14a.2, the UE shall be capable of performing L1-RSRP measurements based on the configured CSI-RS resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS_intra.

The value of TL1-RSRP_Measurement_Period_CSI-RS_intra is defined in table 9.14a.5-1 for FR1 and in table 9.14a.5-2 for FR2, where

-	For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise.

-	For periodic and semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF

- 	N=1 if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with SSB for L1-RSRP measurement; Otherwise,

- 	N=[8] if UE is capable of [skippingSSBbasedL1mesurement-R19] and the CSI-RS resources should be Type-D QCL’ed with the associated SSB for L3 measurement.

-	For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF and TCI state is provided for all resources in the resource set in the MAC CE activating the resource set.

- 	N=1 if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with SSB for L1-RSRP measurement; Otherwise,

- 	N=[8] if UE is capable of [skippingSSBbasedL1mesurement-R19] and the CSI-RS resources should be Type-D QCL’ed with the associated SSB for L3 measurement.

-	P value for CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

For a window W of duration max (TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target CSI-RS resource.

-	Psharing factor = 1, if the CSI-RS configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

-	[PL1_sharing is defined as

-	When number of neighboring cells configured with CSI-RS based L1-RSRP measurement is 1

-	PL1_sharing = 2, if any symbol of the CSI-RS from serving cell and neighbor cell are overlapping or adjacent (in time domain)

-	PL1_sharing = 1, otherwise

-	When number of neighboring cells configured with CSI-RS based L1-RSRP measurement is more than 1

-	When none of TCI state(s) of the intra-frequency neighbor cells or inter-frequency neighbor cells without gap are not in the active TCI state list

-	PL1_sharing = 3*NNeighbor_Cell, where NNeighbor_Cell is the number of neighbor cells (to measure on intra-frequency and inter-frequency without gap) whose TCI state(s) are not in the active TCI state list

-	Otherwise

-	PL1_sharing = 3*NNeighbor_Cell_in_list, where NNeighbor_Cell_in_list  is the number of neigbour cells (including intra-frequency neighbor cells and inter-frequency without gap neighbor cells) whose TCI state(s) are in the active TCI state list. No requirements are defined for any other cell(s) whose TCI state(s) is not in the active TCI state list.]

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer measurement period would be expected if the combination of CSI-RS, SMTC occasion and measurement gap configurations does not meet previous conditions.

For either an FR1 or FR2 cell, longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.14a.5-1: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_CSI-RS_intra in FR1

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TCSI-RS_NBC)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TCSI-RS_NBC is the periodicity of neighbor cell CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. NOTE 3: K = 1.5. |  |

Table 9.14a.5-2: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_CSI-RS_intra in FR2

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*[PL1_sharing]*N)*TCSI-RS_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*[PL1_sharing]*N)*max(TDRX,TCSI-RS_NBC)) |
| DRX cycle > 320 ms | ceil(M*P*[PL1_sharing]*N)*TDRX |
| NOTE 1: TCSI-RS_NBC is the periodicity of neighbor cell CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

### 9.14a.6	Measurement restriction for L1-RSRP measurement

### 9.14a.7	Scheduling availability of UE during L1-RSRP measurement