# 3GPP TSG-RAN WG4 Meeting #115	                                                	     R4-2505870

# Malta, MT, 19 – 23 May, 2025

Agenda Item:	7.27.3.1

Source: 	Apple

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion

# Introduction

RAN4 requirements for OD-SSB was widely discussed in the previous RAN4 meeting. The latest agreements can be found in [1]. There are quite many open issues as captured in [1].

In this contribution, we will discuss all open issues listed in RAN4 WF [1]. After discussion, corresponding proposals are provided.

# Discussion

## OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Sub-issue 3: OD-SSB in FMW collided with MG

Wayforward

- FFS UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

Considering the complexity of cancelling measurement gap, as being discussed in XR, we propose the following UE measurement behavior:

- If FMW covers any measurement gap occasion in time domain, UE is allowed to skip OD-SSB measurement and perform measurement on other carriers within measurement gap occasion(s).

With this, there is no impact on gap-based RRM measurement.

Proposal 1: If FMW covers any measurement gap occasion in time domain, UE is allowed to skip OD-SSB measurement and perform measurement on other carriers within measurement gap occasion(s).

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure

- Background

  - The deactivated SCell measurement requirement in FR1 is as follow.

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |

  - The OD-SSB based deactivated SCell measurement procedure is shown as follow.

Sub-issue 1: The start point of T2

Wayforward

The start point of T2 is

- Option 1:

  - The time point of OD-SSB deactivation

- Option 2:

  - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

- Option 2a:

  - The time point of OD-SSB activation

Sub-issue 2: The minimum interval(T2) between multiple FMWs

Wayforward

- Option 1: 5* measCycleSCell

- Option 1a: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity)

- Option 1b: 5*max (measCycleSCell, DRX cycles)

- Option 2: Minimum measurement period defined in existing spec.

- Option 3: The time period of known Scell condition but with OD-SSB periodicity for FR1

Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2

Wayforward

- Option 1: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

- Option 2: The UE is assumed to perform fast mode measurement without cell identification period.

- Option 3: No requirement for UE within T2.

The open issue is regarding T2 design. The first issue is about UE measurement behaviour when NW re-activates the OD-SSB within the T2. The key motivation of T2 is to avoid unnecessary UE power consumption. Therefore, it is rational to assume that UE doesn’t need to perform “fast measurement” but follow legacy deactivated SCell measurement requirement.

Regarding starting point of T2, it is proposed to have a simple design at both NW and UE side, i.e. a fixed time point rather than some dynamic time point. Existing option 4 (the time point of OD-SSB deactivation) seems a very good solution.

As for T2 duration, it is preferred to reuse existing time period in known cell definition, i.e.

-	for FR1, T2 equals to max(5*measCycleSCell,  5*DRX cycles)

-	for FR2, T2 equals to 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4

Proposal 2: When NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

Proposal 3: The starting point of T2 is the time point of OD-SSB deactivation.

Proposal 4: duration of T2:

-	for FR1, T2 equals to max(5*measCycleSCell,  5*DRX cycles)

-	for FR2, T2 equals to 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4

## OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-2-3: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: Define the requirement based on OD-SSB periodicity.

- Option 2: Not to define requirement.

Issue 1-2-4: Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies)

Agreement

RAN4 to first discuss the RAN4 impact based on RAN2 LS R2-2503015.

- Option 1: RAN4 to define requirement for different frequencies

- Option 2: RAN4 not to define requirements for different frequencies

For the case when the center frequency locations of always-on SSB and on-demand SSB are different, there are several aspects that we need to think about. For instance, does UE need to measure both AO-SSB and OD-SSB? We know that RAN1 agreed UE is not required to measure both AO-SSB and OD-SSB if always-on SSB is CD-SSB on a synchronization raster. But what if AO-SSB is not on a synchronization raster? Besides, how to interpretate serving cell MO and how handle neighbor cell measurement? Specifically, is there any impact on definition of intra/inter-frequency measurement? For instance, if UE only measures OD-SSB after OD-SSB activation, can we still follow intra-frequency measurement requirements to measure neighbor cell whose SSB is on the same frequency as AO-SSB?

Note that some of the above aspects may depend on RAN2 design. Considering we are coming to the end of core part design, we propose to deprioritize this scenario from RAN4 requirements point of view.

Proposal 5: deprioritize the case when the center frequency locations of always-on SSB and on-demand SSB are different in R19 NES from RAN4 requirement point of view.

## OD-SSB general issues

Issue 1-4-1: Additional Processing time for OD-SSB reception – scenario

Issue 1-4-2: Additional Processing time for OD-SSB reception - value

- Background

In RAN4 #113 meeting, RAN4 achieved agreements as follow.

Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

- Information

RAN1 #120-bis Agreement - Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. - If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ

Agreement

UE applies the additional processing timing only in deactivated SCell measurement.

- Note: The additional UE processing timing is not applied to SCell activation procedure.

The additional processing timing is T_processing, the value of T_processing:

- Introduce different values with UE capability: [2 or 3]ms and 5ms.

We are open for 2 or 3 ms for the UE capability. Besides, we would like to point out that the above agreements shall also apply for scenario 3B (after SCell activation is completed).

Proposal 6: RAN4 agreements on additional processing time (T_processing) also apply for scenario 3B.

Issue 1-4-6: Scheduling restriction for L1 measurement after SCell activation

Wayforward

- FFS In Case 2 L1-RSRP measurement, no scheduling restriction and measurement restriction are expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement.

The recommended WF is generally fine but not very precise. Because even though UE may skip AO-SSB for L1 measurement, the AO-SSB occasion may collide with SMTC on other carriers, which may also cause scheduling restriction.

Proposal 7: In Case 2 L1-RSRP measurement, no scheduling restriction and measurement restriction due to L1 measurement are expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement.

Issue 1-4-8: OD-SSB based Neighbor cell measurement before OD-SSB activation

Wayforward

- RAN4 to discuss whether to perform measurement on neighbour cells’ in the same frequency of deactivated SCell before OD-SSB activation for Case#1.

Based on RAN2 design, neighbor cell measurement is same as legacy. If company wants to revise this, RAN2 seems a better place to discuss.

Proposal 8: following RAN2 conclusion, neighbour cell measurement is same as legacy. If company wants to revise this, it is better to raise this in RAN2.

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for OD-SSB. After discussion, the following conclusions are provided:

Proposal 1: If FMW covers any measurement gap occasion in time domain, UE is allowed to skip OD-SSB measurement and perform measurement on other carriers within measurement gap occasion(s).

Proposal 2: When NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

Proposal 3: The starting point of T2 is the time point of OD-SSB deactivation.

Proposal 4: duration of T2:

-	for FR1, T2 equals to max(5*measCycleSCell,  5*DRX cycles)

-	for FR2, T2 equals to 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4

Proposal 5: deprioritize the case when the center frequency locations of always-on SSB and on-demand SSB are different in R19 NES from RAN4 requirement point of view.

Proposal 6: RAN4 agreements on additional processing time (T_processing) also apply for scenario 3B.

Proposal 7: In Case 2 L1-RSRP measurement, no scheduling restriction and measurement restriction due to L1 measurement are expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement.

Proposal 8: following RAN2 conclusion, neighbour cell measurement is same as legacy. If company wants to revise this, it is better to raise this in RAN2.

# Reference

- R4-2504906, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson