# 3GPP TSG-RAN WG4 Meeting #115	                                                    	        R4-2505875

# Malta, MT, 19 – 23 May, 2025

Agenda Item:	7.29.3.1

Source: 	Apple

Title:	Discussion on event triggered L1 measurement reporting

Document for:	Discussion

# Introduction

L1 event triggered measurement reporting has been widely discussed in previous RAN4 meeting. The latest agreements can be found in [1], according to which there are quite many open issues. In this contribution, we continue discussing the open issues.

# Discussion

#### Issue 2-1: applicability of L1-RSRP reporting delay

Candidate options:

- Option 1: (vivo)

  - RAN4 starts the discussion on L1-RSRP reporting delay with the follow assumptions:

    - No indicate TCI change during the evaluation period.

    - No configuration change w.r.t measurement RS during the evaluation period

    - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

- Option 2 (Apple)

  - Seems no need to capture assumptions listed under option-1 into core requirements.

We understand the concern and motivation from vivo regarding the above side conditions from UE side. However, we don’t think we should simply add those conditions since it may result in too much constraint to network implementation. For instance, if TCI switch happens during L1 measurement period, i.e. UE needs to compare the target beam with a different serving beam, UE just needs to perform another round of measurement on the new beam as minimum requirements. Therefore,  it is unnecessary to say that “No indicate TCI change during the evaluation period”. Regarding configuration change, it is similar as legacy to some extent. Configuration can only be changed via RRC. After RRC reconfiguration UE needs to follow the updated configuration to perform measurement, including both L1 and L3.

Proposal 1: seems no need to capture assumptions listed under issue 2-1 into core requirements.

#### Issue 2-2: Event triggered reporting delay for L1 reporting

Agreement:

- Reuse the framework from L3 event triggered reporting. Discuss the details further.

The fundamental issue here is to make sure that both serving cell and neighbor cell are considered in events wherein UE needs to compare measurement results of the two cells, like Event LTM3. That technically makes sense. The event can still be triggered if UE realizes target cell RSRP doesn’t not change but serving cell RSRP somehow drops. However, in real practice whenever UE is moving, channel condition for both serving and target cells are always varying. When serving cell and neighbor cannot be measured simultaneously, which is quite typical, it is challenging to tell whether the event is triggered based on measurement of serving cell or neighbor cell. In principle we agree that every time UE gets new results of either serving cell or target cell, UE shall evaluate whether event can be met.

Proposal 2: Event triggered reporting delay for L1 reporting:

- The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period on the cells corresponding to the event.

  - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

- For periodic MR: after the first report, UE follow the network configuration on the reporting interval.

#### Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category

Candidate options:

- Option 1: Yes (Xiaomi, CMCC, ZTE, CTC, vivo)

- Option 2: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

#### Issue 2-4: whether to define a separate capability for supported number of events triggered L1 measurement reporting

Candidate options:

- Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. (Xiaomi, CMCC, ZTE, E///, CTC)

- Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///)

- Option 2: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO)

We are fine to define a new UE capability for Event Triggering and Reporting Criteria for LTM. However, it is a bit unclear at current stage that if this is really necessary if RAN4 eventually need to define new capabilities regarding support of CSI-RS for L1-RSRP event triggered reporting.

Proposal 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM.

#### Issue 2-7: Impact on other LTM requirements

Agreement:

- For FR1:

  - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command,

    - Case 1: leaving condition of the event is configured and corresponding condition is not met,

      - the related cell switch delay and TCI state activation requirements are applicable

    - Case 2: leaving condition of the event is NOT configured, FFS

- For FR2: FFS

We are open to whether continue optimizing case 2 and FR2. However, we think it is necessary to add a condition that number of active TCI does not exceed that UE can support.

Proposal 5:  add the following condition on agreements under issue 2-7 in RAN4#114bis:

- Number of active TCI does not exceed that UE can support.

# Conclusion

In this contribution, we provide discussion on measurement related enhancements for LTM. After discussion, the following conclusions are provided:

Proposal 1: seems no need to capture assumptions listed under issue 2-1 into core requirements.

Proposal 2: Event triggered reporting delay for L1 reporting:

- The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period on the cells corresponding to the event.

  - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

- For periodic MR: after the first report, UE follow the network configuration on the reporting interval.

Proposal 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM.

Proposal 5:  add the following condition on agreements under issue 2-7 in RAN4#114bis:

- Number of active TCI does not exceed that UE can support.

# Reference

- R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple