# 3GPP TSG-RAN WG4 Meeting #115	                                                	     R4-2505876

Malta, MT, 19 – 23 May, 2025

Agenda Item:	7.29.3.3

Source: 	Apple

Title:	Discussion on conditional Intra-CU LTM

Document for:	Discussion

# Introduction

In RANP#105, an updated WID for NR mobility enhancement phase 4 was approved in [1]. According to the update, conditional intra-CU LTM is to be supported in this release:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

Correspondingly, RAN4 needs to specify RRM requirements to support the new procedure. Conditional LTM was preliminarily discussed in RAN4#112bis. The latest agreements can be found in the approved WF [2]. In this contribution, we further discuss the open issues.

# Discussion

## General aspects on Conditional LTM

Issue 1-1-1: The applicable scenarios for conditional LTM

RAN4#113 < Agreement > ad hoc - FFS: RAN4 requirements of CLTM which is triggered by L1 measurement (e.g. event LTM3 and LTM5) does not apply to candidate cells on deactivated SCC, including both serving cell and neighbor cells. - Note: This can be revisited if RAN4 agreed to introduce measurement requirements for L1 measurement on deactivated SCC.

< Agreement >

  - For the case which CLTM is triggered by L3 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included.

  - For the case which CLTM is triggered by L1 measurement, FFS deactivated SCell and the cells with same frequency as deactivated Scell are included.

we would like to point out that if the CLTM is triggered by L1 measurement, then the LTM candidate cell shall be an active serving SCell in MCG. If CLTM is triggered by L3 measurement, then the LTM candidate cell could be either an active serving SCell or a deactivated SCell. Because if the CLTM is triggered by L1 measurement, Tmeasure for L1 needs to be included in CLTM cell switch delay requirements. However, RAN4 doesn’t have requirements for L1 measurement on deactivated SCell.

Proposal 1: For the case which CLTM is triggered by L1 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are NOT included, given that L1 measurement requirements on deactivated SCell and neighbor cells with same frequency as deactivated SCell are not included in existing spec.

## Early synchronization and TCI state activation

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement

RAN4#114 < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.

< Way forward > FFS the following options:

Conclude this issue in the next meeting.

  - Option 1: Define requirements for L1 measurement on deactivated Scell.

  - Option 2: Define requirements for UE based TA measurement.

  - Option 3: Not define requirement for any of them.

We would like to point out that RAN2 already agreed how to handle TA in conditional LTM in November meeting:

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

Observation 1: RAN2 already agreed new design on how to handle TA for C-LTM. Therefore, C-LTM can work well without UE based TA measurement.

Observation 2: UE based TA measurement has been widely discussed in R18. Eventually RAN4 realized that it is very challenging to define corresponding RAN4 requirements.

Observation 3: UE based TA measurement is a R18 optional feature. It is not clearly in scope of RAN4 R19 requirements.

Proposal 2: Do not define RRM requirements for UE based TA measurement in R19.

## Conditional LTM delay

Issue 1-2-3: The components of conditional LTM delay

Issue 1-2-4: Tinterrupt

< Agreement >

  - Keep either TCLTM_execution or TCLTM-RRC-processing as one component for the conditional LTM delay.

    - Note: The legacy value(s) in previous releases will be reused.

  - TCLTM_execution or TCLTM-RRC-processing is not included in Tinterrupt.

  - Not include the components for TCI state activation and/or PDCCH order PRACH in C-LTM delay.

    - TEvent_DU can be updated based on RAN2 progress if needed.

  - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell.

    - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed.

      - Measurement latency for both serving and candidate cells need to be considered.

    - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO.

    - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19.

< Way forward > FFS

  - Tfirst-RS and TRS-proc are included in Tinterrupt

    - Reuse the values in Rel-18 LTM, and further discuss whether the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can be reused or not.

As minimum requirements in CLTM delay, Tfirst-RS and TRS-proc are included in Tinterrupt as baseline. It also aligns with CHO framework. RAN4 shall confirm the FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19.

Proposal 3: minimum requirements for CLTM delay:

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt

TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

Proposal 4: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19.

## Subsequent Conditional LTM

It is quite straightforward to develop Subsequent conditional LTM cell switch delay requirement based on the conclusion of conditional LTM delay. Since we already agreed the starting point in RAN4#114bis, TRRC will not be considered in subsequent conditional LTM delay. TEvent_DU needs to be updated that it is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM.

Proposal 5: TRRC will not be considered in subsequent conditional LTM delay.

Proposal 6: TEvent_DU needs to be updated: it is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM.

# Conclusion

In this contribution, we provide discussion on measurement related enhancements for LTM. After discussion, the following conclusions are provided:

Proposal 1: For the case which CLTM is triggered by L1 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are NOT included, given that L1 measurement requirements on deactivated SCell and neighbor cells with same frequency as deactivated SCell are not included in existing spec.

Observation 1: RAN2 already agreed new design on how to handle TA for C-LTM. Therefore, C-LTM can work well without UE based TA measurement.

Observation 2: UE based TA measurement has been widely discussed in R18. Eventually RAN4 realized that it is very challenging to define corresponding RAN4 requirements.

Observation 3: UE based TA measurement is a R18 optional feature. It is not clearly in scope of RAN4 R19 requirements.

Proposal 2: Do not define RRM requirements for UE based TA measurement in R19.

Proposal 3: minimum requirements for CLTM delay:

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt

TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

Proposal 4: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19.

Proposal 5: TRRC will not be considered in subsequent conditional LTM delay.

Proposal 6: TEvent_DU needs to be updated: it is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM.

# Reference

- R4-2502602, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom