# 3GPP TSG-RAN WG4 Meeting #115	                                                	     R4-2505878

Malta, MT, 19 – 23 May, 2025

Agenda Item:	7.28.3

Source: 	Apple

Title:	Discussion of RRM impact of XR for NR Phase 3

Document for:	Discussion

# Introduction

RRM impact of XR for NR Phase 3 has been widely discussed in previous RAN4 meetings. The latest agreements can be found in [1], according to which there are quite many open issues. In this contribution, we continue discussing the open issues.

# Topic #2: Measurements with gaps

## Issue 2-1: Types of measurement gaps

In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

<Agreement>:

- Gaps which shall not be cancelled

  - MUSIM

  - Positioning

- For other gap types:

  - The companies bring the issues and solutions on the RAN4#115. If the solution is not agreeable in RAN4#115, the gap type is not part of Rel-19

  - The discussion in RAN4#115 starts with Type 2 MG, then other gap types based on incoming contributions if time allows.

We understand the concern from rapporteur on completion level of core part. However, we still believe it is beneficial to support other types of gap, as long as it doesn’t introduce significant extra work. Therefore, we propose the following solutions:

Proposal 1: type-2 gap, Pre-MG, NCSG and concurrent gaps are considered this work item.

- Type-2 gap: it can be handled in the same way as type-1 gap.

- Pre-MG: it can be handled in the same way as type-1 gap when Pre-MG is ON. Otherwise when Pre-MG is OFF, no need to cancel it.

- Concurrent gaps: handle gap collision first, then apply DCI for gap cancellation. In other words, gap cancellation won’t bring the gap, which is with lower priority and dropped due to gap collision, back.

- NCSG: it can be handled in the same way as type-1 gap assuming a gap occasion is VIL1+ML+VIL2

# Topic #3: XR skipping requirements impact

## Issue 3-2: Formulas for measurement delay extension

<Way forward>:

- Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

    - where Nskip is the number of skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

- Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, ( 8 + Nskip)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

- Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, ( 8 + Nskip)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

    - FFS: CSSFinter

## Issue 3-3: Formulas for measurement delay extension with DRX

<Way forward>:

- Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - DRX cycle ≤ 320 ms

    - Max(200 ms  CSSFinter, Ceil(8 1.5)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

  - DRX cycle > 320 ms

    - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

- Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

  - DRX cycle ≤ 320 ms

    - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

  - DRX cycle > 320 ms

    - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

It is straightforward that measurement cancellation has negative impact on measurement. In order to maintain the same measurement performance, UE still needs same number of measurement samples in physical layer. Assuming during the measurement period, network cancels X measurement occasions, then the total measurement delay shall be extended by X measurement occasions. Take inter-frequency measurement with gap for example:

Note that if too many measurement occasions are cancelled on target carrier (UE cannot measure it for a long time), UE may need to adjust AGC and timing again before it can perform measurement on the carrier successfully. From UE measurement point of view, this is similar to measurement cancellation due to LBT failure. Similarly, max number of occasions shall be defined within a measurement period. If actual number of cancelled occasions exceeds the maximum value, UE shall be allowed to restart a measurement period.

Besides, another point we are considering is whether and how to guarantee mobility performance on some particular carriers, e.g. carrier(s) with high priority like PCC. One potential solution is to assign more measurement opportunities to carriers with high priority, such that legacy measurement requirements can still apply on those carriers. The impact is on other carriers, on which measurement delay may be further extended.

Proposal 2: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

Proposal 3: max number of occasions cancelled during a measurement period shall be defined.

Proposal 4: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, ( 8 + Nskip)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

Proposal 5: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

- DRX cycle ≤ 320 ms

  - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

- DRX cycle > 320 ms

  - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

# Topic #6: Additional signalling aspects

## Issue 6-1: Semi-static schemes

<Way forward>:

- Further discussion based on recommended options

  - Option 1: Do not consider semi-static solutions

  - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution.

    - Option 2a: RRC pattern-based

    - Option 2b: MAC CE bitmap for skipping indication

    - Option 2c: RRC configuration patterns with MAC CE activation

  - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

With the following agreement on UAI, it is likely that gap cancellation is a static pattern. Therefore, it is beneficial to support semi-static scheme, which can save efforts at both UE and NW side, as well as signalling overhead.

<Agreement>:

Capture the following agreement in the LS to RAN1/2. Meanwhile, it is not precluded to update the number of 1 s if any issue will be identified in the future meeting.

- The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s.

  - R ∈ {0, 20, 40, 60}%

Proposal 6: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported.

## Issue 6-2: Skipping indication on frequency domain

<Way forward>:

- Proposals

  - Proposal 1: For the gap/restriction skipping impact in frequency domain,

    - For the gap skipping, depend on whether the UE capable of per-FR gap

      - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

      - Otherwise, no need to distinguish different FR.

    - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

  - Proposal 2:

    - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs.

    - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated.

  - Proposal 3 (Huawei): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

  - Proposal 4 (vivo): RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

We support proposal 3, which is quite straightforward. We are open to further discuss other proposals.

Proposal 7: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

# Conclusion

In this contribution, we provide discussion on RRM impact of XR for NR phase 3. After discussion, the following conclusions are provided:

Proposal 1: type-2 gap, Pre-MG, NCSG and concurrent gaps are considered this work item.

- Type-2 gap: it can be handled in the same way as type-1 gap.

- Pre-MG: it can be handled in the same way as type-1 gap when Pre-MG is ON. Otherwise when Pre-MG is OFF, no need to cancel it.

- Concurrent gaps: handle gap collision first, then apply DCI for gap cancellation. In other words, gap cancellation won’t bring the gap, which is with lower priority and dropped due to gap collision, back.

- NCSG: it can be handled in the same way as type-1 gap assuming a gap occasion is VIL1+ML+VIL2

Proposal 2: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

Proposal 3: max number of occasions cancelled during a measurement period shall be defined.

Proposal 4: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, ( 8 + Nskip)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

Proposal 5: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

- DRX cycle ≤ 320 ms

  - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

- DRX cycle > 320 ms

  - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

Proposal 6: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported.

Proposal 7: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

# Reference

- R4-2504911, WF on RRM requirements for XR_Ph3, Nokia