# 3GPP TSG-RAN WG4 Meeting #115	                                                	     R4-2505880

Malta, MT, 19 – 23 May, 2025

Agenda Item:	9.3

Source: 	Apple

Title:	Reply LS on Collision between MUSIM Gap and Measurement Gap

Document for:	Discussion

# Introduction

RAN4 received an LS [1] from RAN2 on Collision between MUSIM Gap and Measurement Gap, in which RAN2 ask RAN4 to provide answer to their question. In this contribution, we provide our analysis and draft reply to RAN2 on this issue.

# Discussion

The issue is duplicated here for information:

On the collision between the MUSIM Gap and Measurement Gap (for the UE supporting musim-GapPriorityPreference capability) in R18, RAN2 identified that the network can configure the MUSIM gap without priority and the measurement Gap with priority together and vice versa. In addition, it is also supported from RAN2 specification that the network can configure the MUSIM gap without priority and the measurement Gap without priority together. RAN2 respectfully ask RAN4 on whether UE requirement is needed for the above collision cases.

Collision handling between MUSIM gaps and measurement gap is specified in clause 9.1.10.5:

Collisions between MUSIM gaps and measurement gaps configured via GapConfig-r17 with assigned priority but without preConfigInd-r17 or ncsgInd-r17 are resolved based on their assigned priorities: - If ‘keep solution’ in clause 9.1.10.3 is not configured, collisions are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority. For each collision, the occasion of the MUSIM gap or measurement gap with higher priority shall be kept and the occasion of the MUSIM gap or measurement gap with lower priority shall be dropped. Any collisions between MUSIM gaps shall be addressed as specified in clauses 9.1.10.3 and 9.1.10.4. - Otherwise if ‘keep solution’ in clause 9.1.10.3 is configured, keep solution is used for the remaining collided and non-dropped MUSIM gaps, after resolving the collisions between measurement gaps and MUSIM gaps based on their priorities. Collisions between MUSIM gaps and measurement gaps configured via GapConfig or configured via GapConfig-r17 without assigned priority are handled based on MGRP of the colliding gaps: - If ‘keep solution’ in clause 9.1.10.3 is not configured, collisions are resolved sequentially in order of decreasing MGRP, starting with the gap that has the longest MGRP. For each collision, the occasion of the MUSIM gap or measurement gap with longer MGRP shall be kept and the occasion of the MUSIM gap or measurement gap with shorter MGRP shall be dropped. If the colliding MUSIM gap and measurement gap have the same MGRP, the requirements in clause 9 shall not apply to these gaps. Any collisions between MUSIM gaps shall be addressed as specified in clauses 9.1.10.3 and 9.1.10.4. - Otherwise if ‘keep solution’ in clause 9.1.10.3 is configured, keep solution is used for the remaining collided and non-dropped MUSIM gaps, after resolving the collisions between measurement gaps and MUSIM gaps based on their MGRP following the above mentioned collision rule.

RAN4 had discussion on collision between MUSIM gap and type-1 gap. Corresponding requirements are highlighted with yellow above. However, RAN4 didn’t discuss the scenario wherein priority of MUSIM gap is not configured, i.e. the scenario mentioned by RAN2:

- In addition, it is also supported from RAN2 specification that the network can configure the MUSIM gap without priority and the measurement Gap without priority together.

One simple solution is to extend existing collision handling based on MGRP to cover this scenario as well. Therefore, we propose:

Proposal 1: for the scenario where network configures the MUSIM gap without priority and the measurement Gap without priority together, apply the same collision handling based on MGRP.

We also provide a draft LS to RAN2 in Annex.

# Conclusion

In this contribution, we provide discussion on RAN2 LS on Collision between MUSIM Gap and Measurement Gap. After discussion the following conclusions are provided:

Proposal 1: for the scenario where network configures the MUSIM gap without priority and the measurement Gap without priority together, apply the same collision handling based on MGRP.

# Reference

- R2-2503163, LS on Collision between MUSIM Gap and Measurement Gap, ZTE

# Annex: draft LS to RAN2

3GPP TSG-RAN WG4 Meeting #115	                                                	     R4-25xxxxx

Malta, MT, 19 – 23 May, 2025

Title:	Reply LS on collision between MUSIM Gap and Measurement Gap

Response to:	R2-2503163

Release:	Rel-18

Work Item:	NR_DualTxRx_MUSIM-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Qiming Li

Li_qiming@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN4 thanks RAN2 for the LS on collision between MUSIM Gap and Measurement Gap. RAN4 discussed the issue and reached the following agreement:

- For the scenario where network configures the MUSIM gap without priority and the measurement Gap without priority together, apply the same collision handling based on MGRP:

  - If ‘keep solution’ in TS38.133 clause 9.1.10.3 is not configured, collisions are resolved sequentially in order of decreasing MGRP, starting with the gap that has the longest MGRP. For each collision, the occasion of the MUSIM gap or measurement gap with longer MGRP shall be kept and the occasion of the MUSIM gap or measurement gap with shorter MGRP shall be dropped. If the colliding MUSIM gap and measurement gap have the same MGRP, the requirements in TS38.133 clause 9 shall not apply to these gaps. Any collisions between MUSIM gaps shall be addressed as specified in TS38.133 clauses 9.1.10.3 and 9.1.10.4.

  - Otherwise if ‘keep solution’ in TS38.133 clause 9.1.10.3 is configured, keep solution is used for the remaining collided and non-dropped MUSIM gaps, after resolving the collisions between measurement gaps and MUSIM gaps based on their MGRP following the above mentioned collision rule.

# 2	Actions

To RAN2

ACTION: 	RAN4 kindly asks RAN2 to take the above understanding 2 into consideration in RAN2 design.

# 3	Dates of next TSG RAN WG 2 meetings

TSG RAN WG4 Meeting #116			25 – 29 August 2025                  		Bangalore, India

TSG RAN WG4 Meeting #116bis		13 – 17 October 2025                  	        Prague, CZ