3GPP TSG-RAN4 Meeting#115	R4-2505886

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.5.4

Source:	Apple

Title:	On UE RF requirements for fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

At the last RAN4 meeting, a WF was agreed that captures the agreements and open issues [2]. In this contribution, we share our views on the open issues:

- Triggering condition to enable ”One Rx RF chain”

- Discussion on applicability for PCell and SCell

# 2	Discussion

Requirements for UE supporting fragmented carriers with shared RF chains

In the earlier WF, the point on how to specify requirements for UE supporting fragmented carriers with shared RF chains is in brackets, as shown below

- [Adjusted requirements for fragmented carriers, if any, shall be separated from the existing requirements]

It is important to have a common understanding on it. As is well understood, the current RF requirements for intra-band non-contiguous CA were derived based on a partially shared RF chain architecture captured in TR 36.823. Given the blocker in the gap, it is evident that a shared RF chain architecture cannot meet such requirements. That is the reason why the SI scope explicitly contains the bullet “Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking.”

To be clear, whatever the outcome of the SI is, the existing requirements should still apply to UEs using partially shared RF chain architecture to support fragmented carriers (or intra-band non-contiguous CA). With that, we think it is reasonable to say the adjusted requirements for UE supporting fragmented carriers with shared RF chains should be separated from existing requirement. Of course, there should be a UE capability or other means to indicate such UEs.

Proposal 1: It is proposed to agree that adjusted requirements for UEs supporting fragmented carriers with common RF chain architecture, if any, shall be separated from the existing requirements. A TP is needed to capture this agreement in the TR.

Triggering condition to enable ”One RF Rx chain”

There was good progress made at the last meeting as there are only two options to be further discussed, as captured in the WF at the last meeting [2]:

#### Issue 2-3: Triggering condition to enable ”One Rx RF chain” RAN4 agree there are two aspects regarding the triggering condition enable/disable the one Rx RF chain mode: - Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed - Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified - Note: detailed conditions to enable/disable can be further discussed and decided separately

It is understood that Option 1 has no standards impact since it requires no new or updated reporting/indication from the UE. However, if we consider the UE architecture switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA, some questions arise.

- Question 1: When the UE operates with 2 Rx RF chains, the blocker impact is limited or minimal as long as the blocker is not much higher than that supported in the current ACS/blocking requirement. Also, there is no measurement configuration from the network to allow the UE to measurement the blocker power. Therefore, it is unclear how the network can make the decision to allow the UE to switching from 2 Rx RF chains to 1 Rx RF chain. If such switching is transparent to the network, the network cannot configure more CCs to the UE.

- Question 2: When the UE operates with 1 Rx RF chain, relying on existing channel quality reporting and procedures may lead to ambiguity at the network. The network may not know the channel quality degradation is due to blocker impact or natural channel quality worsening such as channel fading. For instance, when two non-contiguous CCs, say CC1 and CC2, experience performance degradation, will the network be able to distinguish the two cases:

  - Case 1: Performance degradation is caused by the blocker when 1 Rx RF chain is used, so the proper action from the network is to drop either CC1 or CC2.

  - Case 2: Performance degradation is caused by channel fading, so the proper action from the network is to wait for RLF to be declared and RRC connection re-establishment procedure should follow.

- Question 3: Is there any UE proprietary solution to address the issues raised in Questions 1 and 2?

By contrast, Option 2 is based on UE indication and therefore both UE and network have clear understanding whether 1 Rx RF chain or 2 Rx RF chains should be used considering the blocker and self-interference impact. More details can be found in [3]. We also acknowledge that Option 2 has standards impact as signaling to support UE indication needs to be specified.

Proposal 2: Regarding the triggering condition enable/disable the one Rx RF chain mode, it is proposed to:

- Further study all the details of Option 1 related to the questions raised above.

- Adopt Option 2 as a viable option.

Discussion on applicability for PCell and SCell

There was discussion on whether any performance degradation can be allowed for PCell at the last meeting. The following is captured in the WF at the last meeting [2]:

#### Issue 2-4: Discussion on applicability for PCell and SCell A more detailed proposal based on the tentative agreement in online discussion: - To address the potential impacts on the DL performance for PCell, RAN4 agrees to further discuss the following aspects: - To limit the use of this feature to CA configurations where the impact is limited - In the CA configurations where there is limited impact, how to further mitigate the impact by means including restricting the PCC and SCC frequency location, or restricting UE locations (e.g., the feature is not enabled when the UE is located at cell edge where wanted signal level is close to receiver sensitivity)

We agree that it is possible to limit the impact for some CA configurations. For example, TDD CA configuration or FDD CA configurations that have large duplex gap would experience no or little self-interference issue. Furthermore, in case of self-interference for FDD CA configurations, PCC SCC Swapping can be used, as copied from the TR [4]:

In cases where the blocker power is not high, the overall impact of using 1 Rx RF chain would become minimal.

In the CA configurations where there is limited impact, there is also ways to mitigate the impact. For example, the network has good knowledge about whether the UE is located at the cell edge using UE L3 RSRP measurement report. Also, the UE CQI reporting can also reflect whether the UE is at the cell edge or not. In the RF analysis, the worse case scenario where wanted signal level is close to receiver sensitivity is considered. Therefore, if the UE is not located at the cell edge, the impact of blocker or self-interference would become manageable. As a result, the feature of 1 Rx RF chain can be enabled as long as the benefit of supporting one more CC outweigh the performance degradation due to the use of 1 Rx RF chain supporting two non-contiguous CCs.

Observation 1: There are ways to limit the potential impacts on the DL performance for PCell, e.g., 1) TDD CA configuration or FDD CA configurations that have large duplex gap would experience no or little self-interference issue. 2) In case of self-interference for FDD CA configurations, PCC SCC Swapping can be used. 3) The use of 1 Rx RF chain can be enabled when the UE is not located at cell edge.

Proposal 3: It is proposed to capture all the mitigation methods in the TR to conclude the SI. How and what mitigation method to use can be further discussed in the follow-on WI.

# 3	Conclusions

In this contribution, we make the following proposals and observations.

Proposal 1: It is proposed to agree that adjusted requirements for UEs supporting fragmented carriers with common RF chain architecture, if any, shall be separated from the existing requirements. A TP is needed to capture this agreement in the TR.

Proposal 2: Regarding the triggering condition enable/disable the one Rx RF chain mode, it is proposed to:

- Further study all the details of Option 1 related to the questions raised above.

- Adopt Option 2 as a viable option.

Observation 1: There are ways to limit the potential impacts on the DL performance for PCell, e.g., 1) TDD CA configuration or FDD CA configurations that have large duplex gap would experience no or little self-interference issue. 2) In case of self-interference for FDD CA configurations, PCC SCC Swapping can be used. 3) The use of 1 Rx RF chain can be enabled when the UE is not located at cell edge.

Proposal 3: It is proposed to capture all the mitigation methods in the TR to conclude the SI. How and what mitigation method to use can be further discussed in the follow-on WI.

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

R4-2505109, "WF on NR FR1 DL fragmented carriers study," Moderator (Mediatek Inc.), Apple, Qualcomm

R4-2504451, "On UE RF requirements for fragmented carriers," Apple

TR 38.755