3GPP TSG-RAN WG4 Meeting #115						R4-2505905

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.17.3

Source:	Apple

Title:	Simulation results for ICI and MU-MIMO with 8RX

Release:	Rel-19

Document for:	Information

# 1. Introduction

In RAN4#114, 114bis UE demodulation enhancements in Rel-19 was discussed and way forwards [1] [2] were agreed. The simulation parameters for evaluation of performance in ICI and MU-MIMO with 8RX were agreed. In this contribution present simulation results for defining requirements.

# 2. Simulation Results

## PDSCH requirements with inter cell interference

For evaluation of performance in inter-cell interference scenario, the same interference scenarios and interference model was used.

Homogeneous network

2 Interference cells

INR1: 7.77 dB

INR2: 2.29 dB

Heterogeneous network

1 Interference cell

INR1: 7.58 dB

Table 1 presents results with baseline receiver for FDD mode.

Table 1: Simulation Results for PDSCH in ICI with baseline receiver

| Duplex Mode and SCS | CHBW | Scenario | MIMO | Rank (target UE) | MCS (target UE) | INR1 | INR2 | Channel Model /Corr | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Alignment | Impairment |
| FDD 15KHz SCS | 10 MHz | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | TDLA30-10 ULA Low | 8.4 | 10.7 |
|  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  | 7.2 | 9.3 |
| TDD 30KHz SCS | 40 MHz | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 |  | 8.7 | 11.0 |
|  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  | 7.5 | 9.5 |

## PDSCH requirements with intra cell inter user interference

For the simulation assumptions agreed in [1] Table 2 below captures the simulation results from initial evaluation with baseline receiver.

Table 2: Simulation results for PDSCH in intra-cell inter user interference with baseline receiver

| Duplex Mode SCS CHBW | MIMO | Rank for target + Co-UE | MCS (target UE) | Precoder selection | Channel Model / Ant Corr | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Alignment | Impairment |
| FDD 15KHz SCS 10MHz BW | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 ULA Low | 9.5 | 11.8 |
| TDD 30KHz SCS 40 MHz BW |  |  |  |  |  | 9.4 | 11.7 |

# 3. Conclusion

In this paper, we provide our simulation results for defining UE demodulation requirements with 8RX.

# Reference

R4-2502275, “Way Forward for [114][324] NR_demod_Ph5_Part2_UE”, Nokia.

R4-2504694, “Way Forward for [114bis][322] NR_demod_Ph5_Part2_UE”, Nokia.