# 3GPP TSG-RAN WG4 Meeting #115	R4-2505927

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.7.4

Source:	Apple

Title:	On RRM requirement for R19 LB CA

Document for:	Discussion

# Introduction

The WF on RRM requirements for NR_LBCA_Sw [1] have been agreed in RAN4#114bis, and the open issues have been summarized in the topic summary for [114bis][204] NR_LBCA_Sw [2].

In this contribution, we continue the discussion for RRM requirement of LB CA via switching.

# Assumptions for R19 LB CA via switching

Issue 1-1-1: SSB assumptions for R19 LB CA via switching

- [moderator]: Decide between option 1 and option 2. The SSB-less operation here does not mean SSB is not transmitted on SDL SCell, and that means UE may use the same operation as for SSB-less case, e.g., skip L3 measurement on SSB-less SCell. - Option 1 (OPPO, LGE, MTK, Nokia,Xiaomi, QC): RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching. - Option 2 (Apple): RAN4 to use SSB-less based operation as baseline for Rel-19 low band CA via switching.

Technically, the WID clarified the scenario for this LB CA, as following. Considering the synchronization and collocated condition between FDD PCC and SDL SCC, we assume the coarse timing between these two CCs can be shared within TAE.

- Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers

Additionally, CA_n5A-n29A is considered in this WI, and these two band are very close to each other on frequency domain, which means the channel conditions are also similar.

| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| --- | --- | --- | --- |
| n29 | N/A | 717 MHz – 728 MHz | SDL |

However, in last meeting, companies commented that the TRS or A-TRS based RRM operation on SCell would be an optional UE capability from previous release, and baseline of SCell operation shall still be based SSB rather than TRS or A-TRS. In our view, with the above explained similar channel condition and synchronization status among FDD PCell and SDL SCell, the sample number needed for SDL SCell activation can be reduced but that does not necessarily mean TRS/A-TRS shall be assumed. Thus, we agree that RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching, rather than using TRS/A-TRS based SDL SCell operation.

Proposal 1: RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching, rather than using TRS/A-TRS based SDL SCell operation.

Issue 1-1-2: MRTD assumptions for R19 LB CA via switching

- FFS: - Option 1 (OPPO, vivo, QC, ZTE, Nokia, Xiaomi, CATT, Samsung, HW, MTK, LGE): The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. - Option 3 (Ericsson): MRTD = CP for in LB-LB CA with switching.

As the deployment for this LB CA is co-located and synchronized network deployment for both carriers, the max RTD shall be same as the TAE at network, and therefore 3us shall be assumed here.

Proposal 2: The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

# Switching delay or interruption requirement

Issue 1-2-1A: Necessity of interruption requirement in RRM between FDD CC and SDL CC

- Option 1 (Xiaomi, CATT, Apple, HW, Ericsson, ZTE): RAN4 RRM to define interruption requirement for carrier switching between FDD PCell and SDL SCell. - Option 1a (HW): Interruption requirement for carrier switching between FDD CC and SDL CC are to be defined, - Whether interruption is present on PCell depends on RAN1 decision on the location of switch gap. - The interruptions on other activated SCell are present. - Option 2 (OPPO, vivo, Nokia): RAN4 to not define interruption or scheduling restriction requirement for carrier switching between FDD PCell and SDL SCell. - Option 2a (Nokia): If a third carrier is configured without switching pattern, no interruption is allowed on the third carrier due to LBCA-sw operation on PCell and SDL SCell. - Option 3 (LGE): The interruption requirements should be discussed after the RF conclusion on time mask issue is reached discussion.

In RAN4#114 RF session, the agreements were captured in WF[3] as following,

Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us - RAN4 to further discuss 13us - LS will capture the currently confirmed values - It is not precluded to send another LS with more values in future meetings

In last RAN1 #120bis meeting, some agreements have been achieved as followings:

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

Since the switching gap design in RAN1 will consider both the switching period and the UL TA, the network will schedule the UE on those DL symbols that are not overlapped with the switching gap. The possible requirement needed in RAN4 could be the scheduling restriction rather than an interruption. Additionally, RAN1 used the following diagrams to clarify their design of switching gap,

SDL to FDD switch FDD to SDL switch

For SDL to FDD switching, the DL symbols on SDL FDD SCC have scheduling restriction if they are colliding with the switching gap, while the UL symbols of FDD PCC shall not be impact as network will consider the TA and corresponding uncertainty of TA. However, for FDD to SDL switching, the switching gap is still under discussing in RAN1. Thus, we propose that,

Proposal 3: For SDL to FDD switching, RAN4 to define scheduling restriction requirement for DL symbols reception on SDL FDD.

Proposal 4: For FDD to SDL switching, RAN4 to wait conclusion of switching gap from RAN1.

Issue 1-2-2: Scheduling restriction requirement (if needed) for carrier switching between FDD CC and SDL CC

- Recommended WF - Discuss following points: - Interruption includes switching period only OR switching period plus time duration when UE works on another carrier? - Slot level or symbol level interruption requirement for carrier switching in R19 LB CA via switching - option1: symbol level (Xiaomi, Ericsson, ZTE, Samsung, MTK) - option 2: slot level (Apple) - If symbol level interruption requirement is chosen, check if Xiaomi proposal is agreeable, - Interruption length of LB(FDD)-LB(SDL) via switching = ceil((switching period + TA adjustment uncertainty + MRTD - CP length)/symbol duration)+1 - The interruption location and application proposals can be FFS. - If slot level interruption requirement is chosen, check if Apple proposal is agreeable, - The interruption requirement for carrier switching in R19 LB CA is TswitchingSlot-length-victim+1, where Tswitching is the switching time concluded in RF session.

As discussed above for proposal 3 and 4, RAN1 agreed that,

- for RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW

- switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch

The switching gap can be part of one slot as in the diagram above for SDL to FDD switch, and therefore symbol level scheduling restriction can be considered. Since NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed), so the scheduling restriction can be defined for the reception of those DL symbols that are overlapping with the switching gap.

Proposal 5: RAN4 to define symbol level scheduling restriction requirement for carrier switching in R19 LB CA.

Proposal 6: For SDL to FDD switching, the scheduling restriction applies on the reception of those SDL SCC DL symbols that are overlapping with the switching gap.

# SDL SCC related RRM requirement

Issue 1-3-1: Activated SDL SCell L3 measurement and corresponding interruption

Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. Recommended WF - Discuss following points: - Whether to define the requirement for activated SDL SCell L3 measurement or not? - Option 1: No (QC, vivo) - Option 2: Yes for SSB based operation (OPPO, Apple, LGE, HW, Ericsson, Samsung, ZTE, MTK, Nokia) - If Yes for above, followings are FFS - Whether to reuse existing corresponding requirement for activated SDL SCC measurement - The scaling factor of CSSF and Kp. - Whether and how such measurement shall consider switching pattern, e.g., only measure SDL SCC during the period of SDL SCell ON. - Whether and how to define the corresponding interruption requirement for this measurement.

Like discussed in issue 1-1-1, considering the synchronization and collocated condition between FDD PCC and SDL SCC, we assume the coarse timing between these two CCs can be shared within TAE. Additionally, CA_n5A-n29A is considered in this WI, and these two band are very close to each other on frequency domain, which means the channel conditions are also similar. To be more specific, our understanding is: for this particular CA case, the power imbalance at UE between PCell and SDL SCell can be in a limited range, e.g., 12dB in NES case; and the RTD between PCell and SDL SCell will be up to 3us, and therefore with such side condition, the L3 measurement may be skipped on SDL SCell, or no requirement is needed for L3 measurement on activated SDL SCell. However, we also understand the comments from companies that since NES or SSB-less operation is optional UE capability and network will always transmit the SSB on SDL SCell, it’s no problem to still require UE perform L3 measurement on SDL SCell as a legacy behavior. Thus, both options work for us, i.e.,

      - Option 1: RAN4 to define the requirement for activated SDL SCell L3 measurement.

      - Option 2: RAN4 can skip activated SDL SCell L3 measurement requirement with considering the following certain conditions

        - Power difference at UE between PCell and SDL SCell is smaller than [12dB]

        - RTD<=3us

But, in order to move forward for the WI, we can compromise to option 1.

Proposal 7: RAN4 to define the requirement for activated SDL SCell L3 measurement.

If SSB-based SCell operation is assumed for SDL SCell measurement, UE needs to measure SSB for L3 on SDL SCell based on SMTC configuration, and the current requirement is defined as:

The current measurement interval is specified based on SMTC periodicity or max(SMTC periodicity, DRX cycle); however, if some of the SMTC occasions are outside the SDL SCC reception duration of switching pattern, UE can only perform the SDL SCell measurement on the overlapped occasions between SMTC window and SDL SCell reception duration indicated by switching pattern. Thus, the legacy measurement interruption shall be revised accordingly.

Additionally, the switching pattern and SMCT configuration are both controlled by network, so we assume network will configure the switching pattern reasonably to make sure UE can at least have one available measurement opportunity in each SDL SCell reception duration.

Proposal 8: For activated SDL SCell measurement, UE can only perform the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern.

Depending on the pattern design from RAN1, if the SDL SCell reception duration can be longer than SMTC periodicity, RAN4 also needs to discuss whether UE shall follow the SMTC periodicity to do the SCell measurement inside SDL SCell reception duration, or UE only needs to measure SCell one time per periodicity of switching pattern.

Issue 1-3-2: Deactivated SDL SCell measurement and corresponding interruption

Agreement: - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement. Recommended WF - Discuss following points: - Whether to define the requirement for deactivated SDL SCell L3 measurement or not? - Option 1: No (QC) - Option 2: Yes for SSB based operation (OPPO, CATT, Apple, LGE, HW, Ericsson, vivo, MTK, Nokia) - If Yes for above, followings are FFS - Whether and how such measurement shall consider switching pattern and/or meaCycleSCell, e.g., only measure SDL SCC during the period of SDL SCell ON. - Whether to reuse existing corresponding requirement for deactivated SDL SCC measurement - Option 1: Yes (Apple, MTK) - Option 2: No - Whether and how to use MG for deactivated SDL SCC measurement (LGE, Nokia). - Whether and how to define the corresponding interruption requirement for this measurement. - Option1 (Apple, HW): If SSB-based SCell operation is assumed for SDL SCell measurement, for deactivated SDL SCell measurement, interruptions on PCell or activated SCell(s) are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell. - Option 2 (Ericsson): interruption on PCell or activated SCell(s) are allowed with up to X % probability of missed ACK/NACK when measCycleSCell is below threshold 1 and Y% otherwise. Y can be 0.

Same as for Issue 1-3-1, our view on the necessity of the deactivated SDL SCell L3 measurement is as following:

Proposal 9: RAN4 to define the requirement for deactivated SDL SCell L3 measurement.

In current requirement, the deactivated SCell measurement follows the measCycleSCell as shown in the following table,

The switching pattern shall be applied only when the target SDL SCell is activated, and therefore the deactivated SCell measurement requirement shall not be impacted by the switching pattern. The existing deactivated SCell measurement requirement can be reused for deactivated SDL SCell in LB CA via switching.

Proposal 10: The existing deactivated SCell measurement requirement can be reused for deactivated SDL SCell in LB CA via switching.

The current deactivated SCell measurement consider the interruption for RF tuning/retuning was specified below,

If measCycleSCell is smaller than 640ms, currently UE is required to stay on deactivated SCC after SCell addition(no interruption is allowed). However, in R19 new LB CA scenario, since UE cannot keep the connection with PCell when it stays on the SDL SCC for measurement, it would be necessary to allow UE switch back to PCC for traffic/control reception/transmission after deactivated SCell measurement regardless of the measCycleSCell.

Proposal 11: For deactivated SDL SCell measurement, interruptions on PCell or activated SCell(s) are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell.

Issue 1-3-3: SDL SCell activation/deactivation

Agreement: - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - RAN4 will also discuss potential scheduling restriction/interruption on PCell during the SCell activation procedure for receiving the above mentioned RS occasions for the SCell activation (in the LS) Recommended WF - Discuss following points: - Side condition for SDL SCell activation requirement - RTD between the FDD PCell and SDL SCell - RTD condition is based on issue 1-1-2 - reception power difference between the FDD PCell and SDL SCell - EPRE difference condition is reused from FR1 inter-band SSB-less SCell activation for Rel-18 NES, e.g., 12dB - The SDL SCell activation consider SSB-less operation or SSB based operation or both? - Option 1: SSB-less only (Apple, Samsung- for UE supporting SSB-less SCell operation) - Option 1a: such SSB-less operation is regardless of whether or not UE supporting SSB-less SCell feature. - Option 2: SSB based only - Option 3: both (HW, vivo) - Can existing SCell activation requirement be reused or not - Option 1: Yes (Apple, Samsung, MTK, Nokia) - Option 2: need a new requirement.(Xiaomi, CATT) - Interruption and scheduling restriction for SDL SCell activation: need FFS - Known/unknown condition for SDL Cell activation and other details of the SDL SCell activation(if needed): need FFS - The other scopes for SDL SCell activation requirements, e.g., in proposal 9: need FFS

Regarding the side condition for the SDL SCell activation requirement, based on the condition in WID: co-located and synchronized network deployment for both carriers and both carriers are in a single TAG, we think the RTD could follow proposal 2 discussion, which is 3us. However, for the power different at UE between FDD PCell and SDL SCell, it’s dependent on how much difference for the Tx power from gNB and we don’t think it’s very necessary to limit it in RAN4 minimum requirement.

On the other hand, the current unknown FR1 SCell activation delay requirement already has such side condition for RTD(<=260ns), power difference(<=6dB) and ssb-PositionInBurst/SMTC-offset for intra-band FR1 CA case, as shown below,

As observed above, if all these side conditions are met, then UE can save 1Trs for synchronization. For SDL SCell activation, we don’t think RAN4 can assume ssb-PositionInBurst/SMTC-offset is same between FDD PCell and SDL SCell, i.e., even though RTD between the FDD PCell and SDL SCell <=3us, UE cannot save the cell synchronization time for SDL SCell activation compared with legacy FR1 intra-band SCell activation case. For known SDL SCell activation, the small RTD between PCell and SDL SCell cannot help anything to save the activation delay in our view, i.e., AGC settling and fine timing tracking is always needed. Thus, the legacy known/unknown SCell activation requirement can be applied for SDL SCell activation. Moreover, we didn’t see any difference about known/unknown condition between SDL SCell and legacy SCell.

In summary, we think the known/unknown condition for legacy SCell activation requirement can be reused for SDL SCell activation, and the legacy FR1 inter-band SCell activation requirement can be reused for SDL SCell activation.

Proposal 12: No need to define side condition of RTD and power difference for SDL SCell activation requirement.

As in proposal 1, the SSB based operation shall be used as baseline, and then the SSB based SCell activation shall also be considered as baseline, and how many SSB occasions UE needs to complete the SCell activation shall be further discussed based on the side condition.

Proposal 13: SSB based SDL SCell activation shall be considered as baseline.

Proposal 14: the legacy FR1 inter-band SCell activation requirement can be reused to SDL SCell activation in LB CA via switching.

The existing interruption window design in TS38.133 for SCell activation RRM operation is as following,

However, during the SDL SCell activation procedure, UE may need to switching between PCell and SCell periodically. Like in the following figure, carrier switch between SDL SCell and PCell during SCell activation procedure can follow the SSB periodicity or SMTC periodicity used for SCell activation. The interruption length can be defined as (switching duration for FDD-to-SDL) + SSB/SMTC duration + (switching time for FDD-to-SDL). To be simpler, we think we can reuse the legacy intra-band FR1 SCell activation interruption requirement for this inter-band SDL SCell activation, since PCell cannot be schedule during the SSB or SMTC measurement occasions.

Figure 1. the CC switching and interruption for SCell activation

Proposal 15: the interruption pattern due to CC switching in SDL SCell activation shall follow SSB periodicity or SMTC periodicity used for SCell activation.

Proposal 16: the interruption length for each CC switching in SDL SCell activation can be defined as {(switching duration for FDD-to-SDL) + SSB/SMTC duration + (switching time for FDD-to-SDL)} or reuse same interruption requirement from legacy FR1 intra-band SCell activation.

Another thing to consider is about CQI and L1-RSRP reporting during the SDL SCell activation. In legacy SCell activation requirement, UE is required to report OOR and lowest L1-RSRP right after the MAC CE decoding.

However, such CQI or L1-RSRP reporting is on PCell, and when UE needs to report CQI/L1-RSRP on PCell, UE cannot stay on SCell for DL synchronization if the SSB or SMTC is colliding with the CQI/L1-RSRP reporting occasion. Like in figure 2,

Figure 2. the CQI/L1-RSRP on PCell colliding with the SSB/SMTC for SDL SCell activation

It’s unclear how UE can handle the CQI/L1-RSRP reporting during the SCell activation procedure, and our view is to prioritize the SSB/SMTC measurement over the OOR or lowest L1-RSRP reporting to complete the SDL SCell activation as soon as possible. But, since it was not discussed in RAN4 before, we would like to trigger this issue and FFS on the solutions.

Proposal 17: RAN4 to discuss the UE behavior if the CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation.

Issue 1-3-4: BFD/CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for SDL SCell

- Discuss following points: - Whether to define requirement for BFD/CBD for SDL SCells in LB CA via switching - Option1 (QC, Apple): No, RAN4 to not define BFD and CBD requirements for SDL SCells in LB CA via switching - Option 2 (MTK): Yes, reuse the existing BFD/CBD requirement for R19 LB CA - Option 3(LGE, CATT, Ericsson): Yes, FFS on detailed requirements. - Whether to define requirement for PL-RS switching delay for SDL SCells in LB CA via switching - Option1 (Apple): No, RAN4 to not define PL-RS switch delay requirements for SDL SCells in LB CA via switching - Option 1a(vivo): No if assuming SSBs on FDD PCell and SDL SCell are QCL-ed - Option 2 (MTK): Yes, reuse the existing PL-RS switch delay requirement for R19 LB CA - Option 3: Yes, FFS on detailed requirements. - Whether to define requirement for TCI switching delay for SDL SCells in LB CA via switching - Option 1 (Apple): existing TCI switching requirement can be used for SDL SCell in R19 LB CA via switching, provided that PCell SSB is configured as the QCL source of SDL SCell channel(s); otherwise, no requirement applies. - Option 2 (MTK): reuse the existing TCI switching delay requirement for R19 LB CA. - Option 3 (Ericsson): consider to change the requirement, as in P7. - Option 4 (vivo): No requirement is needed if assuming SSBs on FDD PCell and SDL SCell are QCL-ed - Whether and how to define requirement for L1-RSRP/SINR measurement for SDL SCells in LB CA via switching - [Moderator]: check if following option 1 is agreeable or not. For other parts can be FFS after RAN1 concludes on the switching pattern. - Option 1: For SSB-less operation, UE can skip SSB/CSI-RS based L1-RSRP measurement on SDL SCell, and the L1 RSRP measurement on collocated PCell can be leveraged

Since SDL SCell has no uplink carrier and the SDL SCell is synchronized/collocated with FDD PCell, we don’t think it’s really necessary to require UE to perform BFD/CBD/PL-RS related measurement for SDL SCell. However, since SSB based operation can be used as baseline and network vendor still would like UE to keep monitoring Tx beams, we can compromise to have BFD/CBD requirement for SDL SCell from standard perspective, even though for low band carrier the beam related operation is not that necessary and practical. We also think the legacy BFD and CDB requirement can be reused.

Proposal 18: RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching and legacy BFD/CBD requirement for SCell can be reused.

Proposal 19: Not define requirement for PL-RS switch delay for SDL SCell in LB CA via switching.

Similarly, for SCell SDL TCI switching, for SSB based operation on SDL SCell, RAN4 can define the TCI switching delay requirement, and the existing TCI switching requirement can be used for LB CA via switching.

Proposal 20: RAN4 to define requirement for TCI switching delay for SDL SCell in LB CA via switching and existing TCI switching requirement can be reused.

For L1-RSRP/SINR measurement requirement for SDL SCell, similar as the SCell L3 measurement, UE can only perform the L1-RSRP/SINR measurement of SDL SCell on the overlapped occasions between L1 RS and SDL SCell reception duration indicated by switching pattern.

Proposal 21: RAN4 to define requirement for L1-RSRP/SINR measurement for SDL SCells in LB CA via switching.

Proposal 22: For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern.

Issue 1-3-5: others for SDL SCell

- Recommended WF - FFS on the potential CSSF change for both inside MG and outside MG cases.

Regarding the MG application, we identified one possible issue for SDL SCell measurement with MG. The existing CSSF for measurement inside MG is as following,

The intra-frequency MO without MG shall be counted in the CSSF inside MG only when it meet above highlighted conditions, however, the existing fully overlapped or partially overlapped conditions may not be accurate when LB CA via switching is used. Like in the figure 3, the MO on PCC is intra-frequency MO without MG, and the SMTCs on the PCC is not fully overlapped with MG for inter-frequency measurement, i.e., it’s partially overlapped case if only considering the SMTC configuration and MG configurations. However, effectively, UE cannot perform measurement in the SMTC of PCC when it’s colliding with the SDL SCC staying duration (in which UE switched to stay on SDL SCC and cannot receive anything from PCC). In this case, PCC SMTC and MG are fully overlapped effectively; and SDL SCC SMTC and MG are fully non-overlapped effectively

Figure 3. fully overlapped and partially overlapped conditions for CSSF in LB CA via switching

Thus, in LB CA via switching, the layers or MOs counted in the CSSF inside MG shall be based on the effective SMTCs of that layer or MO. The effective SMTC means:

(1) the SMTCs of PCC outside the duration time of staying on SDL SCC are treated as effective SMTCs of PCC; and

(2) the SMTCs of SDL SCC inside the duration time of staying on SDL SCC are treated as effective SMTCs of SCC

Figure 4. effective SMTCs to decide overlapping condition for CSSF inside MG

In summary, the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

Proposal 23: the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

# Potential impacts to the PCell related requirement

Issue 1-4-1: Impact on timing requirement for PCell

Regarding the UE timing, the current required UE to meet Te requirement only if at least one SSB is available in the last 160ms before UL transmission, as following. In our view, the design of switching pattern shall also make sure UE to not stay on SDL SCell for longer than 160ms, i.e., UE will have SSB reception available in last 160ms before the UL transmission on PCell. Or alternatively, since PCell and SDL SCell belongs to the same TAG and these cells are collocated and synchronized, we may allow reference cell switching between PCell and SDL SCell in same TAG for R19 DL CA, i.e., UE tracks timing on SDL SCell when it’s on SDL SCell following the switching pattern, and then such timing information can be used for UL transmission on PCell.

In last meeting, RAN1 had agreement below,

Agreement in RAN1#120bis - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction Agreement in RAN1#120bis For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms

Since the P=40ms is agreed as the periodicity of the semi-static switching pattern, that means UE will switch back to PCell at least every 40ms, and therefore we think it won’t happen that UE stay on SDL SCell too long such that UE lost PCell timing. RAN4 can reuse the existing timing requirement for PCell with the same applicability condition (SSB is available in last 160ms). At the meantime, we also agree that RAN4 may revisit the impact to PCell timing requirement if new P (e.g., P>160ms) is introduced by RAN1.

Proposal 24: the existing Tx timing requirement for PCell with the same applicability condition (SSB is available in last 160ms) can apply for R19 LB CA via switching. RAN4 may revisit the impact to PCell timing requirement if new P (e.g., P>160ms) is introduced by RAN1.

Issue 1-4-2: others RRM requirement of FDD PCell in LB CA via switching

In last meeting companies proposed two options for requirement design when CC switching is considered in issue 1-1-4, like followings:

Issue 1-1-4: RRM requirement applicability for R19 LB CA via switching - Recommended WF - [Moderator]: check which method can be used in P1. P2 can be further discussed in the detailed requirement design. - Discuss which method can be used: - Method 1: Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the aforementioned procedures. - Method 2: Specify an applicability condition under which the requirements are valid for low-band CA via switching

In our view, like the other interruption cases (e.g., SRS switching), RAN4 specified some kinds of requirement applicability to the victim cell like HD-FDD impact the L1 DL measurement in RedCap. We prefer to use method 2 for PCell requirement applicability.

Proposal 25: RAN4 to specify an applicability condition under which the existing requirements for PCell are still applicable to the PCell in LB CA via switching.

# Conclusion

In this contribution, we discuss the scope of corresponding RRM requirement for this R19 LB CA scenario.

Proposal 1: RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching, rather than using TRS/A-TRS based SDL SCell operation.

Proposal 2: The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

Proposal 3: For SDL to FDD switching, RAN4 to define scheduling restriction requirement for DL symbols reception on SDL FDD.

Proposal 4: For FDD to SDL switching, RAN4 to wait conclusion of switching gap from RAN1.

Proposal 5: RAN4 to define symbol level scheduling restriction requirement for carrier switching in R19 LB CA.

Proposal 6: For SDL to FDD switching, the scheduling restriction applies on the reception of those SDL SCC DL symbols that are overlapping with the switching gap.

Proposal 7: RAN4 to define the requirement for activated SDL SCell L3 measurement.

Proposal 8: For activated SDL SCell measurement, UE can only perform the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern.

Proposal 9: RAN4 to define the requirement for deactivated SDL SCell L3 measurement.

Proposal 10: The existing deactivated SCell measurement requirement can be reused for deactivated SDL SCell in LB CA via switching.

Proposal 11: For deactivated SDL SCell measurement, interruptions on PCell or activated SCell(s) are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell.

Proposal 12: No need to define side condition of RTD and power difference for SDL SCell activation requirement.

Proposal 13: SSB based SDL SCell activation shall be considered as baseline.

Proposal 14: the legacy FR1 inter-band SCell activation requirement can be reused to SDL SCell activation in LB CA via switching.

Proposal 15: the interruption pattern due to CC switching in SDL SCell activation shall follow SSB periodicity or SMTC periodicity used for SCell activation.

Proposal 16: the interruption length for each CC switching in SDL SCell activation can be defined as {(switching duration for FDD-to-SDL) + SSB/SMTC duration + (switching time for FDD-to-SDL)} or reuse same interruption requirement from legacy FR1 intra-band SCell activation.

Proposal 17: RAN4 to discuss the UE behavior if the CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation.

Proposal 18: RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching and legacy BFD/CBD requirement for SCell can be reused.

Proposal 19: Not define requirement for PL-RS switch delay for SDL SCell in LB CA via switching.

Proposal 20: RAN4 to define requirement for TCI switching delay for SDL SCell in LB CA via switching and existing TCI switching requirement can be reused.

Proposal 21: RAN4 to define requirement for L1-RSRP/SINR measurement for SDL SCells in LB CA via switching.

Proposal 22: For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern.

Proposal 23: the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

Proposal 24: the existing Tx timing requirement for PCell with the same applicability condition (SSB is available in last 160ms) can apply for R19 LB CA via switching. RAN4 may revisit the impact to PCell timing requirement if new P (e.g., P>160ms) is introduced by RAN1.

Proposal 25: RAN4 to specify an applicability condition under which the existing requirements for PCell are still applicable to the PCell in LB CA via switching.

# References

[1] R4-2504895, WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4 #114bis

[2] R4-2503614, Topic summary for [114bis][204] NR_LBCA_Sw, Apple, RAN4 #114bis

[3] R4-2503016, WF on low-low band switching, Apple, RAN4 #114