# 3GPP TSG-RAN WG4 Meeting #115	R4-2505929

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.28.5.2

Source:	Apple

Title:	On RRM core requirements for LP-WUR

Document for:	Discussion

# Introduction

In last RAN4#114bis meeting, the WF for RRM requirements for LP-WUS/WUR was agreed in [1], and open issues have been summarized in [2]. There was one LS[3] from RAN1 after last meeting, and some RAN4 actions are needed as following:

In RAN1, the following agreements were made on the RRM measurement metrics based on SSS for OFDM-based LP-WUR: Agreement (RAN1#120) Confirm the following working assumption with the additional text in red: Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications. - The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement. - FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement. Agreement (RAN1#120bis) For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable. From RAN1 perspective, it was agreed that SMTC window is not applicable for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell. RAN1 respectfully asks RAN4 to confirm it.

In this contribution, we continue the discussion on the open issues in [1].

# General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

We think the case #1 is the case when UE has very good quality of serving cell, and UE can fully offload from MR to LR. But if the serving cell quality is not that good as case #1, UE may consider to use MR relaxation for serving and neighbor cell measurement. Case #2 is a little bit weird case to consider, since when the LR is ON and neighbor cell measurement is OFF on MR, it seems not necessary for UE to keep MR awake for a relaxed serving cell measurement. In our view the threshold to trigger corresponding offloading and relaxation can be as following figure,

Figure 1. threshold assumption for different cases

If the serving cell measurement shows above the threshold of neighbor cell measurement (or a new threshold of MR offloading), UE can turn off the MR, and only rely on LR to perform serving cell measurement, and no neighbor cell measurement is needed (including higher or equal/lower priority layers measurement). If the neighbor cell measurement is triggered (e.g., below the threshold of intra-frequency or inter-frequency measurement triggering condition) but serving cell quality is still above threshold of LP-WUS monitoring threshold, the MR can be ON with relaxation for serving and neighbor cell measurement while the LR is also ON for serving cell measurement, i.e., case #3. Then if UE is below LP-WUS monitoring threshold but above the not-at-cell-edge threshold for RRM relaxation, then UE will be in a legacy power saving mode, which means, LR is OFF and MR is ON with neighbor cell measurement relaxation (no serving cell relaxation). Even further, if UE is below not-at-cell-edge threshold for RRM relaxation, it means UE is in legacy IDLE/Inactive mode status without any RRM relaxation.

To support the above logic, we have some assumptions for such threshold design, even though the final threshold decision shall be made by RAN2. We assume the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.

Based on the illustration in figure 1, we believe the case#2 and case#4 can be removed at least at this stage, but of course we can revisit the cases if RAN1/2 has some conclusions conflicting with RAN4 assumptions. And the following case table can be assumed in RAN4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 1: at this stage, remove case#2 and case#4 from the last meeting WF, and only following cases are considered. RAN4 can revisit the cases if in the future RAN1/2 has any conclusion conflicting with RAN4 assumptions.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 2: assumption of thresholds for RAN4 discussion can be:

- the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.

Issue 1-1-2: Threshold for switch between different cases

One thing to clarify that, LR measurement will be used for UE to decide switch from case#1 to case#3, that is, serving cell measurement on LR will be used to decide (1)if the neighbor cell measurement is triggered and (2)if the neighbor measurement can be relaxed. Previously in R16/17/18, the neighbor cell triggering and relaxation criteria checking is based on the MR measurement, and therefore it means LR measurement result shall be comparable to MR measurement or shall be equivalent to MR measurement with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

In last RAN2 meeting it was agreed that,

- RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any). - It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell. - The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ. - How to define LP-RSRP and LP-RSRQ is up to RAN1.

This issue has been discussed in RAN4 for couple of meetings, but we cannot foresee RAN2’s plan on discussing this dedicated issue. Thus, one possible way is to directly send LS to ask for RAN2’s guidance.

Proposal 3: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1).

Proposal 4: to support proposal 3, LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

Proposal 5: if RAN4 cannot make conclusion on issue 1-1-2, we recommend RAN4 sending LS to RAN2 for guidance.

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR serving cell measurement and MR RRM measurement relaxation

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

In WID, another important enhancement is: Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions. Even though conditions for offloading RRM serving cell measurement from MR to LR and conditions for relaxation of MR serving cell and neighbor cell measurements is mainly decided/designed by RAN2, RAN4 can investigate the mobility performance corresponding to such relaxation, and the mismatch/bias between LP-WUR based measurement result and MR based measurement result. For instance, if LP-SS is used for the measurement to decide when the MR shall be waken up or not, such LP-SS based measurement result at least shall be comparable to the MR based measurement results or some bias offset shall be defined; otherwise, LP-SS triggers MR to wake up for mobility but MR based measurement results may tell that the mobility is not needed after MR wakes up.

In previous RAN2 meeting, there was also some discussion that,

RAN2#128bis: - RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any). - RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both. - Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion). RAN2#128: - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. - FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation. RAN2#127: Serving cell measurements related For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. Neighboring cell measurements related RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. RAN2#127bis: Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Like in power saving feature, the criteria itself has been defined be RAN2 in TS38.304, and therefore here we also think that RAN2 is the main group for criteria (entry/exit conditions) design. However, for RRM relaxation requirement (e.g., scaling factor), RAN4 can work on the RRM measurement relaxations (e.g., scaling factor) and offloading mechanisms based on the criteria defined by RAN2.

Proposal 6: Criteria (entry/exit conditions) for LP-WUR serving cell measurement and MR RRM measurement relaxation can be left to RAN2 to decide.

Proposal 7: For the entry/exit condition for LP-WUS monitoring, follow RAN1/2 conclusions or can be triggered by other groups if necessary.

Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

RAN2 agreements in last meeting ### RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

We also think such discussion can be left to RAN2 as in proposal 6 and 7.

Issue 1-1-8: LP-WUR operating carrier frequency

In last RAN #106 meeting, a WF RP-243266 for LP-WUS was agreed.

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered. - Detailed discussions and final solution selection is left up to WGs.

The solution 1, 2, 3 was defined as in RP-242656, as following:

- Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band)

- Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria)

- Solution 3: MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation)

During the RAN plenary discussion, solution 2 is the solution which needs more further analysis. If the RACH can be performed on the different carrier from the SI/paging carrier, it’s unclear so far how UE can determine the RACH occasion and timing on the RACH carrier. Before concluding on solution 2, some aspects need to be clarified from our perspective, e.g.,:

- Is there any limitation on band combination to apply such RACH on different carrier? E.g., intra-band or inter-band?

- if DL synchronization is performed on one carrier which is for SI reading and paging, how to acquire timing and RO information for RACH on another carrier?

If network configures such RACH transmission mechanism of option 2 on intra-band contiguous or ono-contiguous carrier relative to the paging carrier and these two cells are collocated, the timing from PO cell can be roughly reused to decide the RACH timing for different cell, and RAN4 may further define the RTD limitation and power imbalance limitation to apply such mechanism, e.g., RTD<=CP and power imbalance limitation<=6dB.

Second question is how to acquire the RO information for RACH carrier/cell, it might be carried on SI in PO cell or might be carried on SI in RACH cell. If RO information is carried on RACH cell only, UE reads SIB only after receive paging on PO cell to trigger its RACH on RACH cell, and then UE needs to acquire timing/power on RACH cell as well as reading SIB for RO information.

Obviously, many open issues need to sort out before we can define requirements for such inter-carrier MR/LR scenario. In last RAN2 meeting, one LS has been agreed as following,

RAN2 would like to thank RAN1 for the LS on LP-WUS operation in IDLE/INACTIVE mode in R2-2407921/R1-2407559. In addition to RAN2 reply LS in R2-2410957, RAN2 further discussed the details/pros/cons of solutions based on the way forward from RAN plenary in RP-242656. The corresponding discussion report is in R2-2501443. And RAN2 achieved the following conclusions: - RAN2 understands that UE can report which band(s) is supported by LR to NW. - RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

According to the RAN2 reply, there is no new requirement needs to be defined in RAN4 for the above issue. However, RAN4 can wait one more meeting for RAN1 to make the decision based on RAN2’s reply.

Proposal 8: regarding RAN#106 WF RP-243266, to wait RAN1 for solution selection based on RAN2 reply LS (e.g. solution 1, 2, and 3 in RP-242656), and then RAN4 can work on the requirement design if needed.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

It was agreed in RAN4 #114 meeting, it was agreed that,

At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

It’s not impossible to verify a UE behavior which is purely up to UE implementation, and when UE will turn on LR is either not testable. Thus, we agree that: at legacy state, when LP-WUR is “ON” for evaluating LR threshold, LP-WUR measurement and evaluation requirement will not apply.

Proposal 9: No LP-WUR measurement and evaluation requirement applies at the legacy state.

Issue 1-1-10: Considerations on higher priority frequency layer

Agreement: - Support relaxed higher priority frequency layer search at case 1

In case #1, when UE fully offload from MR to LR, we assume measurement results from LP-WUR will decide whether the MR shall be waken up or not, however, the MR will be turned off or in ultra-deep sleep mode. The FFS part is mainly about whether the higher priority measurement shall still be kept on MR or not. Current higher priority measurement in RAN4 has two main delays: 1min and 1 hour, as followings.

In order to save the power of UE, if MR still needs to wake up every 1min, we didn’t see much gain for this feature, but we are fine to perform higher priority measurement on MR as slow as 1 hour per layer.

Proposal 10: Support relaxed higher priority frequency layer search at case 1, i.e., perform higher priority frequency layer search at least every 1 hour.

# Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-4-1: On LR measurement filtering requirement

Agreement: - The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR.

For LP-SS based measurement filtering, we need to review the LP-SS agreed in RAN1, as duplicated below,

Agreement in RAN1#120bis Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1, M=2: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support LP-SS periodicity of 160ms

One difference between SSB based measurement and LS-SS based measurement is NW will transmit multiple SSBs in between of two LP-WUS occasions while NW won’t transmit any LP-SS in between of a LP-SS periodicity. Thus, the LP-SS periodicity/2 cannot be used for spacing interval of LP-SS measurement filtering. We think the easiest way is to use LP-SS periodicity as the spacing interval for LP-SS measurement filtering, and no differentiation is needed for OFDM and OOK based receiver on LP-SS measurement.

Proposal 11: The UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by LP-SS periodicity for both OFDM and OOK based LR.

Issue 1-2-4-2: On LR requirements for entry/exit threshold evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3)

Issue 1-2-4-2-1: On applicability LR evaluation requirements

Recommendations: Suggest to confirm at least the following two applicability scenarios for LR evaluation requirements. - exit to legacy case from case 1 - exit to legacy case from case LP-WUS monitoring - exit to case 3 from case 1

As we discussed in proposal 9, the LR status in legacy state shall not be specified, and we only need to consider the LR requirement applicability of other cases switch to legacy case. And for all the other cases, like entry/exit of LP-WUS monitoring, case 3, or case 1 without legacy case involvement, we think LR evaluation requirement can be applied in general. Since whether case 3 and LP-WUS monitoring can share the same threshold is still open now, we put these two cases together for the time being.

The LR evaluation requirements will apply for the following scenario:

- exit to legacy case from case 1

- exit to legacy case from case LP-WUS monitoring and case 3

- exit to case 3 from case 1

- entry to case 1 from case 3 or LP-WUS monitoring (if thresholds for both case 1 and 3 are configured)

Proposal 12: The LR evaluation requirements will apply for the following scenario:

- exit to legacy case from case 1

- exit to legacy case from case LP-WUS monitoring and case 3

- exit to case 3 from case 1

- entry to case 1 from case 3 or LP-WUS monitoring (if thresholds for both case 1 and 3 are configured).

Issue 1-2-4-2-3: On how to define LR evaluation requirements

The agreements in RAN4#140 were:

Agreement: LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

The evaluation requirement (x1/y1) can be decided after we have simulation alignment for measurement requirement (x/y). Based on the observation of topic summary [2] in last meeting, the general sample number is 1~3 to meet the accuracy requirement, and whether and how UE to implement sliding window is purely to UE; thus, we can consider using x1=2*x and y1=2*y for the evaluation requirement.

Proposal 13: The evaluation requirement (x1/y1) can be decided after RAN4 has simulation alignment for measurement requirement (x/y). Consider using x1=2*x and y1=2*y for the evaluation requirement.

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR and RAN1 LS R1-2501624

Issue 1-2-6-1: On the number of SSBs

Agreement: Aim to conclude the issue by Tuesday in RAN4 #115.

Issue 1-2-6-2: On SSBs number for different SSB periodicities

Agreement Same SSB number for different SSB periodicity Agreement When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay. Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity

In RAN1 LS, the key agreements are:

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

In last RAN1 #120bis meeting, the updated conclusion on MR wake-up delay was:

Agreement On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, consider the following alternatives for possible down-selection in RAN1#121. - Alt 1: Do not report for SSB periodicities other than 20ms - Note: LP-WUS is not supported for SSB periodicities larger than 20ms - Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns): SSB periodicity (ms); Wake-up delay (ms) UE capability 1; Wake-up delay (ms) UE capability 2; Wake-up delay (ms) UE capability 3. [5]; [x]; [x]; [x]. [10]; [x]; [x]; [x]. 20; [70]; [500]; [900]. [40]; [x]; [x]; [x]. [80]; [x]; [x]; [x]. [160]; [x]; [x]; [x]. - Alt 3: It is up to RAN4

The definition of wake-up delay is: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring, that is same as what RAN4 discussed in last meeting, i.e., interruption requirement in P1 above. The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. Network needs to make sure the gap between LO for UE to monitor and PO shall be NO less than this wake-up delay, and that means it will not have any interruption to the PO reception due to the wake-up delay.

In short, we think such interruption requirement has already been reflected by UE capability indication as agreed in RAN1, thus no dedicated interruption requirement needs to be defined for this case. RAN4 only needs to define the wake-up delay requirement as defined in RAN1, if such requirement is necessary.

Proposal 14: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

In order to check the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms, there are some aspects for consideration:

- (1)Will the number of SSB for synchronization be impacted by the LP-SS periodicity before LP-WUS reception?

- (2)Will the number of SSB for synchronization be impacted by whether LP-SS or SSB is used for time tracking before LP-WUS reception?

- (3)Will the number of SSB for synchronization be impacted by the SINR condition?

If LP-SS has other periodicities besides 320ms, like 160ms, for different LP-SS periodicities, UE may need different SSB sample on MR for AGC settling and/or T/F tracking, and the principle could be larger LP-SS periodicity on LR needs more SSB samples occasions on MR for MR waking-up procedure. However, since MR is waken up after UE successfully received the LP-WUS (read it successfully), the coarse timing information shall already be good enough from LP-SS for LP-WUS decoding. Thus, the answer to question 1 can be No.

If the SSB is used for timing tracking before LP-WUS, UE can use SSB for AGC settling directly, and then during the MR wake-up procedure the sample of SSBs for ACG settling can be saved compared with the “LP-SS based tracking before the LP-WUS”, i.e., SSB based tracking before LP-WUS may need less SSB samples for MR wake-up than LP-SS based tracking. In last meeting RAN1 agreed, the EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB. That means, the coarse AGC estimation from LP-SS symbols may be also usable for AGC settling during MR wake-up procedure. Thus, the answer to question 2 can be No.

In our view, the MR wake-up delay capability is applied to the MR wake-up in case 1 once LP-WUS is received (fully offloading). In the above figure 1, we assume the SINR when entering case can be higher than -3dB, and the wake-up delay for MR can consist of MR ramping-up time, AGC fine tuning time, and T/F tracking time. With such high SINR condition, the 5 SSBs are sufficient for AGC fine tuning and T/F tracking, but RAN4 may need to check if such sample number needs to be revisited for lower SINR condition, e.g., the SINR is dropped when UE received LP-WUS in case 1, like in the following figure 2. The principle could be: lower SINR case needs more SSB sample number on MR synchronization than the higher SINR case. Thus, the wake-up delay capability and the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms shall be associated with specific target SINR condition, e.g., ≥-3dB.

Figure 2. the SINR condition for SSB sample number for MR wake-up

Since RAN4 agreed that, when SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay, and therefore we prefer to use Alt 2 in RAN1 for UE capability, and the wake-up delay requirement can be defined based on such UE capability.

- Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns):

| SSB periodicity (ms) | Wake-up delay (ms) UE capability 1 | Wake-up delay (ms) UE capability 2 | Wake-up delay (ms) UE capability 3 | Note |
| --- | --- | --- | --- | --- |
| [5] | [70] | [500] | [900] | when SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay |
| [10] | [70] | [500] | [900] |  |
| 20 | [70] | [500] | [900] |  |
| [40] | [10+3*40]=130 | [400+5*40]=600 | [800+5*40]=1000 | Capability 1: 10ms RF ramping up and 3 SSB occasions for synchronization Capability 2: 400ms RF ramping up and 5 SSB occasions for synchronization Capability 3: 800ms RF ramping up and 5 SSB occasions for synchronization |
| [80] | [10+3*80]=250 | [400+5*80]=800 | [800+5*80]=1200 |  |
| [160] | [10+3*160]=490 | [400+5*160]=1200 | [800+5*160]=1600 |  |

We propose to reply to RAN1 that,

RAN4 confirms the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms is sufficient, and it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

In addition, RAN4 assumes this MR wake-up delay capability and the number of SSBs for MR synchronization shall be associated with specific target SINR condition, e.g., ≥-3dB.

Proposal 15: RAN4 to reply to RAN1 LS that,

- RAN4 confirms the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms is sufficient, and it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

- In addition, RAN4 assumes this MR wake-up delay capability and the number of SSBs for MR synchronization shall be associated with specific target SINR condition, e.g., ≥-3dB.

# LP-WUR CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

Suggestion from the proposed company: Discuss the interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. Check whether the interruption requirements specified in Clause 8.2 of TS 38.133 also apply to WUS monitoring by WUR in the Connected mode

In the WID, the scope for connected mode is to discuss the procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring, however, like in R16 WUS design, it would not have any RRM impact by using LP-WUS in connected mode. Furthermore, WID mentioned that,

In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

All the RRM/RLM/BFD/CSI measurement would be as same as legacy case, i.e., use MR to perform these functionalities. However, in last meeting, companies whether interruption requirement shall be applied to LP-WUS monitoring in connected mode or not.

RAN1 agreed in RAN1#120 that,

Agreement

For RRC connected mode, UE does not monitor LP-WUS during C-DRX active time

- FFS FAR and MDR impact

Thus, the interruption to LP-WUS shall be the ones which is outside active time of C-DRX. However, so far the dual-DRX group for CA/DC for LP-WUS is still under discussing, we still think we shall delay such discussion until the DRX related discussion has been concluded in RAN1/2.

Proposal 16: for LP-WUR at CONNECTED mode, discuss the interruption requirement for LP-WUS monitoring after the DRX related discussion is concluded in RAN1/2.

# Others

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode

In RAN4 #111 and 112 meeting, the different cases for MR offloading and relaxation are captured in the WF R4-2410296 and R4-2413899, and so far, at least case#1 and case#3 are valid case (like in proposal 13) to consider in this WI,

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

However, the EMR measurement has not been considered yet for both case #1 and case #3. In legacy IDLE/Inactive mode RRM requirement, the relaxation is not allowed when EMR is configured on that frequency layer and T331 is running, like followings:

Then, if EMR measurement is also configured and TS331 is running for case#1 and case#3, the MR status may need further discussion. For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, 	UE may keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

Proposal 17: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

Old issue from RAN4#112: eDRX related

As discussed previously, if the LP-SS based measurement is not following eDRX cycle or DRX cycle in IDLE/Inactive mode, i.e., LP-SS based measurement will follow the LP-SS periodicity, then when UE decides to turn on MR inside a PTW window (if PTW is used), UE needs to check whether rest of the PTW window is sufficient for MR based RRM measurement or evaluation time (MR based RRM measurement/evaluation shall be contained in one PTW window). We think it’s still worthwhile to discuss how to perform offloading based on LP-SS based measurement when eDRX is configured.

Figure 3. example for MR offloading(based on LP-SS based measurement) when eDRX is configured with PTW

Proposal 18: RAN4 to discuss followings LP-SS based RRM issue in IDLE/Inactive mode:

- how to enter and exit offloading status if eDRX is configured with PTW.

New LS from RAN1[3]

In last meeting, RAN1 sent LS to RAN4 to check the following:

From RAN1 perspective, it was agreed that SMTC window is not applicable for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell. RAN1 respectfully asks RAN4 to confirm it.

Even though the measurement is based on SSB, the OFDM based LR could have totally different timeline to do SSB measurement since the serving cell SSB periodicity is known to the UE. In addition, we assume that the SSB detection mainly relies on MR, i.e., MR will perform the SSB detection and coarse synchronization, and then LR will keep tracking on the SSB detected by MR. Thus, LR is able to track the SSBs even outside the SMTC window for the serving cell, and we think the SMTC window shall not be a limit to LR measurement.

Proposal 19: to reply RAN1 LS R1-2503103, RAN4 to confirm that SMTC window is not applicable for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell.

# Conclusion

In this contribution, we continue the discussion on the open issues in [2].

Proposal 1: at this stage, remove case#2 and case#4 from the last meeting WF, and only following cases are considered. RAN4 can revisit the cases if in the future RAN1/2 has any conclusion conflicting with RAN4 assumptions.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 2: assumption of thresholds for RAN4 discussion can be:

- the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.

Proposal 3: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1).

Proposal 4: to support proposal 3, LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

Proposal 5: if RAN4 cannot make conclusion on issue 1-1-2, we recommend RAN4 sending LS to RAN2 for guidance.

Proposal 6: Criteria (entry/exit conditions) for LP-WUR serving cell measurement and MR RRM measurement relaxation can be left to RAN2 to decide.

Proposal 7: For the entry/exit condition for LP-WUS monitoring, follow RAN1/2 conclusions or can be triggered by other groups if necessary.

Proposal 8: regarding RAN#106 WF RP-243266, to wait RAN1 for solution selection based on RAN2 reply LS (e.g. solution 1, 2, and 3 in RP-242656), and then RAN4 can work on the requirement design if needed.

Proposal 9: No LP-WUR measurement and evaluation requirement applies at the legacy state.

Proposal 10: Support relaxed higher priority frequency layer search at case 1, i.e., perform higher priority frequency layer search at least every 1 hour.

Proposal 11: The UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by LP-SS periodicity for both OFDM and OOK based LR.

Proposal 12: The LR evaluation requirements will apply for the following scenario:

- exit to legacy case from case 1

- exit to legacy case from case LP-WUS monitoring and case 3

- exit to case 3 from case 1

- entry to case 1 from case 3 or LP-WUS monitoring (if thresholds for both case 1 and 3 are configured).

Proposal 13: The evaluation requirement (x1/y1) can be decided after RAN4 has simulation alignment for measurement requirement (x/y). Consider using x1=2*x and y1=2*y for the evaluation requirement.

Proposal 14: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

Proposal 15: RAN4 to reply to RAN1 LS that,

- RAN4 confirms the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms is sufficient, and it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

- In addition, RAN4 assumes this MR wake-up delay capability and the number of SSBs for MR synchronization shall be associated with specific target SINR condition, e.g., ≥-3dB.

Proposal 16: for LP-WUR at CONNECTED mode, discuss the interruption requirement for LP-WUS monitoring after the DRX related discussion is concluded in RAN1/2.

Proposal 17: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

Proposal 18: RAN4 to discuss followings LP-SS based RRM issue in IDLE/Inactive mode:

- how to enter and exit offloading status if eDRX is configured with PTW.

Proposal 19: to reply RAN1 LS R1-2503103, RAN4 to confirm that SMTC window is not applicable for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell.

# References

[1] R4-2504908, WF on RRM requirements for NR_LPWUS, Moderator (vivo), RAN4 #114bis

[2] R4-2503627, Topic summary for [114bis][217] NR_LPWUS, vivo, RAN4 #114bis

[3] R1-2503103, LS on the RRM measurement metrics for OFDM-based LP-WUR, Apple, RAN1 #120bis