# 3GPP TSG-RAN WG4 Meeting #115	R4-2505932

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.31.5.1

Source:	Apple

Title:	On R19 NTN (e)RedCap RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#114, and the open issues have been summarized in the topic summary for [114bis][221] NR_NTN_Ph3_Part1 [2].

In this contribution, we continue the discussion on the RRM requirement for R19 NTN (e)RedCap.

# RRM for support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN

Issue 1-2-1-1a: The proximity distance between two SMTCs

<Way Forward>: - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms. - Y=5 ms

With the multiple SMTC configured on the same carrier, in legacy NTN requirement, RAN4 defined the proximity for the SMTCs as following,

9.1C.9.3 Collision between multiple SMTCs on a SAN carrier Collision between occasions of multiple SMTCs may occur as specified in this clause. When UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4 ms. The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

The proximity distance between the SMTCs reflects the UE capability of how fast it can complete the post-processing for the data buffered from prior SMTC. If network configures 2 SMTCs on a NTN carriers and the proximity distance between these two SMTCs are less than 4ms, UE is required to measurement only one of them in legacy NTN. Now, considering the RedCap UE has reduced UE processing capability than legacy non-RedCap UE, the proximity distance might need to be revisited in the spec. We think the relaxation for proximity could be considered for (e)RedCap UE in NTN, e.g.,

    - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

      - the two SMTC occasions are fully or partially overlapping in time domain, or

      - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)). We propose x=1ms.

    - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

Proposal 1: When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

- the two SMTC occasions are fully or partially overlapping in time domain, or

- the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms.

  - Y=5 ms.

#### Issue 3-1-1: General HD-FDD related impact

Agreement: - The legacy requirements and applicable conditions can be reused as baseline: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - [Minimum requirement for L1 indication for RLM and LR] - Scheduling availability of UE performing intra/inter measurements RAN4 to further discuss: - The legacy requirements and applicable conditions can be reused as baseline for defining requirements of RedCap UE in NTN scenario, and discuss whether there is spec impact for the following requirements by HD-FDD: - FFS: Scheduling availability of UE during L1-RSRP measurement - Other requirement is not precluded - Candidate options for the potential spec impact for each of the above requirements (if needed): - Option a: the requirement is extended, e.g., the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Option b: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Option c: The requirement applies when there is at least one RS within each measuremen interval. - FFS the same or different option for DRX and non-DRX

In legacy RLM/BFD requirement for RedCap HD-FDD, it was specified that,

For HD-FDD UE, the above conditions and requirements TIndication_interval_BFD_Redcap apply given that for each BFD-RS configuration, at least one BFD-RS sample must fall with DL occasion within an indication period

It’s very clearly defined to have an applicability condition of “at least one RS sample must fall with DL occasion within an indication period”. Thus, we propose the legacy RedCap requirements and applicable conditions can be reused as baseline for RLM and BFD of RedCap UE in NTN scenario.

Proposal 2: the legacy RedCap requirements and applicable conditions can be reused as baseline for RLM and BFD of RedCap UE in NTN scenario.

RAN1 has designed the prioritization rule between UL and DL for different scenarios/cases in previous meeting, but our understanding is: to RAN4, as long as there is at least one RS within each measurement interval, the requirement shall be applied, otherwise, longer measurement delay shall be expected, or no requirement shall be applied. However, if a-periodic measurement is scheduled, the following agreement from last RAN1 meeting can be used as reference:

For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all cases - This is signaled by one UE specific RRC parameter Note1: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note2: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

Thus, RRM requirement for a-periodic CSI-RS based L1 measurement shall be applied when this AP CSI-RS is prioritized following RAN1 agreed handling rule.

Proposal 3: to RAN4, as long as there is at least one RS within each measurement interval, the requirement shall be applied, otherwise, longer measurement delay shall be expected or no requirement shall be applied.

Proposal 4: RRM requirement for AP CSI-RS based L1 measurement shall be applied when this AP CSI-RS is prioritized following RAN1 agreed handling rule.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

- the two SMTC occasions are fully or partially overlapping in time domain, or

- the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms.

  - Y=5 ms.

Proposal 2: the legacy RedCap requirements and applicable conditions can be reused as baseline for RLM and BFD of RedCap UE in NTN scenario.

Proposal 3: to RAN4, as long as there is at least one RS within each measurement interval, the requirement shall be applied, otherwise, longer measurement delay shall be expected or no requirement shall be applied.

Proposal 4: RRM requirement for AP CSI-RS based L1 measurement shall be applied when this AP CSI-RS is prioritized following RAN1 agreed handling rule.

# References

[1] R4-2504912, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT, RAN4#114bis

[2] R4-2503631, Topic summary for [114bis][221] NR_NTN_Ph3_Part1, CATT, RAN4#114bis