# 3GPP TSG-RAN WG4 Meeting #115	R4-2505934

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.31.5.2

Source:	Apple

Title:	On R19 other NTN RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#114, and the open issues have been summarized in the topic summary for [114bis][222] NR_NTN_Ph3_Part2 [2].

In last meeting, RAN1 sent LS[3] to RAN4 for the following question:

For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe.

In this contribution, we continue the discussion on the other RRM requirements for R19 NTN.

# Topic #1: SSB periodicity larger than 20 ms

## Issue 1-0: SMTC configuration and actual SSB transmission period

FFS: Wait for RAN2 progress and discuss the following - Decide whether the following needs to be captured only in the RAN4 specification as part of the requirement applicability rule, or if the group will send an LS to RAN2 to ensure the following constraint is included in their specifications. - If different neighbour cells in the same frequency layer have different SSB periodicities, and if SMTC is not configured to match the largest SSB periodicity among the cells, the periodicity and active patterns of the neighbour cells shall be informed to the UE to assist with UE measurement. - Further discuss the following if the decision needs to be made in RAN4 RRM: - For UEs supporting DL-CE feature in NR Rel-19, it shall be mandatory the support to 4 SMTCs per frequency layer

## In last RAN2#129bis meeting, followings were agreed:

## 3. RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases). ## 4. We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking) ## 5. Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode. ## In LS to RAN4, RAN2 agreed that, ## For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

## Firstly, we need to differentiate the cases between connected mode and IDLE/inactive mode. For RRC connected mode, network is aware of which serving cell or footprint UE is located in, and therefore the RRC configuration of SMTC shall consider the rough UE location (cell level). In RAN2 LS, RAN2 proposed to have two different SMTC periodicities (with different offsets) in one frequency layer, which means the different offsets are applied for different SMTC periodicity, however, RAN2 didn’t limit if these SMTCs can be overlapped on time domain or not. In the following example, if SSB of target satellite is contained in both SMTC1 and SMTC2 (SMTC1/2 has different periodicity), it will still confuse UE implementation which of SMTC UE shall follow for the target satellite measurement, i.e., in case UE follow the SMTC1 to perform measurement of that SSB, UE will fail the measurement because some of the SMTC1 occasion has no such SSB.

Figure 1. SSB of target satellite is contained in both SMTC 1 and 2

In IDLE/Inactive mode, network is unaware of any serving cell information for a particular UE. RAN2 agreed that network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. That can really help UE to understand which SMTC(s) can be used for cell searching at a specific location, but again this cannot address the issue when SMTC periodicity is smaller than SSB periodicity of a target satellite as well as the issue in figure 1.

In our view, there might be two alternatives to address this issue:

Alt1: if configured SMTC periodicity is smaller than the largest SSB periodicity among the cells on same frequency layer, the association between SMTC periodicity and neighbour cells shall be informed to the UE to assist with UE measurement.

Alt 2: in RAN4 we define applicability for RRM requirement that: no requirement applies if configured SMTC periodicity is smaller than the largest SSB periodicity among the cells on same frequency layer.

## Proposal 1: following alternatives are proposed:

- Alt1: if configured SMTC periodicity is smaller than the largest SSB periodicity among the cells on same frequency layer, the association between SMTC periodicity and neighbour cells shall be informed to the UE to assist with UE measurement.

- Alt 2: in RAN4 we define applicability for RRM requirement that: no requirement applies if configured SMTC periodicity is smaller than the largest SSB periodicity among the cells on same frequency layer.

## Regarding the UE capability of supporting 4 SMTCs per frequency layer, RAN2 had very clear agreement that at any time the UE will not use more SMTCs in parallel than in previous releases. Thus, we don’t think RAN4 shall mandate UE to support 4 SMTCs per frequency layers for DL-CE feature in NR Rel-19.

## Proposal 2: no need to modify Feature 25-1(parallelSMTC-r17) for UEs supporting DL-CE feature in NR Rel-19.

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i)

Agreement (online): - In RRC re-establishment requirements, for case where the SMTC for the inter-frequency carrier is not provided, the value of TSMTC,i is assumed to be 160ms: - Discuss the applicability of the requirement for with the updated value of TSMTC,i to 160ms after the related RAN1/2 agreement. FFS: Wait for RAN2 progress and discuss the following - The requirements for intra-frequency and inter-frequency RRC re-establishment are applicable provided that the configured SMTC period is not shorter than the periodicity of the actual SSB transmission of the target cell.

## Same discussion for issue 1-0, we are fine with alternative 1 and 2 in proposal 1.

## Reply to RAN2 LS:

## In our view, configuring “two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements” won’t impact UE’s implementation too much, however, like in issue 1-0 discussion, RAN4 needs to confirm with RAN2 if the association between SMTC periodicity and neighbor cells shall be provided to UE or not, otherwise, RAN4 will define applicability rule for the case when configured SMTC periodicity is smaller than the largest SSB periodicity among the cells on same frequency layer. There also might be some clarification needs to be done in RAN4 NTN requirement, e.g., in 9.3C.4 Inter-frequency measurement with measurement gaps,

## The SMTC period in the above requirement for K is not very clear if SMTC configuration can have 2 SMTC periodicities. In our view, here the SMTC period shall be the SMTC which is used for target satellite measurement, like in pervious SMTC1 and SMTC2 discussion for TN, we can consider to clarify that: if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified/measured.

## Proposal 3: to reply RAN2 that,

- It is feasible to support these RAN2 agreements in Rel-19 timeframe.

- Need RAN2 to confirm whether the association between SMTC periodicity and neighbor cells shall be provided to UE when configured SMTC periodicity is smaller than the largest SSB periodicity among the cells on same frequency layer.

## Proposal 4: if RAN4 agrees to have two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements, some clarifications are needed for existing NTN RRM requirement, e.g., like TN requirement for SMTC1 and SMTC2, it can clarify in the corresponding requirement that, “if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified/measured”.

# Topic #2: Satellite Beam Hopping (e.g. Cell DTX/DRX)

In last RAN1 FL Summary #4: NR-NTN downlink coverage enhancements (R1-2501728), following issue was discussed:

However, no conclusion has been achieved in last RAN1 meeting. In the current TN FR1 scheduling restriction for L1 or L3 measurement, it has two main kinds:

- TDD based scheduling restriction because UE cannot do UL transmission when UE is operation on DL for neighbor cell measurement.

- Other scheduling restriction applies because UE cannot do e.g., mixed numerology Rx or Tx when UE is performing SSB or CSI-RS measurement.

R17 NTN has new scheduling restriction if UE does’t support parallelMeasurementWithoutRestriction, when UE is performing SS-RSRP/RSRQ/SINR measurement on a neighbor cell served by a different satellite in LEO.

The above scheduling restriction happens during the SSB measurement occasions of the target cells, e.g., UE is doing SSB measurement but cannot receive the data/control or transmit data/control/RACH to the serving satellite. However, in this special case for DL coverage enhancement with Tx beam hopping, even on other time period outside the configured SSB occasions, NW may be still not possible to schedule or receive any channel/signal to or from UE, as shown in following figure 1. To the example UE in cell 2, outside the cell 2 SSB occasions, before T2 and after T2+40ms, NW hopped Tx beams to other cells (cell 1/3/4), and therefore serving satellite cannot transmit/receive anything to or from UE in cell 2. Therefore, we need to reconsider the scheduling restriction applicability in this scenario of DL coverage enhancement.

Figure 2. cell 2 UE(s) may not transmit or receive signaling from satellite when beam hopped away

If the RO periodicity configuration from serving cell is not greater than the SSB periodicity (less than 160ms in the example figure 2), the scheduling restriction applies to UE RACH and RACH related channels transmission outside the time period of serving Tx beam availability, time period of serving Tx beam availability means, e.g., the time period between the beginning of slot containing available SSB (i.e., T2 in example figure 2) and (original SSB periodicity + T2). Thus, the scheduling restriction for RACH could be: If DL coverage enhancement or satellite beam hopping is enabled, the UE is not expected to transmit PRACH on symbols outside the serving satellite Tx/Rx beam availability durations.

Figure 3. example for RO in SSB periodicity

Proposal 5: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

## Proposal 1: following alternatives are proposed:

- Alt1: if configured SMTC periodicity is smaller than the largest SSB periodicity among the cells on same frequency layer, the association between SMTC periodicity and neighbour cells shall be informed to the UE to assist with UE measurement.

- Alt 2: in RAN4 we define applicability for RRM requirement that: no requirement applies if configured SMTC periodicity is smaller than the largest SSB periodicity among the cells on same frequency layer.

## Proposal 2: no need to modify Feature 25-1(parallelSMTC-r17) for UEs supporting DL-CE feature in NR Rel-19.

## Proposal 3: to reply RAN2 that,

- It is feasible to support these RAN2 agreements in Rel-19 timeframe.

- Need RAN2 to confirm whether the association between SMTC periodicity and neighbor cells shall be provided to UE when configured SMTC periodicity is smaller than the largest SSB periodicity among the cells on same frequency layer.

## Proposal 4: if RAN4 agrees to have two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements, some clarifications are needed for existing NTN RRM requirement, e.g., like TN requirement for SMTC1 and SMTC2, it can clarify in the corresponding requirement that, “if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified/measured”.

Proposal 5: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE.

# References

[1] R4-2504913, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm, RAN4#114bis

[2] R4-2503632, Topic summary for [114bis][222] NR_NTN_Ph3_Part2, Qualcomm, RAN4#114bis

[3] R2-2503068, LS on SMTC enhancements, Xiaomi, RAN2#129bis