# 3GPP TSG-RAN WG4 Meeting #115	R4-2505935

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.18.2

Source:	Apple

Title:	On feature list for RRM phase 5 WI

Document for:	Discussion

# Introduction

In this contribution, we discuss the UE capability and feature groups for RRM phase 5 WI.

# Detailed design for capability of BSF enhancement

At the last meeting, the following agreement on UE capability has been made:

### Issue 1-2-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Agreement in the ad-hoc: - Introduce the UE capability of reduced N, the values of which are {2, 4 and 6}. And UE chooses one value to report according to its capability. FFS the granularity of the UE capability in issue 1-2-4.

Regarding the capability type, we propose to make it “per band” because the UE antenna panels in FR2-1 are usually band-specific. This also aligns with the R18 multi-RX UE capability “Fast beam sweeping for layer-1 measurement when the UE is in multi-Rx operation.” Therefore, we propose to specify the UE capability as follows:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49- | Fast Rx beam sweeping for FR2-1 L3 measurement delay reduction | - Supports beam sweeping factor reduction for L3 measurement for FR2-1. |  | Yes | N/A |  | Per band | TDD only | FR2-1 only |  | Candidate values: {2,4,6} for FR2-1 Note: It is only supported for power class 3. | Optional with capability signalling |

# Detailed design for capability of CSSF enhancement

In previous meeting, the agreements for solution 1 and solution 3 under CSSF enhancement were:

Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19. Agreement: - UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR. - UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

Based on the template shared by moderator of the feature list thread, the following detailed capability design is proposed as followings.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-x | One serving carrier measurement per-band for intra-frequency measurements without measurement gap | Support of measuring serving cell and neighbor cells on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured for measurement in a same band |  | Y |  | UE does not support measuring serving cell and neighbor cells on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured for measurement in a same band. | Per-UE | No | Yes |  | UE is required to meet the serving cell and neighbor cells measurement requirements in TS38.133 [section 9.2.3.2, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.3, and 9.1.5.1.4] if the feature is supported. | Optional with capability signaling |
| 49. NR_RRM_Ph5 | 49-y | Parallel measurement on three serving carriers for intra-frequency measurements without measurement gap | Support of measuring serving cell and neighbor cells on three serving carriers for intra-frequency measurements without measurement gap. There are separated indications of supporting this feature for different cases: {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, and {FR1+FR2 NR-DC}. |  | Y |  | UE does not support measuring serving cell and neighbor cells on three serving carriers for intra-frequency measurements without measurement gap. | Per-UE | No | Yes |  | UE is required to meet the serving cell and neighbor cells measurement requirements in TS38.133 [section 9.1.5.1.1, 9.1.5.1.2, and 9.1.5.1.3] if the feature is supported. | Optional with capability signaling |

# Detailed design for capability of EMR enhancement

New fast SCell activation delay requirements are not 100% same as known cell activation delay. A R18 UE supporting R18 eEMR probably can only meet legacy known cell activation delay. Therefore, a new UE capability is needed.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-x | Fast SCell activation based on early measurement report | - Supports fast SCell activation specified in TS38.133 clause 8.3.2A | 39-8 or 39-9 in R18 RAN4 feature list | No | N/A | UE cannot meet fast SCell activation requirements as specified in TS38.133 clause 8.3.2A | Per-UE | No | No |  |  | Optional without capability signalling |

# Conclusion

In this contribution, we discuss the UE capability and feature groups for RRM phase 5 WI.

# References