3GPP TSG-RAN WG4 Meeting #115                            R4-2505944

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.22.3

Source:	Apple

Title:	Study on CSI Compression: Testability and Interoperability issues

Release:       Rel-19

Document for:	Discussion

# 1 Introduction

Rel-19 Work Item (WI) was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (WID in [1]). The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air.

This work item provides normative support for the general framework of AI/ML concerning air interfaces. It also enables the implementation of recommended use cases outlined in the previous study. Furthermore, several study objectives within this project aim to address outstanding issues identified during the study, with the goal of enhancing understanding in preparation for future normative effort.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [2]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for AI/ML for NR air interface for CSI compression and prediction.

# 2 Testability and interoperability issues for CSI compression Option 3

In R4-2420334/ R4-2503953 it was agreed to perform another round of simulations based on some agreements on model alignment of model training and data set parameters for AI/ML-based CSI compression it was agreed to study the feasibility of the test decoder for Option 3.

Issue 1-2: Option 3 next step - decoder(s) selection

The following agreement was reached:

Agreement:

choose the decoders with Low, medium, high SGCS among the decoders submitted by companies

Continue the feasibility study on the two tracks below:

Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset)

-	The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders

-	At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Each company is encouraged to train own encoder using the fixed decoder and their own dataset, and then test with every companies datasets, in order to check for consistency in results

Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s).

-	At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Once the frozen decoder is selected, companies are invited to train their own encoders using the mixed dataset or/and private dataset and test using the mixed testset

2.1 Simulation parameters for verifying the feasibility of aligning model among companies

The following simulation parameters have been agreed:

System-level simulation parameters (from TR):

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

Based on the two rounds of discussions, the following parameters were agreed for the AI models:

Fig 1. Detailed model diagrams of encoder.

Fig 2. Detailed model diagrams of decoder.

System parameters:

| Frequency range | 2GHz (4GHz optional) |
| --- | --- |
| MIMO Layers | 1 layer (2 layers optional, companies to |
| Sub-bands | 13 |
| Channel estimation | ideal |
| Metrics | SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI. |

2.2. Simulation results for the agreed test decoder for Option 3: Track-1

In this section we provide simulation results for the test decoders for Option 3: Samsung test decoder, CATT and HW decoder for the quantized models.

We perform quantization aware training by bypassing the quantization in the backward propagation to enable zero gradient (due to round) and enable learning.

CATT test decoder: We trained the quantization aware test decoder developed by CATT along with our own Encoder and own dataset. Subsequently, we tested the trained Enc/Dec pair model with datasets across different companies as shown in Fig 3.

Fig 3. Training/testing of CATT Quantized Test Decoder

Samsung quantized test decoder: We trained the quantization aware test decoder developed from Samsung along with our own Encoder and Apple’s dataset (/NOK dataset). Subsequently, we tested the trained Enc/Dec pair model with datasets across different companies as shown in Fig 4 / Fig 5.

Fig 4. Training/testing of Samsung Quantized Test Decoder with Own dataset

Fig 5. Training/testing of Samsung Quantized Test Decoder with NOK dataset

HW test decoder:  We trained the quantization aware test decoder developed from Huawei along with our own Encoder and Apple’s dataset (/Ericsson dataset). Subsequently, we tested the trained Enc/Dec pair model with datasets across different companies as shown in Fig 6 / Fig 7.

Fig 6. Training/testing of Huawei Quantized Test Decoder with Apple dataset

Fig 7. Training/testing of Huawei Quantized Test Decoder with Ericsson dataset

Observation 1: The feasibility of Option 3 Test Decoder under Track 1 cannot be established for all the chosen test decoders due to the performance dependency on the testing dataset. That is, in the absence of an agreed training dataset, if each company trains each own encoder with own training dataset, then the testing performance at TE with the trained DUT will be testing-dataset dependent. DUT will perform differently depending on the testing dataset. From simulation results, there is a statistical difference between companies’ datasets.

Proposal 1: To establish the feasibility of the Test Decoder under Option 3 and Track1, each company should train its own encoder using the fully specified Test Decoder and its own training data. The trained encoder should then be tested with the Test Decoder using datasets from different companies and demonstrate performance consistency

To investigate the SGCS performance dependency on the dataset, we propose to define a similarity/dissimilarity metric between CSI datasets. In such a way we could confirm that the performance degradation is a result from the dataset mismatch metric.

Proposal 2: Investigate a Channel State Information (CSI) metric that provides a statistical measure of how well CSI can be compressed, directly translating to SGCS performance. Using this approach, a dissimilarity metric between CSI training and testing datasets can be derived to predict or interpret SGCS results. One such metric could be Angular-Delay Channel Entropy (ADCE), which quantifies the uncertainty in CSI’s spectral distribution and reflects its compressibility. The ADCE can be defined as ADCE=ijPijlog2Pij, where Pij=Hij2||H||F2, and Hij=FdHFaH is a virtual channel representation in angular/delay domain which separates multipath components into resolvable delay taps and spatial angles. ADCE captures the sparsity or randomness of these components.

By doing so, the test decoder would be deemed feasible if we knew that the QC dataset had a high ADCE (less compressible). We could then design the testing framework according to the characteristics of the testing CSI metric suggested in proposal 2.

2.3. Simulation results for the agreed test decoder for Option 3: Track-2

For Track-2 we simulated a system comprising of our own Encoder and test decoder (Samsung), trained with a Mixed dataset as it was specified in the reflector. After training, we tested the pair of trained Encoder and Decoder with the mixed testing dataset as well as across different companies test dataset. This is shown in Fig 8.

Fig 8. Performance with trained Enc/Dec with mixed dataset (training =mixed dataset)

Fig 9. Performance with trained Enc/Dec with mixed dataset (training =Apple dataset)

Observation 2: Performance of Own trained (mixed dataset) Encoder with Mixed Dataset trained Decoder is more consistent across various companies testing datasets. However, the performance on testing datasets that don’t belong to the training dataset (QC, CATT) is significantly degraded mirroring the issues observed in track1.

Proposal 3: To determine the initial feasibility of test decoder under Option 3 and Track 2, each company to train Own Encoder with Test decoder with the mixed dataset. Subsequently each company tests the Enc/Dec pair with the mixed dataset to ensure performance consistency across all companies.

Remaining issue is how to guarantee that if DUT passes the test (trained/tested with synthetic data) it also performs well in the field (with field data)

Proposal 4: RAN4 to investigate potential performance loss due to the dataset distribution mismatch (field data distributions) when one or both parts of the AI/ML models are specified with a synthetic dataset.

Proposal 5: RAN4 to investigate the similarity between Option 3 and Inter-vendor collaboration direction C (Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1) from RAN1 study

Proposal 6: RAN4 to investigate how many test decoders need to be specified under Option 3. Identify the granularity of scenarios/deployments that will dictate a new test decoder.

Proposal 7: To complete feasibility analysis under Option 3, the following question should be answered:

Performance Consistency Across UE Encoder Implementations: Can the performance be maintained across different UE encoder implementations? Specifically, if the UE encoder uses a different AI/ML backbone (e.g., a Transformer-based architecture), does the system still achieve comparable results?

Proposal 8: To align with real-world deployment scenarios and advance toward defining performance requirements, the feasibility analysis should be expanded to include at least two layers

2.4 Preliminary observations for simulation results Option 3 Track 2

From the WF in RAN4#115 the following observations were made:

- There is reasonable SGCS convergence when companies train an encoder-decoder pair using the mixed dataset and test using the mixed testset

  - Around 1% variation in SGCS; see results spreadsheet for more details

  - Preliminary observation; further results may be added

- It is possible for UE vendors to train an encoder based on a frozen test decoder that is functional

- When training “own” encoder with the frozen decoder and testing, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results from results presented so far

  - Around [+- 2%] variation in SGCS from the average; see results spreadsheet for more details

  - Preliminary observation; further results may be added

- Up to now, the same encoder structure is considered as used when creating the reference decoder by most companies; the impact of different encoder structure is not yet fully considered

2.5 Further simulation activity for option 3 track 2

From the WF in RAN4#115 the following agreement for simulation activity was reached:

- Definitions

  - Training Dataset 1 is the dataset used for training of the frozen decoder.

  - Training Dataset 2 is the dataset used for training of the new encoder based on the frozen decoder.

- For the existing results above, Training dataset 1 = training dataset 2 = mixed training dataset. Test dataset = mixed test dataset.

- Additional results 1 for RAN4#115:

  - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset)

  - For the new encoder: Training dataset 2 = company specific training dataset. Test Dataset = Mixed test dataset.

- Additional results 2 for RAN4#115:

  - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset)

  - For the new encoder: Training dataset 2 = company specific training dataset. Test dataset = company specific test dataset.

- Additional results 3 for RAN4#115: Further results using a lower complexity CNN or transformer encoder

  - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset)

  - For the new encoder: Training dataset 2 = mixed. Test Dataset = Mixed test dataset.

  - Encoder structure: As suggested in [R4-2505156] or company specific selection. Structure and complexity should be reported.

Additional results 1:

We performed separate training with our own Encoder and with the mixed dataset as the testing dataset the SGCS was 0.68.

Additional results 2:

Simulation results are shown in the table below:

Table 1: Additional results 2

Additional results 3:

The simulation results are obtained with separate training where an Encoder is trained with the frozen Samsung decoder or joint training. Simulation results are shown in the table below:

Table 3: Additional results 3

Observation 3: A lower complexity and a different AI/ML backbone Encoder network can inter-operate well with the frozen decoder trained with a high complexity encoder for Option 3 and Track2

2.6 Updated diagram feasibility analysis for Option 3

For the two-sided model, two key challenges arise:

- How can tests be designed to ensure they are feasible to implement across different test equipment and yield repeatable results?

- How can we ensure that the requirements and test setup are sufficiently related to real network conditions, so that the tests are meaningful in relation to actual network operation? (importance of having a robust test decoder and a rich latent space for effective real-world operation)

To meet these criteria, according to option 3, a test decoder is standardized. By doing so, a consistent, repeatable test can be implemented across various test equipment, ensuring that test results are both reliable and applicable to real-world scenarios.

Feasibility analysis: Once the test decoder is standardized and established, all vendors can create practical product encoders that leverage the knowledge of the test decoder and its latent space. Before the final adoption of a test decoder, companies can validate its effectiveness by confirming that they can indeed develop encoders based on the decoder

We provide an updated flowchart diagram (from RAN4#112) for the procedures to follow for testing the feasibility of the test decoder for Option 3:

Figure 10. Flowchart of procedures for feasibility analysis of Option 3 Test decoder

For aiding the performance alignment of the simulation results reported by each company, each company should list the training parameters that the encoder, decoder pair was trained with.  As a next step we could employ data sharing as shown in step 8.

Proposal 9: RAN4 to employ the procedures shown in the flowchart for studying the feasibility analysis of test decoder Option 3. (It captures both Track-1 and Track-2)

We further elaborate on the steps described in the flowchart. We need to agree on the criteria to determine if feasibility is met. A test decoder that delivers marginal throughput performance compared to e-type II under ideal conditions (unquantized training data, high SNR) should be considered not feasible. A test decoder with better performance should be considered for feasibility analysis (loopback to step 1). Another aspect of feasibility concerns the alignment of all the UE preferred encoder with the chosen test decoder (step 6).

Proposal 10: The following criteria should establish the feasibility of the test decoder:

- The test decoder (along with the encoder) should deliver reasonable throughput performance gain compared to e-type II in ideal conditions. RAN4 to extend the feasibility analysis to higher ranks and different payload sizes to determine consistency of performance improvement. FFS on how to quantify the acceptable performance gain. (performance-complexity tradeoff)

- All UE preferred encoders should be aligned (work well) with the chosen test decoder

Therefore, the feasibility should entail both a performance and alignment aspect.

We will also need to agree on how to determine the alignment aspect.

Proposal 11: The following principles should guide the establishment of the alignment aspect of the feasibility analysis (steps 6-7):

- A well-trained UE encoder with the same or higher complexity compared to the Ref encoder that the test decoder was trained with, should have at least similar performance with the reference performance.

- FFS on how to define a criterion for the similarity/dissimilarity of all companies' performance results with respect to the reference performance, considering UE encoder complexity as well.

2.7 Remaining Issues for Option 3

The following issues need to be addressed:

Issue 1: How many test decoders/encoders need to be captured (due to different channel conditions and configuration s)

Issue 2: For setting performance requirements (throughput based) we will need to have an Encoder/Decoder pair with at least 2 layers. We will need to investigate how to standardize such an decoder and what kind of simulations we need to perform

Issue 3: It would be hard to specify a testing dataset, since the real testing takes place with the real channel and practical channel estimation at UE and subsequent pre-processing to derive the required CSI at the input of the encoder which will be UE implementation specific

We have the following proposals:

Proposal 12: RAN4 to investigate and define the number and types of test decoder scenarios required to adequately evaluate CSI performance testing. These scenarios may need to account for a range of channel conditions (e.g., varying SINR, fading profiles) and deployment configurations (e.g., different numerologies, bandwidths, antenna configurations).

Proposal 13: To establish meaningful performance requirements for AI/ML-based CSI compression, it is proposed that RAN4 define a test decoder capable of supporting at least 2-layer reconstruction. This would enable evaluation of throughput and compression efficiency in realistic multi-layer MIMO scenarios, which are common in advanced deployments. RAN4 is encouraged to study the feasibility of such a decoder, including potential architectures, necessary simulation assumptions for feasibility analysis. If feasibility is established with Option3 there is no need to study another Option.

Proposal 14: RAN4 extend its ongoing work on standardizing a test decoder—used for evaluating the UE (Device Under Test)—to also include the specification of a test encoder for testing the gNB-side decoder. Defining a standardized test encoder would ensure consistent, fair, and repeatable testing of gNB implementations, in the context of AI/ML-based CSI compression and feedback. Furthermore, insights and methodologies developed during the specification of the test decoder can be leveraged to streamline the design of the test encoder.

## 3 Testability and interoperability issues for CSI compression Option 4

During RAN4#112bis the following agreement was reached:

Interested companies are invited to bring further proposals for the next steps for the feasibility study of Option 4(for any of the sub-options of Option 4)

In the discussion below we elaborate on our proposal for Option 4 and we provide a flow chart with the next steps.

Option 4 entails utilizing a reproducible test decoder for testing purposes, while the decoder itself is not explicitly detailed in the specification. The specification should include sufficient information to allow different test equipment (TE) vendors, and eventually network (NW) vendors, to design decoders that deliver consistent and reproducible test results.

The ultimate objective of both Option 3 and Option 4 is to establish a common and standardized decoder that facilitates reproducible and consistent testing for test equipment (TE). In Option 3, the decoder is explicitly defined, whereas in Option 4, a reference encoder or a dataset is specified , allowing the TE vendor to derive the decoder. The test decoder should also be interoperable across multi-vendor Encoders.

Observation 4: To achieve reproducible tests and guarantee interoperability, the specification will need to capture the following aspects:

- The latent space representation to be captured by the decoder

- An aggregate dataset from UE vendors, featuring UE-preferred encoders in the form of {V, c}, aiming to enrich the latent space and enhance interoperability.

For Option 4 test decoders, we require partially specified decoders to satisfy the principle of test repeatability and bounded variation across TE implementations.

Test repeatability should be guaranteed for Option 4 (bounded variations across various TE implementations). If the DUT passes or fails the test with one TE implementation, it should exhibit the same outcome with another TE implementation.

Based on the WF from RAN4 #111 [R4-2410571] and the ad-hoc meeting notes:

Option 4 for 2-sided model

Several companies brought proposal on how to further study/check the feasibility of option 4.

Issue 4-6: Option 4 for 2-sided model

- Proposals

  - Option 1: Qualcomm (R4-2407334)

- Option 4a-1 standardized dataset

  - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure.

  - Step 2: RAN4 uses this encoder/decoder pair and the generation procedure to generate a set of decoder input and output data and captures this dataset in the specification.

  - Step 3: RAN4 specifies a test decoder verification procedure based on the specified dataset.

- Option 4a-2 standardized aggregated dataset

  - Step 1: RAN4 achieves some agreements (e.g., part of but not all the parameters in the test decoder parameter table in the previous meeting WF[1]) for the test decoder.

  - Step 2: Interested companies can design their own encoder/decoder pairs based on the agreements to contribute the (decoder input, decoder output) dataset to RAN4

  - Step 3: RAN4 aggregates the datasets from all the contributing companies, and capture the aggregated dataset in the specification

  - Step 4: RAN4 specifies a test decoder verification procedure based on the specified dataset.

- Option 4b reference encoder/decoder pair

  - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure

  - Step 2: RAN4 capture the encoder/decoder as a reference encoder/decoder pair and the encoder input data generation procedure in the specification.

  - Step 3: RAN4 specifies a test decoder verification procedure based on the reference encoder.

- Option 2: Xiaomi (R4-2407847): Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Option 3: vivo(R4-2408294)

  - Option 4a: Reference encoder + test decoder model structure, and channel generation method are in the spec;

  - Option 4b: Test decoder model structure + reference encoder model structure, and dataset (PMI and corresponding channel) are in the spec;

  - Option 4 – Huawei (R4-2409003))

- Option 4a: Model structure is not specified in RAN4. Training dataset is specified, where each training sample consists of both the raw channel matric/precoding matrix and the bit stream forwarded to the test decoder.

- Option 4b: Model structure is specified in RAN4. Training dataset is not specified for verifying the encoder at DUT. The test decoder developed by TE vendor needs verification.

  - FFS: How to determine the test metric for test decoder developed by each TE vendor.

  - Option 5: Nokia (R4-2408659)

- Option 2 (Dataset based):

  - Option 2a: Freeze complete training data while leaving model architecture for implementation.

  - Option 2b: Freeze the important characteristics of training data, e.g., number of bits of latent message while leaving actual data samples and model architecture for implementation.

- Option 3: Freeze the important characteristics of training data, e.g., number of bits of latent message, and a backbone of model architecture while leaving actual data samples and architectural details for implementation.

  - Option 6: Ericsson (R4-2408492)

    - the latent space needs to be standardized

    - standardize a reference encoder in order to capture the latent space

  - Option 7: others

- Recommended WF

  - To be discussed

Likely multiple options need to be chosen/combined RAN4 should agree on a minimum set such that companies can continue the study.

We are elaborating on the steps described on the way forward from RAN#114 for option 4a and option 4b.

- The architecture of the test decoder is partially specified through some high-level parameters (to be agreed) while leaving room for vendor specific implementations

- Interested companies implement a test decoder according to the partially specified agreement

- The partially specified test decoder is trained from an aggregate data set of UE encoder latent space distributions (dataset is stored in a server). Multiple test decoders are trained from participating companies. The test decoders are stored in a database.

- Each UE vendor tests its own preferred Encoder with the test decoders stored in the database

Regarding the goal of Option 4 test decoder:

Test repeatability should be guaranteed for Option 4 (bounded variations across various TE implementations). If the DUT passes or fails the test with one TE implementation, it should exhibit the same outcome with another TE implementation.

The proposed procedures could determine the feasibility of the Option 4 test decoder goal since DUT => preferred UE encoder and {another TE implementation} => Test Decoder stored in database.

The flowchart of procedures is shown below:

Fig 11. Flowchart of procedures for feasibility analysis of Option 4a Test decoder.

Fig 12. Flowchart of procedures for feasibility analysis of Option 4b Test decoder.

Proposal 15: RAN4 to employ the detailed procedures shown in the flowchart for studying the feasibility analysis of test decoder Option 4a and Option 4b based on the aggregate dataset and database of test decoder models

- The architecture of the test decoder is partially specified through some high-level parameters (to be agreed) while leaving room for vendor specific implementations

- Interested companies implement a test decoder according to the partially specified agreement

- The partially specified test decoder is trained from an aggregate data set of UE encoder latent space distributions (dataset is stored in a server). Multiple test decoders are trained from participating companies. The test decoders are stored in a database.

- Each UE vendor tests its own preferred Encoder with the test decoders stored in the database

- Analysis of results to determine feasibility of test decoder

3.1 Simulation activity for option 4

Further simulation activity for option 4 was captured during RAN4#114bis

- All interested companies are requested to upload their “own” decoders to the server as soon as possible.

  - “Own” decoder means a decoder trained using the Samsung Frozen encoder and the mixed training dataset.

- Definitions

  - Dataset 1 = Dataset used for training the frozen encoder

  - Dataset 2 = Dataset used for training the own decoder (using the frozen encoder)

  - Dataset 3 = Dataset used for training the own encoder (using the own decoder)

- First results priority results for RAN4#115:

  - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset

  - Test dataset = Mixed test dataset

  - Provide results for

    - Your company’s “own” decoder and “own encoder”

    - Other companies uploaded “own” decoders and your company’s “own” decoder (this is a test for interoperability between encoders and test decoders)

- Second priority results for RAN4#115:

  - Dataset 1 = Dataset 2 = Mixed training dataset

  - Dataset 3 = Company own dataset

  - Test Dataset: Mixed Dataset, also consider own test dataset if interested.

  - Provide results for

    - Your company’s “own” decoder and “own encoder”

    - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders)

- Results for consideration of lower complexity encoder for RAN4#115:

  - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset

  - Test dataset = Mixed test dataset

  - Encoder: Lower complexity “own” encoder, examples are provided in [R4-2505156]

  - Provide results for

    - Your company’s “own” decoder and “own encoder”

    - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders)

- For a future meeting, a further consideration, for option 4b is to consider the frozen encoder with a company own dataset; i.e. Dataset 1 = Mixed training dataset, Dataset 2 = Dataset 3 = Company specific dataset.

Table 4: Simulation results for Option4

In our simulation results our own decoder was trained with the mixed dataset and the reference Samsung encoder. We used two architectures for own decoder: CNN based and transformer based. Own Encoder had the reference CNN architecture. Based on simulations we have the following observation:

Observation 5: The own-train encoder demonstrates stable performance across various test decoders trained on the mixed dataset. This consistency indicates that Option 4 may be feasible—assuming such robustness extends across diverse decoder implementations, reflecting flexibility in TE vendor designs, including CNNs, transformers, and other architectures.

## 4 Generalization/scalability aspects

The performance of generalization is of importance for a successful AI/ML feature deployment and the necessity and feasibility of defining requirements or test to verify the generalization of AI/ML has been studied. The content below from the TR discusses achieving minimum performance under certain conditions while ensuring that performance is not significantly degraded under other conditions Accordingly for the generalization verification aspects, the following contents are agreed to be captured in TR 38.843 [1]:

The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing: - details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - what the minimum level performance for each identified scenario and/or configuration is - what the significant degradation for other scenarios and/or configurations is It should also be considered that generalization and/or scalability related requirements for different scenarios/ configurations can be implicitly handled in the test case definition. As for the handling of generalization tests, the following option is considered as baseline: Signaling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test) - Specified UE configuration includes functionality and/or model ID if defined

Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use:

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc)

- Various antenna virtualization (TxRU mapping)

- Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz)

- Various outdoor/indoor UE distributions for UMa/Umi

- Various UE speeds.

Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use:

- Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers

- Various sizes of CSI feedback payloads

Proposal 16: Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use:

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc)

- Various antenna virtualization (TxRU mapping)

- Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz)

- Various outdoor/indoor UE distributions for UMa/Umi

- Various UE speeds.

Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use:

- Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers

- Various sizes of CSI feedback payloads

## 2.4 Monitoring Procedures/KPI

During the RAN1#117 meeting, the following agreements on model monitoring are agreed to be studied further

For UE side monitoring, UE can calculate the intermediate KPI between the ground-truth CSI and the recovery CSI (i.e., output of the CSI reconstruction model) indicated by NW. eT2-like high-resolution codebook for reporting format and SGCS for the type of intermediate KPI are likely used to deliver the recovery CSIs. This will create significant signaling overhead.

Instead, we could rely on the input CSI data distribution to detect any anomaly or data drift. Channel State Information (CSI) reflects the state of the wireless channel, which is influenced by the environment (e.g., movement of objects, user mobility) and the system configuration (e.g., antenna array changes, beamforming adjustments). Any significant change in the environment or configuration/scenario will inherently alter the statistical or semantic properties of the CSI samples. We aim at reducing the frequency of monitoring procedures unless there is an indication of an event change that would indicate that the AI/ML model has drifted or the input data distribution to the AI/ML model has drifted

Observation 6: Input CSI data distribution can be used to detect a monitoring event. By detecting monitoring events without ground truth CSI excessive overhead signaling can be avoided

By continuously monitoring how these properties evolve over time, sudden deviations or trends can be detected, signaling a monitoring event. Performing monitoring operations is expensive due to the necessity of signaling the ground truth and/or the assumption of the availability of the ground truth

Proposal 17: Investigate CSI similarity of input data distributions using Angular-Delay Channel Entropy ADCE as a statistical measure. This approach aims to reduce the reliance on more expensive monitoring procedures, such as those based on KPIs and ground truth availability, by initiating these procedures only when significant changes in data distribution are detected. The ADCE can be defined as ADCE=ijPijlog2Pij, where Pij=Hij2||H||F2, and Hij=FdHFaH is a virtual channel representation in angular/delay domain

By monitoring this metric (its computation is inexpensive) and by confirming similarity there is no need to employ the more expensive monitoring procedures (monitoring based on KPIs and availability of the ground truth). Only when the computation of ADCE indicates an event change the KPI based monitoring procedures will be initiated

Another advantage of ADCE is that it can be used as a Measure of Smoothness and Correlation: In the spatial-frequency domain, smoothness or correlation in CSI samples indicates how coherent or predictable the wireless channel is across space and frequency. A low ADCE value suggests a more predictable (smooth/correlated) channel, meaning power is concentrated in a smaller number of dominant components. A high ADCE value indicates a less predictable (less smooth, less correlated) channel, meaning power is distributed more evenly across components. Lower ADCE implies higher sparsity in the delay-beam domain, which simplifies the representation of CSI samples and enhances the efficiency of AI/ML-based compression methods.

Dynamic configuration can obtain an optimal tradeoff between signaling overhead (payload size), performance under various channels and noise conditions. By doing so the UE/NW could adaptively adjust the compression process, optimizing the balance between feedback overhead and accuracy in representing the channel state

Proposal 18: RAN4 to investigate defining metrics for Monitoring CSI Input Distribution to Trigger Monitoring Procedures and their Application to Dynamic CSI Compression

To facilitate network-side (NW) monitoring with minimal feedback overhead, we propose an approach to insert a small number of redundancy elements into the input CSI vector of the encoder at the UE. This allows the NW to use the reconstructed redundancy elements to assess the quality of the CSI after decompression. This method enables the NW to monitor the quality of each CSI feedback in real-time. These redundancy elements, which are additional elements of CSI samples added to the original CSI vector, serve as a reference for the NW to assess the quality of the CSI once it is decompressed. By embedding only, a small number of redundancy elements into the feedback, this approach introduces very little additional overhead, making it suitable for reducing the signaling overhead for monitoring procedures

Redundancy Elements in Input CSI where a subset of known values is embedded within the CSI vector before encoding at UE These elements are designed to be distinguishable and remain consistent across encoding and decoding processes. Fixed Reference Patterns: Predefined sequences (e.g., CSI patterns of small matrices) known to both the UE and NW. Derived Elements: Generated from portions of the CSI vector using known mathematical functions, such as low-dimensional projections.

Proposal 19: For Monitoring CSI AI/ML Models investigate embedding a subset of known values, referred to as redundancy elements, into the CSI vector at the UE before encoding. These elements act as reference points that can be tracked throughout the encoding and decoding processes. The approach is designed to facilitate the monitoring of AI/ML models by providing a measurable and consistent benchmark for evaluating model performance.

For CSI model monitoring identifying the root cause of degradation is important. We propose dividing the process into two stages.

The initial stage focuses on determining whether the AI/ML model is the source of performance degradation or if external factors are at play. This involves assessing the effectiveness of the CSI generation and reconstruction processes, particularly through analyzing the SGCS the encoder/decoder pair which represents nominal CSI feedback. If the first stage identifies the AI/ML model as a potential cause, the second stage proceeds to isolate the exact issue within the model, enabling a more targeted investigation.

Proposal 20: We propose a two-stage approach for CSI model monitoring, first distinguishing whether performance degradation stems from the AI/ML model or external factors and then isolating the specific issue within the model if identified as the cause.

We are also facing with the challenge of how to define requirements for Model Monitoring?  How to evaluate if the monitoring procedure is “Accurate”?

Current approach: Accuracy of the KPI Model monitoring metric: In R18 study, SGCS accuracy for model monitoring is computed by averaging the per-sample difference between the genie SGCS and calculated SGCS based on ground-truth reporting and SGCS estimation, i.e., ESGCSgenie-SGCSmeasured

However, this accuracy metric fails to reflect the performance of decision-making processes critical for monitoring. We would prefer to make use of a different approach: performance of decision-making processes that indicates the accuracy of predicting an event that indicates AI/ML model drift and performance degradation

Monitoring systems are not just about measuring accuracy; they are about identifying and reacting to anomalies, model degradation, or environmental changes.

Therefore, there is a need for Decision-Making Accuracy:

The primary goal of performance monitoring is to detect issues (like data drift, model degradation, hardware faults) and trigger appropriate actions (e.g., model re-training, re-configuration, AI/ML model switch or deactivation).

Thus, what's more important is the accuracy of monitoring decisions rather than just the numerical accuracy of SGCS measurements.

Performance monitoring in AI/ML-driven wireless networks is essential to detect issues such as data drift, model degradation, and hardware faults, and to trigger appropriate actions like model re-training, re-configuration, or deactivation. To achieve this, the system must ensure high event detection accuracy with low false positives and negatives, real-time monitoring, and multi-dimensional metrics. Proactive prediction capabilities, adaptive thresholds, and automated decision-making are critical to address issues before they impact performance. The implementation requires scalable and flexible solutions, including robust data collection, advanced detection algorithms, and predictive models. By meeting these requirements, the system will enhance reliability, maintain optimal performance, and ensure seamless adaptation to changing channel conditions.

Proposal 21: RAN4 to investigate a performance monitoring framework to detect issues such as data drift, model degradation, and AI/ML model aging, triggering actions like re-training, re-configuration, or model switching. The system will ensure high detection accuracy, real-time monitoring, and proactive predictions, enabling reliable adaptation to changing channel conditions while maintaining optimal performance.

Proposal 22: For a performance monitoring framework, event detection for data distribution drift and anomalous behavior is based on the output of a trained model that predicts performance deviations (the event). Consider the following cases as shown below:

(1) Indirect Event Detection: AI/ML model for SGCS EST is trained to predict SGCS from latent space c. Subsequently a detection logic can trigger event detection (2) Direct Event Detection is based on AI/ML model to directly predict an event

Fig 13. Performance monitoring framework for Event detection

There are couple of issues by using SGCS as an intermediate KPI for CSI compression. We list the issues below:

- SGCS Doesn't Capture Information or Capacity Loss Directly

SGCS only measures angular similarity between CSI matrices, ignoring magnitude differences that impact channel quality. Information loss due to quantization, feedback delay, or imperfect CSI compression is not explicitly reflected in SGCS. Capacity loss is a function of channel power and spatial correlation, but SGCS does not account for power scaling effects, which are crucial in MIMO systems.

- SGCS is Weakly Correlated with Throughput

Throughput depends on spectral efficiency, which is determined by the achievable rate formula. SGCS does not directly relate to the determinant or eigenvalues of H, making it an incomplete indicator of spectral efficiency. The impact of CSI errors on MCS selection and beamforming is non-trivial, making it hard to infer actual throughput loss from SGCS degradation.

- SGCS Does Not Capture CSI Feedback Imperfections Well

Compression artifacts: If a UE sends quantized CSI feedback, the distortion in signal space may not align with angular deviations captured by SGCS. Interference and noise effects: SGCS does not distinguish between CSI errors due to channel noise versus quantization/compression, leading to ambiguities in interpreting feedback accuracy.

Observation 7:

- SGCS Doesn't Capture Information or Capacity Loss Directly

SGCS only measures angular similarity between CSI matrices, ignoring magnitude differences that impact channel quality. Information loss due to quantization, feedback delay, or imperfect CSI compression is not explicitly reflected in SGCS. Capacity loss is a function of channel power and spatial correlation, but SGCS does not account for power scaling effects, which are crucial in MIMO systems.

- SGCS is Weakly Correlated with Throughput

Throughput depends on spectral efficiency, which is determined by the achievable rate formula. SGCS does not directly relate to the determinant or eigenvalues of H, making it an incomplete indicator of spectral efficiency. The impact of CSI errors on MCS selection and beamforming is non-trivial, making it hard to infer actual throughput loss from SGCS degradation.

- SGCS Does Not Capture CSI Feedback Imperfections Well

Compression artifacts: If a UE sends quantized CSI feedback, the distortion in signal space may not align with angular deviations captured by SGCS. Interference and noise effects: SGCS does not distinguish between CSI errors due to channel noise versus quantization/compression, leading to ambiguities in interpreting feedback accuracy.

We aim at establishing a KPI metric which addresses the issues discussed with SGCS and it provides a close relation to throughput and spectral efficiency. For this purpose, lets denote a general precoder matrix P. Under ideal conditions (no compression) the ideal CSI would be denoted as V. Under AI/ML model compression and other artifacts the reconstructed CSI is Vc, where c denotes corrupted CSI. Our goal is to derive an approximate formula that describes the capacity loss under two different effective channels: HP|P=V  and : HP|P=Vc .  Assuming we have a system with rank L. Then the system capacity is given by the formula: C=k=1Llog21+SINRk, where SINRk takes into account the interstream interference and it depends on the MIMO receiver.

Proposal 23:  Consider a new KPI for CSI compression that could offer an indication of spectral efficiency loss due to misalignment of precoders. It can be conceptually described as KPI:Closs=C​P=V-C​P=Vc where Vc is the corrupted reconstructed CSI

Proposal 24: Investigate the benefits of defining a new KPI for CSI compression that is linked to capacity loss. For an SVD receiver this metric is  approximated as Closs,SVD≈k=1Llog2ρρIkλk/L+1ρIk+vkHvck2, where ρIk=j=1,j≠kLvkHvck2  is defined as the inter-stream interference to layer k caused by the mismatch between Vc and V.  Also ρ is the SNR and λk is the eigenvalue of the channel H'H

For rank 1: the proposed metric becomes Closs=-log2v1Hvc1, while SGCS=v1Hvc122/vc122.  Under ideal conditions the term ρIk = 0 due to the orthogonality. But under imperfection this term contributes to the denominator and the capacity decreases.

Another advantage of the proposed KPI is that it could be used as an compensation factor to determine the CQI.

In legacy CSI report using PMI codebook, since the PMI construction algorithm is not complicated, it is feasible for UE to construct the PMI for CQI calculation. In this way, the precoder used in CQI calculation is same as the precoding vectors reported to the gNB. However, in AI/ML based CSI compression and feedback, the CSI reconstruction model and CSI generation model are normally run at gNB side and UE side separately, it may or may not be feasible for UE to follow the legacy principle. In RAN1 the following discussions and options are available:

Proposal 25: Investigate the feasibility of using the spectral efficiency KPI to determine the CQI adjustment for option 1b.

# Conclusions

In conclusion the following observations and proposals were discussed in this contribution:

Observation 1: The feasibility of Option 3 Test Decoder under Track 1 cannot be established for all the chosen test decoders due to the performance dependency on the testing dataset. That is, in the absence of an agreed training dataset, if each company trains each own encoder with own training dataset, then the testing performance at TE with the trained DUT will be testing-dataset dependent. DUT will perform differently depending on the testing dataset. From simulation results, there is a statistical difference between companies’ datasets.

Observation 2: Performance of Own trained (mixed dataset) Encoder with Mixed Dataset trained Decoder is more consistent across various companies testing datasets. However, the performance on testing datasets that don’t belong to the training dataset (QC, CATT) is significantly degraded mirroring the issues observed in track1.

Observation 3: A lower complexity and a different AI/ML backbone Encoder network can inter-operate well with the frozen decoder trained with a high complexity encoder for Option 3 and Track2

Observation 4: To achieve reproducible tests and guarantee interoperability, the specification will need to capture the following aspects:

- The latent space representation to be captured by the decoder

- An aggregate dataset from UE vendors, featuring UE-preferred encoders in the form of {V, c}, aiming to enrich the latent space and enhance interoperability.

Observation 5: The own-train encoder demonstrates stable performance across various test decoders trained on the mixed dataset. This consistency indicates that Option 4 may be feasible—assuming such robustness extends across diverse decoder implementations, reflecting flexibility in TE vendor designs, including CNNs, transformers, and other architectures.

Observation 6: Input CSI data distribution can be used to detect a monitoring event. By detecting monitoring events without ground truth CSI excessive overhead signaling can be avoided

Observation 7:

- SGCS Doesn't Capture Information or Capacity Loss Directly

SGCS only measures angular similarity between CSI matrices, ignoring magnitude differences that impact channel quality. Information loss due to quantization, feedback delay, or imperfect CSI compression is not explicitly reflected in SGCS. Capacity loss is a function of channel power and spatial correlation, but SGCS does not account for power scaling effects, which are crucial in MIMO systems.

- SGCS is Weakly Correlated with Throughput

Throughput depends on spectral efficiency, which is determined by the achievable rate formula. SGCS does not directly relate to the determinant or eigenvalues of H, making it an incomplete indicator of spectral efficiency. The impact of CSI errors on MCS selection and beamforming is non-trivial, making it hard to infer actual throughput loss from SGCS degradation.

- SGCS Does Not Capture CSI Feedback Imperfections Well

Compression artifacts: If a UE sends quantized CSI feedback, the distortion in signal space may not align with angular deviations captured by SGCS. Interference and noise effects: SGCS does not distinguish between CSI errors due to channel noise versus quantization/compression, leading to ambiguities in interpreting feedback accuracy.

Proposal 1: To establish the feasibility of the Test Decoder under Option 3 and Track1, each company should train its own encoder using the fully specified Test Decoder and its own training data. The trained encoder should then be tested with the Test Decoder using datasets from different companies and demonstrate performance consistency

Proposal 2: Investigate a Channel State Information (CSI) metric that provides a statistical measure of how well CSI can be compressed, directly translating to SGCS performance. Using this approach, a dissimilarity metric between CSI training and testing datasets can be derived to predict or interpret SGCS results. One such metric could be Angular-Delay Channel Entropy (ADCE), which quantifies the uncertainty in CSI’s spectral distribution and reflects its compressibility. The ADCE can be defined as ADCE=ijPijlog2Pij, where Pij=Hij2||H||F2, and Hij=FdHFaH is a virtual channel representation in angular/delay domain which separates multipath components into resolvable delay taps and spatial angles. ADCE captures the sparsity or randomness of these components.

Proposal 3: To determine the initial feasibility of test decoder under Option 3 and Track 2, each company to train Own Encoder with Test decoder with the mixed dataset. Subsequently each company tests the Enc/Dec pair with the mixed dataset to ensure performance consistency across all companies.

Proposal 4: RAN4 to investigate potential performance loss due to the dataset distribution mismatch (field data distributions) when one or both parts of the AI/ML models are specified with a synthetic dataset.

Proposal 5: RAN4 to investigate the similarity between Option 3 and Inter-vendor collaboration direction C (Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1) from RAN1 study

Proposal 6: RAN4 to investigate how many test decoders need to be specified under Option 3. Identify the granularity of scenarios/deployments that will dictate a new test decoder.

Proposal 7: To complete feasibility analysis under Option 3, the following question should be answered:

Performance Consistency Across UE Encoder Implementations: Can the performance be maintained across different UE encoder implementations? Specifically, if the UE encoder uses a different AI/ML backbone (e.g., a Transformer-based architecture), does the system still achieve comparable results?

Proposal 8: To align with real-world deployment scenarios and advance toward defining performance requirements, the feasibility analysis should be expanded to include at least two layers

Proposal 9: RAN4 to employ the procedures shown in the flowchart for studying the feasibility analysis of test decoder Option 3. (It captures both Track-1 and Track-2)

Proposal 10: The following criteria should establish the feasibility of the test decoder:

- The test decoder (along with the encoder) should deliver reasonable throughput performance gain compared to e-type II in ideal conditions. RAN4 to extend the feasibility analysis to higher ranks and different payload sizes to determine consistency of performance improvement. FFS on how to quantify the acceptable performance gain. (performance-complexity tradeoff)

- All UE preferred encoders should be aligned (work well) with the chosen test decoder

Proposal 11: The following principles should guide the establishment of the alignment aspect of the feasibility analysis (steps 6-7):

- A well-trained UE encoder with the same or higher complexity compared to the Ref encoder that the test decoder was trained with, should have at least similar performance with the reference performance.

- FFS on how to define a criterion for the similarity/dissimilarity of all companies' performance results with respect to the reference performance, considering UE encoder complexity as well.

Proposal 12: RAN4 to investigate and define the number and types of test decoder scenarios required to adequately evaluate CSI performance testing. These scenarios may need to account for a range of channel conditions (e.g., varying SINR, fading profiles) and deployment configurations (e.g., different numerologies, bandwidths, antenna configurations).

Proposal 13: To establish meaningful performance requirements for AI/ML-based CSI compression, it is proposed that RAN4 define a test decoder capable of supporting at least 2-layer reconstruction. This would enable evaluation of throughput and compression efficiency in realistic multi-layer MIMO scenarios, which are common in advanced deployments. RAN4 is encouraged to study the feasibility of such a decoder, including potential architectures, necessary simulation assumptions for feasibility analysis. If feasibility is established with Option3 there is no need to study another Option.

Proposal 14: RAN4 extend its ongoing work on standardizing a test decoder—used for evaluating the UE (Device Under Test)—to also include the specification of a test encoder for testing the gNB-side decoder. Defining a standardized test encoder would ensure consistent, fair, and repeatable testing of gNB implementations, in the context of AI/ML-based CSI compression and feedback. Furthermore, insights and methodologies developed during the specification of the test decoder can be leveraged to streamline the design of the test encoder.

Proposal 15: RAN4 to employ the detailed procedures shown in the flowchart for studying the feasibility analysis of test decoder Option 4a and Option 4b based on the aggregate dataset and database of test decoder models

- The architecture of the test decoder is partially specified through some high-level parameters (to be agreed) while leaving room for vendor specific implementations

- Interested companies implement a test decoder according to the partially specified agreement

- The partially specified test decoder is trained from an aggregate data set of UE encoder latent space distributions (dataset is stored in a server). Multiple test decoders are trained from participating companies. The test decoders are stored in a database.

- Each UE vendor tests its own preferred Encoder with the test decoders stored in the database

- Analysis of results to determine feasibility of test decoder

Proposal 16: Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use:

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc)

- Various antenna virtualization (TxRU mapping)

- Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz)

- Various outdoor/indoor UE distributions for UMa/Umi

- Various UE speeds.

Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use:

- Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers

- Various sizes of CSI feedback payloads

Proposal 17: Investigate CSI similarity of input data distributions using Angular-Delay Channel Entropy (ADCE), as a statistical measure. This approach aims to reduce the reliance on more expensive monitoring procedures, such as those based on KPIs and ground truth availability, by initiating these procedures only when significant changes in data distribution are detected. The ADCE can be defined as ADCE=ijPijlog2Pij, where Pij=Hij2||H||F2, and Hij=FdHFaH is a virtual channel representation in angular/delay domain

Proposal 18: RAN4 to investigate defining metrics for Monitoring CSI Input Distribution to Trigger Monitoring Procedures and their Application to Dynamic CSI Compression

Proposal 19: For Monitoring CSI AI/ML Models investigate embedding a subset of known values, referred to as redundancy elements, into the CSI vector at the UE before encoding. These elements act as reference points that can be tracked throughout the encoding and decoding processes. The approach is designed to facilitate the monitoring of AI/ML models by providing a measurable and consistent benchmark for evaluating model performance.

Proposal 20: We propose a two-stage approach for CSI model monitoring, first distinguishing whether performance degradation stems from the AI/ML model or external factors, and then isolating the specific issue within the model if identified as the cause.

Proposal 21: RAN4 to investigate a performance monitoring framework to detect issues such as data drift, model degradation, and AI/ML model aging, triggering actions like re-training, re-configuration, or model switching. The system will ensure high detection accuracy, real-time monitoring, and proactive predictions, enabling reliable adaptation to changing channel conditions while maintaining optimal performance.

Proposal 22: For a performance monitoring framework, event detection for data distribution drift and anomalous behavior is based on the output of a trained model that predicts performance deviations (the event). Consider the following cases as shown below:

(1) Indirect Event Detection: AI/ML model for SGCS EST is trained to predict SGCS from latent space c. Subsequently a detection logic can trigger event detection (2) Direct Event Detection is based on AI/ML model to directly predict an event

Proposal 23:  Consider a new KPI for CSI compression that could offer an indication of spectral efficiency loss due to misalignment of precoders. It can be conceptually described as KPI:Closs=C​P=V-C​P=Vc where Vc is the corrupted reconstructed CSI

Proposal 24: Investigate the benefits of defining a new KPI for CSI compression that is linked to capacity loss. For an SVD receiver this metric is  approximated as Closs,SVD≈k=1Llog2ρρIkλk/L+1ρIk+vkHvck2, where ρIk=j=1,j≠kLvkHvck2  is defined as the inter-stream interference to layer k caused by the mismatch between Vc and V.  Also ρ is the SNR and λk is the eigenvalue of the channel H'H

For rank 1: the proposed metric becomes Closs=-log2v1Hvc1, while SGCS=v1Hvc122/vc122.  Under ideal conditions the term ρIk = 0 due to the orthogonality. But under imperfection this term contributes to the denominator and the capacity decreases.

Proposal 25: Investigate the feasibility of using the spectral efficiency KPI to determine the CQI adjustment for option 1b.

# Reference

[1] RP-242399 - Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[2] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”