3GPP TSG-RAN WG4 Meeting #115	R4-2505954

Malta, 19-23 May, 2025

Source: 	Qualcomm Incorporated

Title: 	Views on SCM for UE Demodulation

Agenda item:	 7.16.3

Document for:	Agreement

- Introduction

In the previous RAN4 Meeting #114-bis, we contributed to the SCM Study Item sharing considerations on the Study Item on the use spatial channel model for demodulation performance requirements.

According to the outcome of the discussion during the meeting and based on the approved Way Forward [2], we provide in this contribution further observations as part of the ongoing study.

# Views on CDL modeling

## Angular Scaling approach

In previous RAN4 meetings, including the last #114-bis, the interpretation of the angular scaling procedure has been subject of multiple discussions, during and in between meetings.

The discussions regarded for example whether to implement the scaling per-ray or per cluster, how to apply wrapping, and how to interprete the details of the formulation currently included in 3GPP TR 38.901, which can be open to different interpretations.

In the meanwhile it is our understanding that RAN1 is working on a reformulation of the angular scaling procedure to be implemented in a new release of 3GPP TR 38.901, which will clarify the application of this procedure, including proper application of wrapping.

To that end, we believe that it would be the best course of action for RAN4 in the scope of the current Work Item to proceed with the alignment and comparison efforts based on the agreements reached so far on the CDL profile tables.

Proposal 1: RAN4 to continue the work in this SI using the tables included in Annex A in this contribution for alignment and comparison of CDL Option A4, as per offline alignment regarding the application of the angular scaling procedure.

At the same time, RAN4 should agree to reflect in the TR the expected update of the angular procedure, and to generate new CDL tables using the procedures agreed so far including the updated procedure.

Proposal 2: RAN4 should capture the updated angular scaling procedure expected from the RAN1 WI on TR 38.901 and reference it in TR 38.753, recommending it for future use.

RAN4 should also further discuss how to address the CDL tables used for alignment after the update to the angular scaling procedure is used to the new ones that will once generated with the new angular scaling procedure, if different, will replace of be added alongside the tables used  will be captured in a new CDL profiles or it will replace the

Proposal 3: RAN4 should discuss how to address the CDL tables used for alignment and comparison in future releases of TR 38.753, if the updated procedure for angular scaling results in differences to the outcome. One option could be to deprecate CDL profiles based on outdated procedure, or different CDL profiles could be captured in the TR with an identification to which Release of TR 38.901 they are based upon.

## Alignment with TR 38.901 in the TR 38.753

This section deals with the way forward regarding how to capture the procedures, and related explanations, contained in 3GPP TR 38.901 which are related to CDL channel model implementations which are found to be necessary to the content of 3GPP TR 38.753 in order to execute the steps that RAN4 has agreed in this SI for the derivation of the CDL Profiles for UE Demodulation.

We believe that the most reliable and future-proof implementation is to introduce in the body of 3GPP TR 38.753 exclusively references to the relevant section, subsection, or equation from a target release of 3GPP TR 38.901, without any duplication of the content. Additional text can be included in 3GPP TR 38.753 to explain any change on top of the referenced procedure, or to further detail the choice of parameters, or the implications.

Proposal 4: RAN4 to introduce in the body of 3GPP TR 38.753 exclusively references to the relevant section, subsection, or equation from a target release of 3GPP TR 38.901, without any duplication of the content. Additional text can be included in 3GPP TR 38.753 to explain any change on top of the referenced procedure, or to further detail the choice of parameters, or the implications.

It is our view that this procedure will allow TR 38.753 to both keep consistency regardless of future changes in TR 38.901, and to easily implement future incoming changes when necessary.

- Conclusions

Proposal 1: RAN4 to continue the work in this SI using the tables included in Annex A in this contribution for alignment and comparison of CDL Option A4, as per offline alignment regarding the application of the angular scaling procedure.

Proposal 2: RAN4 should capture the updated angular scaling procedure expected from the RAN1 WI on TR 38.901 and reference it in TR 38.753, recommending it for future use.

Proposal 3: RAN4 should discuss how to address the CDL tables used for alignment and comparison in future releases of TR 38.753, if the updated procedure for angular scaling results in differences to the outcome. One option could be to deprecate CDL profiles based on outdated procedure, or different CDL profiles could be captured in the TR with an identification to which Release of TR 38.901 they are based upon.

Proposal 4: RAN4 to introduce in the body of 3GPP TR 38.753 exclusively references to the relevant section, subsection, or equation from a target release of 3GPP TR 38.901, without any duplication of the content. Additional text can be included in 3GPP TR 38.753 to explain any change on top of the referenced procedure, or to further detail the choice of parameters, or the implications.

- References

[1] RP-241610, “Study on spatial channel model for demodulation performance requirements”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17th – 20th June, 2024;

[2] R4-2504756, “Way Forward for [114bis][320] NR_SCM”, 3GPP TSG-RAN WG4 Meeting #114-bis

# ANNEX A: CDL-C Profiles Used for Option A4, AAV(1,1)

| CLID | Delay, nsec | Power, dB | AoD, deg | AoA, deg | ZoD, deg | ZoA, deg |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -97 | 96.8 | 98.1 |
| 2 | 132 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 138 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 402 | 0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 416 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 502 | -10.1 | -50.5 | 63.4 | 94.5 | 56.6 |
| 7 | 518 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 776 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 826 | -9.8 | -50.4 | -63.5 | 105.2 | 50.1 |
| 10 | 1372 | -11.7 | -60.2 | 79 | 91.7 | 51.7 |
| 11 | 1712 | -16.2 | 64.6 | 26.3 | 105.2 | 121 |
| 12 | 2906 | -16.9 | 55 | -1.6 | 91.8 | 61.6 |
|  | C-ZSD deg | C-ZSA deg | C-ASD deg | C-ASA deg | XPR in [dB] |  |
|  | 3.6 | 12.2 | 1.4 | 15.6 | 7 |  |