3GPP TSG-RAN4 Meeting #115	R4-2505958

Malta, MT , 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2771 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-1 - Correction of A-MPR tables for band n68 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_band_n68-Core |  |  |  |  |  | Date: |  |  | 2025-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correct A-MPR tables for NS_26 and NS_36 for band n68 |  |  |  |  |  |  |  |  |
| Summary of change: |  | RBend changed to RBstart. Also 1 MHz terminology added to correct units |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NS_26 and NS_36 tables would not specify the A-MPR regions correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.3.34, 6.2.3.35 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.1	UE maximum output power

<Start of the change>

#### 6.2.3.34	A-MPR for NS_26

Table 6.2.3.34-1: A-MPR regions for "NS_26"

| Channel Bandwidth, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- |
|  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | ≤ 2.16 | ≥ 1 | A1 |
| 15 MHz | ≤ 4.5 | ≥ 1 | A2 |

Table 6.2.3.34-2: A-MPR for NS_26

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 3 | 3.5 |
|  | QPSK | 3.5 | 3.5 |
|  | 16 QAM | 4 | 4 |
|  | 64 QAM | 4.5 | 4.5 |
|  | 256 QAM | 5 | 5 |
| CP-OFDM | QPSK | 5 | 5 |
|  | 16 QAM | 5 | 5 |
|  | 64 QAM | 5 | 5 |
|  | 256 QAM | 6.5 | 6.5 |

<End of the change>

<Start of the change>

#### 6.2.3.35	A-MPR for NS_36

Table 6.2.3.35-1: A-MPR regions for "NS_36"

| Channel Bandwidth, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- |
|  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | ≤ 1.08 | >0 | A1 |
|  | ≥ 1.08 and ≤ 2.7 | ≥ 1.8 | A2 |
| 10 MHz | ≤2.16 | >0 | A3 |
|  | ≥ 2.34 and ≤3.78 | ≥ (12*SCS*RB_start/1MHz) + 1.62 | A4 |
|  | ≥ 2.34 and ≤ 4.32 | ≥ (0.5*12*SCS*RB_start/1MHz) + 0.63 and < (12*SCS*RB_start/1MHz) + 1.62 | A5 |
|  | >4.32 | ≥ 7.2 -(12*SCS*RB_start/1MHz) | A6 |
| 15 MHz | ≤ 4.14 | >0 | A7 |
|  | > 4.14 and ≤ 6.12 | ≥ (12*SCS*RB_start/1MHz) + 2.16 | A8 |
|  | > 4.14 and ≤ 7.2 | ≥ (0.5*12*SCS*RB_start/1MHz) + 0.63 and < (12*SCS*RB_start/1MHz) + 2.16 | A9 |
|  | > 7.2 | ≥ 11.0 -(12*SCS*RB_start/1MHz) | A10 |

Table 6.2.3.35-2: A-MPR for NS_36

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer /Inner | Outer/ Inner | Outer/ Inner | Outer/I nner | Outer/ Inner | Outer/I nner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM | PI/2 BPSK | 8 | 4 | 13 | 10 | 4.5 | 4.5 | 12.5 | 10.5 | 5 | 5 |
|  | QPSK | 8 | 4.5 | 13 | 10 | 5.5 | 5.5 | 12.5 | 10.5 | 6 | 6 |
|  | 16 QAM | 8.5 | 4.5 | 13 | 10.5 | 7 | 6.5 | 12.5 | 11 | 7.5 | 7 |
|  | 64 QAM | 8.5 | 4.5 | 13 | 10.5 | 7.5 | 7 | 12.5 | 11 | 7.5 | 7.5 |
|  | 256 QAM | 8.5 | 5 | 13 | 10.5 | 8 | 7.5 | 12.5 | 11 | 7.5 | 7.5 |
| CP-OFDM | QPSK | 9 | 5 | 13.5 | 12 | 7 | 5.5 | 13.5 | 12 | 7.5 | 6 |
|  | 16 QAM | 9 | 5 | 13.5 | 12 | 7 | 5.5 | 13.5 | 12 | 7.5 | 6 |
|  | 64 QAM | 9 | 5 | 13.5 | 12 | 7 | 5.5 | 13.5 | 12 | 7.5 | 6 |
|  | 256 QAM | 9 | 6.5 | 13.5 | 12 | 8 | 6 | 13.5 | 12 | 7.5 | 6 |

<End of the change>