3GPP TSG-RAN WG4 Meeting #115	R4-2505959

Malta, MT, 19th – 23rd May 2025

Source:	Qualcomm Incorporated

Title:	3 Tx NR specifications

Agenda item:	7.24.3

Document for:	Approval

# 1.	Introduction

In RAN4#114-bis most issues relating to 3 Tx were finalized. We think that there is one further issue regarding PCMAX,c tolerance for 3Tx which is still unresolved and should be discussed.  In this paper we present further views on this remaining unresolved issue.

# 2. 	Discussion

Currently, in TS38.101-1 [1] the PCMAX,c tolerance for 2Tx and 4Tx has been specified. However, the values for 3Tx remains to be defined. For reference the PCMAX,c values for 2Tx and 4Tx are given below:

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

Table 1- PCMAX,c  for 2Tx

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 24.5 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 23.5 ≤ PCMAX,c < 24.5 | 5.0 | 2.0 |
| 22.5 ≤ PCMAX,c < 23.5 | 5.0 | 3.0 |
| 21.5 ≤ PCMAX,c < 22.5 | 5.0 | 4.0 |
| 18 ≤ PCMAX,c < 21.5 | 5.0 |  |
| 13 ≤ PCMAX,c < 18 | 6.0 |  |
| -40 ≤ PCMAX,c < 13 | 7.0 |  |

Table 2- PCMAX,c  for 4Tx

For 3Tx we think that it makes sense to use PCMAX,c  steps that are in between the values for 2Tx and 4Tx. Therefore, we propose using the PCMAX,c   steps given below:

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23.5 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 22.5 ≤ PCMAX,c < 23.5 | 5.0 | 2.0 |
| 21.5 ≤ PCMAX,c < 22.5 | 5.0 | 3.0 |
| 20.5 ≤ PCMAX,c < 21.5 | 5.0 | 4.0 |
| 17 ≤ PCMAX,c < 20.5 | 5.0 |  |
| 12 ≤ PCMAX,c < 17 | 6.0 |  |
| -40 ≤ PCMAX,c < 12 | 7.0 |  |

Table 2- PCMAX,c  for 3Tx

.

Proposal 1: Use the PCMAX,c  table for 3Tx as shown below:

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23.5 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 22.5 ≤ PCMAX,c < 23.5 | 5.0 | 2.0 |
| 21.5 ≤ PCMAX,c < 22.5 | 5.0 | 3.0 |
| 20.5 ≤ PCMAX,c < 21.5 | 5.0 | 4.0 |
| 17 ≤ PCMAX,c < 20.5 | 5.0 |  |
| 12 ≤ PCMAX,c < 17 | 6.0 |  |
| -40 ≤ PCMAX,c < 12 | 7.0 |  |

# Conclusion

In this paper we propose a PCMAX,c   power vs tolerance table  for 3Tx:

Proposal 1: Use the PCMAX,c  table for 3Tx as shown below:

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23.5 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 22.5 ≤ PCMAX,c < 23.5 | 5.0 | 2.0 |
| 21.5 ≤ PCMAX,c < 22.5 | 5.0 | 3.0 |
| 20.5 ≤ PCMAX,c < 21.5 | 5.0 | 4.0 |
| 17 ≤ PCMAX,c < 20.5 | 5.0 |  |
| 12 ≤ PCMAX,c < 17 | 6.0 |  |
| -40 ≤ PCMAX,c < 12 | 7.0 |  |

# References

[1] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V19.1.0