3GPP TSG-RAN WG4 Meeting #115	R4-2505963

Malta, MT, 19th – 23rd May, 2025

Source:	Qualcomm Incorporated

Title:	MPR and A-MPR for Intra-band contiguous UL CA for PC1.5

Agenda item:	7.1.2.1.2

Document for:	Approval

# 1.	Introduction

For this WI companies were requested to measure/simulate MPR/AMPR numbers for several intra-band contiguous and non-contiguous scenarios. In RAN4#114-bis MPRs for contiguous CCs with contiguous RBs were agreed and tentative MPR values for contiguous CCs with non-contiguous RBs were agreed[1].  In this paper we present further views on the MPR/A-MPR for contiguous CCs with contiguous and non-contiguous RBs.

# 2. 	Discussion

In RAN4#114-bis a WF was created that stated the following [1]:

## 1.2 MPR values for intra-band contiguous ULCA with non-contiguous RB allocation (with TxD)

  - The MPR values for PC3(1Tx) and PC2(2Tx) listed in the below table is just for reference

| Modulation |  | PC3 (1Tx) |  |  | PC2 (2Tx) |  |  | PC1.5 (2Tx) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | Handheld |  |  | Handheld |  |  |
|  |  | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 6 | 11.5 | 41 | 8.5 | 14.5 | [4.5-5] | [9-9.5] | [15.5-17] |
|  | QPSK | 2.5 | 6 | 11.5 | 41 | 8.5 | 14.5 | [4.5-5] | [9-9.5] | [15.5-17] |
|  | 16QAM | 3 | 6 | 11.5 | 41 | 8.5 | 14.5 | [4.5-5] | [9-9.5] | [15.5-17] |
|  | 64QAM | 5 | 6 | 11.5 | 6 | 8.5 | 14.5 | [6.5-7] | [9-9.5] | [15.5-17] |
|  | 256QAM | 6.5 | 6.5 | 11.5 | 7.5 | 8.5 | 14.5 | [8-8.5] | [9-9.5] | [15.5-17] |
| CP-OFDM | QPSK | 3.5 | 7 | 12 | 4.51 | 9 | 15.5 | [5-5.5] | [9.5-10] | [16.5-17] |
|  | 16QAM | 3.5 | 7 | 12 | 4.51 | 9 | 15.5 | [5-5.5] | [9.5-10] | [16.5-17] |
|  | 64QAM | 5 | 7 | 12 | 6 | 9 | 15.5 | [6.5-7] | [9.5-10] | [16.5-17] |
|  | 256QAM | 7.5 | 7.5 | 12 | 8.5 | 9 | 15.5 | [9-9.5] | [9.5-10] | [16.5-17] |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |  |  |  |

Agreement:

- Further discussion in upcoming meetings based on the tentative values in the above table.

- The MPR contributions from companies is captured in the spreadsheet below:

Based on our latest measurements we suggest the following MPRs for PC1.5 contiguous CC with non-contiguous RBs

| Modulation |  | PC1.5 (2Tx) |  |  | PC1.5 (2Tx) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | FWA |  |  |
|  |  | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 |
| DFT-s-OFDM | PI/2 BPSK | 5 | 9.5 | 16 | 4.5 | 9 | 15.5 |
|  | QPSK | 5 | 9.5 | 16 | 4.5 | 9 | 15.5 |
|  | 16QAM | 5 | 9.5 | 16 | 4.5 | 9 | 15.5 |
|  | 64QAM | 7 | 9.5 | 16 | 6.5 | 9 | 15.5 |
|  | 256QAM | 8.5 | 9.5 | 16 | 8 | 9 | 15.5 |
| CP-OFDM | QPSK | 5.5 | 10 | 16.5 | 5 | 9.5 | 16.0 |
|  | 16QAM | 5.5 | 10 | 16.5 | 5 | 9.5 | 16.0 |
|  | 64QAM | 7 | 10 | 16.5 | 6.5 | 9.5 | 16.0 |
|  | 256QAM | 9.5 | 10 | 16.5 | 9 | 9.5 | 16.0 |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |

Table1: MPRs for contiguous CC/non-contiguous RBs for PC1.5 with 2Tx

Proposal 1: For contiguous CC/non-contiguous RBs adopt the following MPR for handheld and FWA devices

| Modulation |  | PC1.5 (2Tx) |  |  | PC1.5 (2Tx) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | FWA |  |  |
|  |  | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 |
| DFT-s-OFDM | PI/2 BPSK | 5 | 9.5 | 16 | 4.5 | 9 | 15.5 |
|  | QPSK | 5 | 9.5 | 16 | 4.5 | 9 | 15.5 |
|  | 16QAM | 5 | 9.5 | 16 | 4.5 | 9 | 15.5 |
|  | 64QAM | 7 | 9.5 | 16 | 6.5 | 9 | 15.5 |
|  | 256QAM | 8.5 | 9.5 | 16 | 8 | 9 | 15.5 |
| CP-OFDM | QPSK | 5.5 | 10 | 16.5 | 5 | 9.5 | 16.0 |
|  | 16QAM | 5.5 | 10 | 16.5 | 5 | 9.5 | 16.0 |
|  | 64QAM | 7 | 10 | 16.5 | 6.5 | 9.5 | 16.0 |
|  | 256QAM | 9.5 | 10 | 16.5 | 9 | 9.5 | 16.0 |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |

We also performed A-MPR measurements for n41 CA_NS_04 for contiguous CC/contiguous RBs. Based on the measured results it was seen that for cases where RBstart ≤ 0.33*BWchannel_CA/0.18MHz the A-MPR required for PC1.5 was 1.5dB higher than for PC3 and PC2 for UE handhelds and 1.5dB higher than for PC3 and PC2 for FWA devices. Therefore, it is recommended to adopt the following wording for PC1.5 in section 6.2A.3.1.1.1 of TS38.101-1 [2].

###### 6.2A.3.1.1.1	A-MPR for CA_NS_04

6.2A.3.1.1.1.1	Contiguous allocations

For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR

For all modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

if the RB allocation is an inner allocation as defined in clause 6.2A.2.1,  then A-MPR = MPR

Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc).

if the RB allocation is an outer allocation as defined in clause 6.2A.2.1,

then A-MPR = MPR+1.5dB for BW Class B A-MPR = MPR for BW class C for PC3 and PC2 and A-MPR=MPRHH+1.5dB for BW class C for PC1.5 UE handheld devices and A-MPR=MPRFWA+1.5dB for BW class C for PC1.5 FWA devices.

Where

-	MPR is the MPR as defined in Table 6.2A.2.1-1, Table 6.2A.2.1-1a and Table 6.2A.2.1-1b for PC3 and PC2 respectively and the respective CA bandwidth class

-	 AMPRcc is defined as the PC3_A2, PC2_A4 or PC1.5_A6 AMPR in Table 6.2.3.2-2 for PC3, PC2 and PC1.5 respectively.

Proposal 2: PC1.5 A-MPR for CA_NS_04 for contiguous CC/contiguous RBs section 6.2A.3.1.1.1 of TS38.101-1 [2] should be revised as follows:

###### 6.2A.3.1.1.1	A-MPR for CA_NS_04

6.2A.3.1.1.1.1	Contiguous allocations

For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR

For all modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

if the RB allocation is an inner allocation as defined in clause 6.2A.2.1,  then A-MPR = MPR

Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc).

if the RB allocation is an outer allocation as defined in clause 6.2A.2.1,

then A-MPR = MPR+1.5dB for BW Class B,  A-MPR = MPR for BW class C for PC3 and PC2 and A-MPR=MPRHH+1.5dB for BW class C for PC1.5 UE handheld devices and A-MPR=MPRFWA+1.5dB for BW class C for PC1.5 FWA devices.

Where

-	MPR is the MPR as defined in Table 6.2A.2.1-1, Table 6.2A.2.1-1a, Table 6.2A.2.1-1b and [Table TBD] for PC3, PC2 and PC1.5 respectively and the respective CA bandwidth class

-	 AMPRcc is defined as the PC3_A2, PC2_A4 or PC1.5_A6 AMPR in Table 6.2.3.2-2 for PC3, PC2 and PC1.5 respectively.

Measurements were done for contiguous CC/non-contiguous RBs however when the A-MPR formula in the standard [2] was examined an error was noticed in the current formula for PC3 and PC2. The standard states the following for contiguous CC/non-contiguous RB CA_NS_04:

6.2A.3.1.1.1.2	Non-contiguous allocations

For intra-band contiguous CA_n41B and CA_n41C and it receives IE CA_ NS_04, the UE determines the allowed Additional Maximum Power Reduction (AMPR) for the maximum output power as specified in this clause. The AMPR is specified by AMPRIM3 to meet -25dBm/MHz when IM3 falls in -25dBm/MHz region of Table 6.5A.2.3.1.1-1 or Table 6.5A.3.3.1.1-1. And uses MPR for all other cases.

The UE determines the AMPR type as follows:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA ≥ 2490.5 MHz,

-	if allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

-	if RB allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if RB allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3.

where

-	MPR is the MPR as defined in Table 6.2A.2.1-2, Table 6.2A.2.1-3 and Table 6.2A.2.1-4 for PC3 and PC2 respectively and the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.1.1

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.1.1

6.2A.3.1.1.1.3	AMPRIM3 to meet -25dBm/MHz

AMPR in this clause is for intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA = 	13; 	0 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

10.5;       3.24 ≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8; 	10.08 ≤ B < 16.56

7;        16.56 ≤ B < 21.96

6; 	      21.96 ≤ B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 (MHz), where SCS1 and SCS2 are expressed in kHz.

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.

Our understanding is that if Ffilter,low < FIM3,low_block,high <2490.5 MHz Then the A-MPR values in the ELSE section should be used.

Proposal 3: In section 6.2A.3.1.1.2 for non-contiguous allocations make the following correction:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

IF (Ffilter,low < FIM3,low_block,high) OR  (2490.5 MHz> FIM3,low_block,low)

-  A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3

Else

-	if RB allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if RB allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3.

where

-	MPR is the MPR as defined in Table 6.2A.2.1-2, Table 6.2A.2.1-3, Table 6.2A.2.1-4, Table 6.2A.2.1-5 for PC3 for bandwidth classes B and C, PC2 for bandwidth classes B and C and PC1.5 bandwidth class C respectively and the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-     FIM3,low_block,low = (2 * Flow_alloc,low_edge ) – Fhigh_alloc,high_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.1.1

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.1.1

A CR, R4-2506141, has been submitted in the maintenance agenda to make the above described corrections to the non-contiguous A-MPR algorithm.

Also, it seems that the A-MPRs given in section 6.2A.3.1.1.1.3 are only for PC3 and not PC2. This can be seen by comparing NCCA AMPRs for PC3 and PC2 given for -25dBm/MHz in sections 6.2A.3.1.2.1.1 and 6.2A.3.1.2.1.3. Also, if you compare the PC3 contiguous CC/non-contiguous RBs in section 6.2A.3.1.1.1.3 with the PC3 NCCA in section 6.2A.3.1.1.3 the following delta AMPR is observed between the 2 sets of values:

| NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  | NS04 NCCA (section 6.2A.3.1.2.1.1) |  | Delta between C-CA and NCCA |
| --- | --- | --- | --- | --- |
| MA[dB] | Allocation BW[MHz] | MA[dB] | Allocation BW[MHz] | [dB] |
| 13 | 0 ≤ B < 2.16 | 12 | 0 ≤ B < 2.16 | 1 |
| 11.5 | 2.16 ≤ B < 3.24 | 11 | 2.16≤ B < 3.24 | 0.5 |
| 10.5 | 3.24 ≤ B < 5.04 | 10 | 3.24 ≤ B < 5.04 | 0.5 |
| 9.5 | 5.04 ≤ B < 10.08 | 9 | 5.04 ≤ B < 10.08 | 0.5 |
| 8 | 10.08 ≤ B < 16.56 | 8 | 10.08 ≤ B < 16.38 | 0 |
| 7 | 16.56 ≤ B < 21.96 | 7 | 16.38 ≤ B < 21.78 | 0 |
| 6 | 21.96 ≤ B | 6 | 21.78 ≤ B | 0 |

Table2: A-MPR comparison between contiguous CC/NC RBs and NCCA for PC3

Based on the deltas in the above table the A-MPRs for contiguous CC/NC RBs can be derived by adding the same A-MPR deltas to the NCCA PC2 values given in section 6.2A.3.1.2.1.3. Therefore, we have the following values as illustrated below:

| NS04 NCCA (section 6.2A.3.1.2.1.1) |  | Delta between C-CA and NCCA | NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  |
| --- | --- | --- | --- | --- |
| MA[dB] | Allocation BW[MHz] | [dB] | MA[dB] | Allocation BW[MHz] |
| 14.5 | 0 ≤ B < 1.44 | 1 | 15.5 | 0 ≤ B < 1.44 |
| 14 | 1.44 ≤ B < 2.88 | 0.5 | 14.5 | 1.44 ≤ B < 2.88 |
| 13 | 2.88 ≤ B < 5.76 | 0.5 | 13.5 | 2.88 ≤ B < 5.76 |
| 11 | 5.76 ≤ B < 10.8 | 0.5 | 11.5 | 5.76 ≤ B < 10.8 |
| 9.5 | 10.8 ≤ B < 23.04 | 0 | 9.5 | 10.8 ≤ B < 23.04 |
| 9 | 23.04 ≤ B | 0 | 9 | 23.04 ≤ B |

Table3: A-MPR for contiguous CC/NC RBs PC2

Proposal 4: In section 6.2A.3.1.1.3 adopt the following A-MPR table for 1Tx PC2

| NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  |
| --- | --- |
| MA[dB] | Allocation BW[MHz] |
| 15.5 | 0 ≤ B < 1.44 |
| 14.5 | 1.44 ≤ B < 2.88 |
| 13.5 | 2.88 ≤ B < 5.76 |
| 11.5 | 5.76 ≤ B < 10.8 |
| 9.5 | 10.8 ≤ B < 23.04 |
| 9 | 23.04 ≤ B |

The NS04 contiguous CA with NC RB allocations for section 6.2A.3.1.1.3 the PC1.5 A-MPR table can be derived by adding MPR deltas similar to that seen between PC3(1Tx) and PC2(2Tx) bandwidth class C in sections 6.2A.2.1 of approximately 1dB to the PC2 table derived for 1 TxPC2. The PC1.5 A-MPR table is shown below

| NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  |
| --- | --- |
| MA[dB] | Allocation BW[MHz] |
| 16.5 | 0 ≤ B < 1.44 |
| 15.5 | 1.44 ≤ B < 2.88 |
| 14.5 | 2.88 ≤ B < 5.76 |
| 12.5 | 5.76 ≤ B < 10.8 |
| 10.5 | 10.8 ≤ B < 23.04 |
| 10 | 23.04 ≤ B |

Table3: A-MPR for contiguous CC/NC RBs PC1.5

Proposal 5: In section 6.2A.3.1.1.3 adopt the following A-MPR table for 2Tx PC1.5

| NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  |
| --- | --- |
| MA[dB] | Allocation BW[MHz] |
| 16.5 | 0 ≤ B < 1.44 |
| 15.5 | 1.44 ≤ B < 2.88 |
| 14.5 | 2.88 ≤ B < 5.76 |
| 12.5 | 5.76 ≤ B < 10.8 |
| 10.5 | 10.8 ≤ B < 23.04 |
| 10 | 23.04 ≤ B |

Proposal 6: For PC1.5 A-MPR, for CA_NS_04 contiguous CA with NC RB allocations sections 6.2A.3.1.1.2 and 6.2A.3.1.1.1.3 of TS38.101-1 [2] should be revised as follows:

6.2A.3.1.1.1.2	Non-contiguous allocations

For intra-band contiguous CA_n41B and CA_n41C and it receives IE CA_ NS_04, the UE determines the allowed Additional Maximum Power Reduction (AMPR) for the maximum output power as specified in this clause. The AMPR is specified by AMPRIM3 to meet -25dBm/MHz when IM3 falls in -25dBm/MHz region of Table 6.5A.2.3.1.1-1 or Table 6.5A.3.3.1.1-1. And uses MPR for all other cases.

The UE determines the AMPR type as follows:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA ≥ 2490.5 MHz,

-	if allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

IF (Ffilter,low < FIM3,low_block,high) OR  (2490.5 MHz> FIM3,low_block,low)

-  A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3

Else

-	if RB allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if RB allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3.

where

-	MPR is the MPR as defined in Table 6.2A.2.1-2, Table 6.2A.2.1-3, Table 6.2A.2.1-4, Table [TBD] for PC3 for bandwidth classes B and C, PC2 for bandwidth classes B and C and PC1.5 bandwidth class C respectively and the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-     FIM3,low_block,low = (2 * Flow_alloc,low_edge ) – Fhigh_alloc,high_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.1.1

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.1.1

6.2A.3.1.1.1.3	AMPRIM3 to meet -25dBm/MHz

AMPR in this clause is for intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA = 	13; 	0 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

10.5;       3.24 ≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8; 	10.08 ≤ B < 16.56

7;        16.56 ≤ B < 21.96

6; 	      21.96 ≤ B

AMPRIM3=MA, Where MA is defined as follows for 2 Tx PC1.5

MA = 	16.5; 	0 ≤ B < 1.44

15.5; 	1.44 ≤ B < 2.88

14.5;         2.88 ≤ B < 5.76

12.5; 	5.76 ≤ B < 10.8

10.5; 	10.8 ≤ B <23.04

10;        23.04 ≤ B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 (MHz), where SCS1 and SCS2 are expressed in kHz.

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.

# Conclusion

In this paper we present our measurements for MPR/A-MPR for contiguous CC with continguous/NC RBs for various modulations and waveform types.

Proposal 1: For contiguous CC/non-contiguous RBs adopt the following MPR for handheld and FWA devices

| Modulation |  | PC1.5 (2Tx) |  |  | PC1.5 (2Tx) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | FWA |  |  |
|  |  | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 |
| DFT-s-OFDM | PI/2 BPSK | 5 | 9.5 | 16 | 4.5 | 9 | 15.5 |
|  | QPSK | 5 | 9.5 | 16 | 4.5 | 9 | 15.5 |
|  | 16QAM | 5 | 9.5 | 16 | 4.5 | 9 | 15.5 |
|  | 64QAM | 7 | 9.5 | 16 | 6.5 | 9 | 15.5 |
|  | 256QAM | 8.5 | 9.5 | 16 | 8 | 9 | 15.5 |
| CP-OFDM | QPSK | 5.5 | 10 | 16.5 | 5 | 9.5 | 16.0 |
|  | 16QAM | 5.5 | 10 | 16.5 | 5 | 9.5 | 16.0 |
|  | 64QAM | 7 | 10 | 16.5 | 6.5 | 9.5 | 16.0 |
|  | 256QAM | 9.5 | 10 | 16.5 | 9 | 9.5 | 16.0 |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |

Proposal 2: PC1.5 A-MPR for CA_NS_04 for contiguous CC/contiguous RBs section 6.2A.3.1.1.1 of TS38.101-1 [2] should be revised as follows:

###### 6.2A.3.1.1.1	A-MPR for CA_NS_04

6.2A.3.1.1.1.1	Contiguous allocations

For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR

For all modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

if the RB allocation is an inner allocation as defined in clause 6.2A.2.1,  then A-MPR = MPR

Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc).

if the RB allocation is an outer allocation as defined in clause 6.2A.2.1,

then A-MPR = MPR+1.5dB for BW Class B,  A-MPR = MPR for BW class C for PC3 and PC2 and A-MPR=MPRHH+1.5dB for BW class C for PC1.5 UE handheld devices and A-MPR=MPRFWA+1.5dB for BW class C for PC1.5 FWA devices.

Where

-	MPR is the MPR as defined in Table 6.2A.2.1-1, Table 6.2A.2.1-1a, Table 6.2A.2.1-1b and [Table TBD] for PC3, PC2 and PC1.5 respectively and the respective CA bandwidth class

-	 AMPRcc is defined as the PC3_A2, PC2_A4 or PC1.5_A6 AMPR in Table 6.2.3.2-2 for PC3, PC2 and PC1.5 respectively.

Proposal 3: In section 6.2A.3.1.1.2 for non-contiguous allocations make the following correction:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

IF (Ffilter,low < FIM3,low_block,high) OR  (2490.5 MHz> FIM3,low_block,low)

-  A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3

Else

-	if RB allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if RB allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3.

where

-	MPR is the MPR as defined in Table 6.2A.2.1-2, Table 6.2A.2.1-3, Table 6.2A.2.1-4, Table 6.2A.2.1-5 for PC3 for bandwidth classes B and C, PC2 for bandwidth classes B and C and PC1.5 bandwidth class C respectively and the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-     FIM3,low_block,low = (2 * Flow_alloc,low_edge ) – Fhigh_alloc,high_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.1.1

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.1.1

Proposal 4: In section 6.2A.3.1.1.3 adopt the following A-MPR table for 1Tx PC2

| NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  |
| --- | --- |
| MA[dB] | Allocation BW[MHz] |
| 15.5 | 0 ≤ B < 1.44 |
| 14.5 | 1.44 ≤ B < 2.88 |
| 13.5 | 2.88 ≤ B < 5.76 |
| 11.5 | 5.76 ≤ B < 10.8 |
| 9.5 | 10.8 ≤ B < 23.04 |
| 9 | 23.04 ≤ B |

Proposal 5: In section 6.2A.3.1.1.3 adopt the following A-MPR table for 2Tx PC1.5

| NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  |
| --- | --- |
| MA[dB] | Allocation BW[MHz] |
| 16.5 | 0 ≤ B < 1.44 |
| 15.5 | 1.44 ≤ B < 2.88 |
| 14.5 | 2.88 ≤ B < 5.76 |
| 12.5 | 5.76 ≤ B < 10.8 |
| 10.5 | 10.8 ≤ B < 23.04 |
| 10 | 23.04 ≤ B |

Proposal 6: For PC1.5 A-MPR, for CA_NS_04 contiguous CA with NC RB allocations sections 6.2A.3.1.1.2 and 6.2A.3.1.1.1.3 of TS38.101-1 [2] should be revised as follows:

6.2A.3.1.1.1.2	Non-contiguous allocations

For intra-band contiguous CA_n41B and CA_n41C and it receives IE CA_ NS_04, the UE determines the allowed Additional Maximum Power Reduction (AMPR) for the maximum output power as specified in this clause. The AMPR is specified by AMPRIM3 to meet -25dBm/MHz when IM3 falls in -25dBm/MHz region of Table 6.5A.2.3.1.1-1 or Table 6.5A.3.3.1.1-1. And uses MPR for all other cases.

The UE determines the AMPR type as follows:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA ≥ 2490.5 MHz,

-	if allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

IF (Ffilter,low < FIM3,low_block,high) OR  (2490.5 MHz> FIM3,low_block,low)

-  A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3

Else

-	if RB allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if RB allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3.

where

-	MPR is the MPR as defined in Table 6.2A.2.1-2, Table 6.2A.2.1-3, Table 6.2A.2.1-4, Table [TBD] for PC3 for bandwidth classes B and C, PC2 for bandwidth classes B and C and PC1.5 bandwidth class C respectively and the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-     FIM3,low_block,low = (2 * Flow_alloc,low_edge ) – Fhigh_alloc,high_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.1.1

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.1.1

6.2A.3.1.1.1.3	AMPRIM3 to meet -25dBm/MHz

AMPR in this clause is for intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA = 	13; 	0 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

10.5;       3.24 ≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8; 	10.08 ≤ B < 16.56

7;        16.56 ≤ B < 21.96

6; 	      21.96 ≤ B

AMPRIM3=MA, Where MA is defined as follows for 2 Tx PC1.5

MA = 	16.5; 	0 ≤ B < 1.44

15.5; 	1.44 ≤ B < 2.88

14.5;         2.88 ≤ B < 5.76

12.5; 	5.76 ≤ B < 10.8

10.5; 	10.8 ≤ B <23.04

10;        23.04 ≤ B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 (MHz), where SCS1 and SCS2 are expressed in kHz.

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.

# References

[1] R4-2505100, WF on HPUE RF requirements, 7-11 April 2025

[2] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V19.0.0