3GPP TSG-RAN WG4 Meeting #115            	R4-2505965

Malta, MT, 19th-23th, May, 2025

Agenda item:	7.17.4

Source:	Samsung

Title:	View on the remaining issue for BS demodulation requirement with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the remaining issue of Rel-19 WI on demodulation performance requirements enhancement with MMSE-IRC receiver was further discussed. The related agreements are captured in the WF [1] as following

- Test parameters and simulation assumptions - Number of interferers for 2Rx - Consider 1 interferer for 2Rx for both HomNet and HetNet scenarios - Propagation condition - TDLA30-10 for HetNet and TDLC300-100 for HomNet - RAN4 will discuss the applicability for different BS classes and different test scenarios - Simulation result collection - Companies are encouraged to provide simulation results for all the agreed test cases in the next meeting. - Manufacturer declarations and applicability rules - Manufacturer declaration on MMSE-IRC receiver - Introduce new manufactory declaration for BS supporting enhanced performance requirements. - Note: MMSE-IRC as specified in Rel-19 may have been widely deployed in the earlier release BS products. (This note is not going to be captured in the specification or technical report) - Applicability for different BS classes and test scenarios - Follow the LTE approach – Only specifies that HomNet requirements are not applicable for Local Area BS. - FFS whether to consider additional applicability rule on how the test is performed: - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped - Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

In this contribution, the view on the remaining issue of test setup for BS demodulation with MMSE-IRC were provide.

# 2	General

## 2.1	Test scenario

Additional applicability rule

For test scenario, both HomNet and HetNet scenarios were agreed to cover the performance requirement of MMSE-IRC receiver. Regarding the applicability for requirement in different scenarios, it will be further discussed.

In the last meeting, RAN4 agreed that only HomNet requirements are not appliable for local Area BS. That means both HetNet and HomNet requirements can be applied for wide area and medium area BS. From the baseband processing perspective of MMSE-IRC, there is no difference foreseen. Since the test applicability for different HomNet and HetNet requirements will be considered, which means if BS is declared to support wide area BS, Medium Area BS, pass the tests of HomNet (DIP Set 1), the test for HetNet (DIP Set 2) can be skipped. Therefore, based on the declaration of BS categories, to reduce the test effort, we are fine to consider additional applicability rule for perform the testing on HetNet and HomNet requirement based on BS declaration on BS classes.

Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped.

## 2.2	Simulation Assumption

Number of CDM group for Interference cells

In the previous meeting, RAN4 has agreed the same DMRS port#0 is configured for both serving and interfering cells, as indicated in R4-2419780

- DMRS configuration - Cover DMRS configuration type 1, FFS whether to include DMRS configuration type 2 - Use DMRS port#0 for both serving and interfering PUSCH - On DMRS sequence generation, use different DMRS sequence for each gNB: - For CP-OFDM: nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively - FFS DMRS sequence generation configuration for DFT-s-DFDM

Regarding the number of CDM group, there is no discussed yet. In Rel-15, the typical assumption for BS demodulation related with number of CDM group is 2, where 3dB power boosting will be applied for DMRS REs.

For the interferer cells, considering the same DMRS port is configured for both serving cell and interfering cells, the same assumption for number of CDM group should be applied for interferer cells.

To facilitate the implementation of MMSE-IRC, as agreed in previous meeting, the baseline assumption for covariance matrix is based on DM-RS REs

- Agreement in R4-2413570

    - MMSE IRC Receiver Assumption

    - Reuse the same receiver model for MMSE-IRC in TR36.884 as below:

    - Baseline receiver assumption (for performance gain study only)

The assumption is reused from LTE, while the DMRS pattern in NR is different with LTE. The DMRS pattern is full allocated within in RB with cycling shift to support multiple ports for LTE, while the DMRS pattern is comb structure in NR

Assuming the receiving signal at DM-RS REs and data RE expressed as following,

yDMRS=2P1hDMRS, 1XDMRS,1+2P2hDMRS, 2XDMRS,2+nDMRS

yDATA=P1hDATA, 1XDMRS,1+P2hDATA, 2XDATA,2+nDATA

And also receiving signal at non-DM-RS REs in DM-RS symbol is,

ynon-DMRS=nnon-DMRS

If only using DM-RE, where 6 REs per RB, ideally, the covariance matrix can be calculated as,

RDMRS=6(2P2hDMRS, 2XDMRS,2+nDMRS2P2hDMRS, 2XDMRS,2+nDMRSH)

= 2P2|hDMRS, 2|2+σ2

While, the covariance matrix at Data RE is

RDATA=P2|hDATA, 2|2+σ2

Therefore, if only using available DM-RE. there will be interference power mismatch between data and DM-RS under interference condition.

To guarantee the same interference power between data and DMRS symbols, the non DMRS RE in the DMRS symbol should be also used for interference covariance calculation, then the related interference covariance matrix can be calculated as

RDMRS=122P2|hDMRS, 2|2+σ2+σ2=P2|hDMRS, 2|2+σ2

Actually, in the practical implementation, gNB have no information about the interference condition. So, both DMRS REs and non DMRS REs in the DMRS symbol will be used for interference covariance estimation. Therefore, if the assumption is based on DMRS for interference covariance estimation, it will result in the improper requirement specified.

Observation 1: Power mismatch between data and DMRS under interference condition if based on assumption that using the DMRS REs for interference covariance calculation

Proposal 2:  Revise the baseline assumption for MMSE-IRC as the estimation of interference covariance matrix is performed based on DM-RS symbol per PRB and per TTI basis, instead of based on DMRS REs

# 3	Conclusion

In this contribution, the view on the remaining issues of BS demodulation requirement with MMSE-IRC receiver was provided.

Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped.

Observation 1: Power mismatch between data and DMRS under interference condition if based on assumption that using the DMRS REs for interference covariance calculation

Proposal 2:  Revise the baseline assumption for MMSE-IRC as the estimation of interference covariance matrix is performed based on DM-RS symbol per PRB and per TTI basis, instead of based on DMRS REs

# References

R4-2504768, Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS