3GPP TSG-RAN WG4 Meeting #115            	R4-2505966

Malta, MT, 19th-23th, May, 2025

Agenda item:	7.17.4

Source:	Samsung

Title:	Simulation results on BS demodulation requirement with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In this contribution, the simulation results for BS demodulation with MMSE-IRC are provided for alignment based on agreed simulation assumption in [1].

# 2	Simulation Results

In this section, the initial simulation results are provided to check the performance gain of MMSE-IRC over MMSE receiver. the simulation results for the following test cases are provided

- Number of Rx: 2/4/8Rx

- Number of Interferes: 1 for 2Rx, and 2 for 4Rx/8Rx

- PUSCH mapping type: type A

- Interferes scenario: HomNet and HetNet

- Channel of interferes: TDLC300-100

- Channel of target cell:

  - TDLA30-10 or TDLC300-100 for HetNet, and TDLC300-100 for HomNet

- Waveform:  CP-OFDM

- MCS: 16

- SCS and BW:

  - 15KHz:  5/20MHz

  - 30KHz: 10/100MHz

Table 1: PUSCH performance with HomNet scenario under 2Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS 16 | 14.65 | 11.89 | 2.76 |

Table 2.  PUSCH performance with HetNet scenario under 2Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 17.56 | 12.38 | 5.18 |

Table 3.  PUSCH performance with HomNet scenario under 2Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 15.05 | 12.18 | 2.87 |

Table 4.  PUSCH performance with HetNet scenario under 2Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 17.5 | 12.55 | 4.95 |

Table 5.  PUSCH performance with HomNet scenario under 2Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS16 | MCS16 | 14.75 | 12.05 | 2.7 |

Table 6.  PUSCH performance with HetNet scenario under 2Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 16.58 | 11.49 | 5.09 |

Table 7.  PUSCH performance with HomNet scenario under 2Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS16 | MCS16 | 15.05 | 12.26 | 2.79 |

Table 8.  PUSCH performance with HetNet scenario under 2Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 16.8 | 11.82 | 4.98 |

Table 9.  PUSCH performance with HomNet scenario under 4Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.33 | 8.31 | 5.02 |

Table 10.  PUSCH performance with HetNet scenario under 4Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 16.45 | 7.92 | 8.53 |

Table 11.  PUSCH performance with HomNet scenario under 4Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.68 | 8.68 | 5 |

Table 12.  PUSCH performance with HetNet scenario under 4Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 16.49 | 8.27 | 8.22 |

Table 13.  PUSCH performance with HomNet scenario under 4Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.37 | 8.54 | 4.83 |

Table 14.  PUSCH performance with HetNet scenario under 4Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 16.11 | 7.82 | 8.29 |

Table 15.  PUSCH performance with HomNet scenario under 4Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.8 | 8.95 | 4.85 |

Table 16.  PUSCH performance with HetNet scenario under 4Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 16.29 | 8.15 | 8.14 |

Table 17.  PUSCH performance with HomNet scenario under 8Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 10.69 | 4.34 | 6.35 |

Table 18.  PUSCH performance with HetNet scenario under 8Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.31 | 3.59 | 9.72 |

Table 19.  PUSCH performance with HomNet scenario under 8Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 11.4 | 4.55 | 6.85 |

Table 20.  PUSCH performance with HetNet scenario under 8Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.6 | 4.02 | 9.58 |

Table 21.  PUSCH performance with HomNet scenario under 8Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 10.82 | 4.63 | 6.19 |

Table 22.  PUSCH performance with HetNet scenario under 8Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.21 | 3.76 | 9.45 |

Table 23.  PUSCH performance with HomNet scenario under 8Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 11.6 | 4.87 | 6.73 |

Table 24.  PUSCH performance with HetNet scenario under 8Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.39 | 4.09 | 9.3 |

# 3	Conclusion

In this contribution, the initial simulation results are provided.

# References

R4-2504768, Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS