3GPP TSG-RAN4 Meeting #115	 R4-2505967

# Malta, MT, 19th - 23th, May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | <38.141-1> | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on conducted performance requirements for MMSE-IRC in 38.141-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19, RAN4 introduced PUSCH requirement based MMSE-IRC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new section for PUSCH requirment with MMSE-IRC - Add new section for INR defintion |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PUSCH requirement with MMSR-IRC can be not verfied |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.X, B.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.2.xx	Performance requirements for PUSCH with enhanced DM-RS

8.2.xx.1	Definition and applicability

The performance requirement of PUSCH with enhanced DM-RS is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in clause xxxxx.

8.2.xx.2	Minimum Requirement

The minimum requirement is in TS 38.104 [2] clause [8.2.xx].

8.2.xx.3	Test Purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR with enhanced DM-RS.

8.2.xx.4	Method of test

8.2.xx.4.1	Initial Conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

8.2.xx.4.2	Procedure

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

2)	Adjust the AWGN generator, according to the channel bandwidth, defined in table 8.2.15.4.2-1.

Table 8.2.xx.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15kHz | 5 | -86.5 dBm / 4.5MHz |
|  | 20 | -80.2 dBm / 19.08MHz |
| 30kHz | 10 | -83.6 dBm / 8.64MHz |
|  | 100 | -73.1 dBm / 98.28MHz |
| NOTE: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |

3)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in table 8.2.xx.4.2-2.

Table 8.2.xxx.4.2-2: Test parameters for testing PUSCH with enhanced DM-RS

| Parameter |  |  | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interfer 1 | Interfer 2 |
| Transform precoding |  |  | Disabled |  |  |
| Example TDD UL-DL pattern (Note 1) |  |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |  |  |
| INR (Note) |  | Set 1 with 1 interferer | N/A | 5.35 dB | N/A |
|  |  | Set 1 with 2 interferers |  | 7.28 dB | -2.52 dB |
|  |  | Set 2 with 1 interferer |  | 9.82 dB | N/A |
|  |  | Set 2 with 2 interferers |  | 12.38dB | -0.97 dB |
| Cell ID |  |  | 0 | 1 | 2 |
| Interference model |  |  | N/A | As specified in clause B.x.2 | As specified in clause B.x.2 |
| HARQ | Maximum number of HARQ transmissions |  | 4 | N/A | N/A |
|  | RV sequence |  | 0, 2, 3, 1 | N/A | N/A |
| DM-RS | DM-RS configuration type |  | 1 |  |  |
|  | DM-RS duration |  | single-symbol DM-RS |  |  |
|  | Additional DM-RS position |  | pos1 |  |  |
|  | Number of DM-RS CDM group(s) without data |  | 2 |  |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |  |  |
|  | DM-RS port(s) |  | {0} |  |  |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 | NID0=1, nSCID =0 | NID0=2, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type |  | A, B |  |  |
|  | Start symbol |  | 0 |  |  |
|  | Allocation length |  | 14 |  |  |
| Frequency domain resource assignment | RB assignment |  | Full applicable test bandwidth |  |  |
|  | Frequency hopping |  | Disabled |  |  |
| Code block group based PUSCH transmission |  |  | Disabled |  |  |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. NOTE 2: One explicit interferer, i.e., interferer 1, is modelled for tests with 2/4/8 RX antennas. Two explicit interferers are modelled for tests with 2/4/8 RX antennas. NOTE 3: The respective received energy of each interferer relative to is defined by its associated INR value as specified in clause B.X.1. INR set 1 and set 2 are derived respectively in homogeneous and heterogeneous network scenarios. NOTE 4 All cells are time-synchronous. |  |  |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

5)	Adjust the equipment so that required SNR specified in table 8.2.15.5-1 to table 8.2.15.5-4 is achieved at the BS input.

6)	For each of the reference channels in table 8.2.15.5-1 to table 8.2.15.5-4 applicable for the base station, measure the throughput.

8.2.xx.5	Test Requirement

The throughput measured according to clause 8.2.xx.4.2 shall not be below the limits for the SNR levels specified in table 8.2.xx.5-1 to table 8.2.xx.5-4.

Table 8.2.XX.5-1: Test requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  |  |  |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer 1 |  |  | Interferer2 |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 2 | Normal | TDLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |

Table 8.2.XX.5-2: Test requirements for PUSCH with 70% of maximum throughput, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  |  |  |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer 1 |  |  | Interferer2 |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 2 | Normal | TDLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |

Table 8.2.XX.5-3: Test requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  |  |  |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer 1 |  |  | Interferer2 |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 2 | Normal | TDLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |

Table 8.2.XX.5-4: Test requirements for PUSCH with 70% of maximum throughput, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  |  |  |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer 1 |  |  | Interferer2 |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 2 | Normal | TDLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |

Table 8.2.XX.5-5: Test requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  |  |  |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer 1 |  |  | Interferer2 |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 2 | Normal | TDLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |

Table 8.2.XX.5-6: Test requirements for PUSCH with 70% of maximum throughput, Type B, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  |  |  |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer 1 |  |  | Interferer2 |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 2 | Normal | TDLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |

Table 8.2.XX.5-7: Test requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  |  |  |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer 1 |  |  | Interferer2 |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 2 | Normal | TDLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |

Table 8.2.XX.5-8: Test requirements for PUSCH with 70% of maximum throughput, Type B, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  |  |  |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer 1 |  |  | Interferer2 |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 2 | Normal | TDLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 1 interferer | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 4 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | TDLC300-100 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 1 with 2 interferers | 70 % | TBD | pos1 | TBD |
|  | 8 | Normal | DLA30-10 Low |  | TDLC300-100 Low | TDLC300-100 Low |  | Set 2 with 2 interferers | 70 % | TBD | pos1 | TBD |

<End of Change 1>

<Start of Change 2>

B.X	Interference model for PUSCH requirement with intercell interference

B.x.1	Interference to Noise ratio (INR)

Each interfering cell involved in PUSCH performance requirements with intercell interference is characterized by its associated interferer to noise ratio (INR) value:

INRi=DIPi1-i=1MDIPi, i=1,…, M

Where

DIPi=Ior(i)N,, j=1,…, M

where  Ior(i) is the received energy from the i-th strongest inter-cell interferer involved in the requirement scenario and  N'=j=1MIor(j)+N  where N is the the energy of the white noise source consistent with the definition provided in clause xx and M  is the total number of simultaneously transmitted inter-cell interferers involved in a given requirement scenario.

B.X.2	Interference model for PUSCH requirements

This subclause provides synchronous network deployment interference model for each explicitly modelled interfering cell in the requirement scenario.

In each subframe, each interferer shall transmit 16QAM randomly modulated data over the entire PUSCH region and the same resource blocks as the tested signal. Demodulation reference signal, configured according to Table xxx for PUSCH requirement with intercell interference.

<End of Change 2>