3GPP TSG-RAN WG4 Meeting #115            	R4-2505968

Malta, MT, 19th-23th, May, 2025

Agenda item:	7.10.5.1

Source:	Samsung

Title:	Discussion on NTN testing NGSO channel model

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the NGSO channel model for NTN testing was under discussion, the related agreements are captured in the WF [1] as following

Issue 1-1-1 TE emulated channel model comparison with GMAT - agreement - Keep previous agreement: RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay based on the Kepler’s equation and equation of motion. It is up to TE implementation which method is used to derive the varying Doppler shift and propagation delay for NGSO test. Specify both the satellite position/velocity calculation methods in TS 38.101-5/TS 36.102 - Note: the details models for Kepler’s equation and equation of motion can be referred in Appendix - Not need to compare the Kepler’s equation and equation of motion model with GMAT Issue 1-1-2 FFS to consider the Earth’s rotation during signal flight time for more accurate estimations of propagation delay - Proposal - Option 1 ΔtDL=RDLtTc1-1cvUEtT・RDLtTRDLtT - Option 2 DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR> - Option 4 Δt=DelayERt=ρtRECEFc+vtRECI⋅ρtRECEFρtRECEF=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF ρtR ECEF=rtRECEF-UEtRECEF - agreement - Consider the earth rotation during signal flight time impact into TE-emulated channel modelling - Take the proposals from option 1 or option2/4 for DL propagation delay calculation - Companies are encouraged to cross check the equations to derive the total propagation delay and its error. - Note: The details of option 1, option 2 and option 4 can be referred as R4-2503710, R4-2503247 and R4-2504344, respectively - Not consider the earth rotation for doppler shift calculation Issue 1-1-7 Accuracy requirements for maximum delay errors to obtain the propagation delay and Doppler shift trajectory - Agreement - The number of iterations to solve Kepler’s equation based on Newton-Raphson method is up to TE implementation - The interpolation method for the Doppler shift and Propagation delay trajectory is up to TE implementation - Note: the earth rotation during signal flight time impact should be considered to calculate the propagation delay - To derive accuracy requirement, - Baseline assumption - number of iterations is fixed to 4 for solving Kepler’s equation based on Newton-Raphson method - Calculate the Propagation delay and Doppler shift every 1ms - For accuracy requirement - Apply linear interpolation to calculate the propagation delay and Doppler shift with 1ms granularity as baseline, no restrict the implementation on the TE side - Consider the propagation delay error is less than TBD (s) and Doppler shift error is less than TBD (Hz) - The time period to calculate the error is 1s - TE should meet the minimum accuracy requirement defined. - Companies are encouraged to provide the acceptable value with associated propagation delay error and Doppler shift error calculation method for accuracy requirement in the next meeting

In this contribution, the view on the remaining issue of NTN NGSO testing channel model is provided.

# 2	Channel Model

RAN4 agreed to introduce TE-emulated time varying doppler shift and propagation delay based on the Kepler’s equation and equation of motion.  Regarding the propagating delay model, the earth rotation was considered during signal flight time for more accurate estimations of propagation delay. In the last meeting, some correction models are proposed.

Correction model

Regarding option 2 and option 4 proposed by company, the results are same, only difference is how to calculate the distance vector between satellite and UE. Option 2 is based on the calculation of (UE position vector – satellite position vector), while the option 4 is based on the calculation of (satellite position vector- UE position vector).

Regrading option 1, it is assuming UE is moving with considering rotation, while the satellite is fixed. The propagation delay calculation is based on transmission time at satellite. Since the test is from UE perspective, it is more proper to calculate the delay based on receiving time at the UE side. So, we prefer to take option 4 for propagation delay calculation.

Observation 1: Option 2 and Option 4 are same; the only difference is the how to calculate the distance vector between satellite and UE.

Proposal 1: Take option 4 for propagation delay calculation

Accuracy requirements for maximum delay errors to obtain the propagation delay and Doppler shift trajectory

In order to facilitate the TE implementation and alignment for channel model and TE should meet minimum accuracy requirement with different implementation methods, the baseline assumption was agreed to derive the accuracy requirement

based on the detail’s models for Kepler equation in the [1], the propagation delay and doppler shift error are illustrated in following figures, where the propagation delay is based on option 4.

where the initial ephemeris information for LEO-600 stating from elevation angles of 30 degrees as following table in [1]

Table 1: Initial ephemeris information for LEO-600 stating from elevation angles of 30 degrees.

| Parameters | Unit | Values |
| --- | --- | --- |
| positionX-r17 | km | [-3451.1273] |
| positionY-r17 | km | [5703.5635] |
| positionZ-r17 | km | [2071.0809] |
| velocityVX-r17 | km/s | [0.87360] |
| velocityVY-r17 | km/s | [-2.06700] |
| velocityVZ-r17 | km/s | [7.21231] |

In order to calculate the error requirement, we apply different metric including NMSE, MSE and Average Error

- NMSE

NMSE=i=11000|xi-xi|2i=11000|xi|2

- MSE

MSE=11000i=11000|xi-xi|2

- Average Error

AVG=11000i=11000|xi-xi|

where xi is the delay or doppler shift calculated by Kepler model per 1ms,  xi is estimated delay or doppler shift calculated based on interpolation with 1ms granularity

Table 2: accuracy requirement (To be update)

| Error metric | Propagation delay (s) | Doppler Shift (Hz) |
| --- | --- | --- |
| NMSE | 8.485468283568684e-21 | 1.141531684126212e-19 |
| MSE | 1.096143560650351e-25 | 1.196207224711651e-05 |
| Average Error | 2.957513223038333e-13 | 1.196210506168427e-05 |

# 3	Conclusion

In this contribution, the view on the NTN testing NGSO channel model is provided.

Observation 1: Option 2 and Option 4 are same; the only difference is the how to calculate the distance vector between satellite and UE.

Proposal 1: Take option 4 for propagation delay calculation

# References

R4-2504700, Way Forward for [114bis][323] NTN_testing_NGSO_channel_model