3GPP TSG-RAN WG4 Meeting #115            	R4-2505969

Malta, MT, 19th-23th, May, 2025

Agenda item:	7.10.5.3

Source:	Samsung

Title:	Discussion on demodulation requirement for NTN NGSO testing

Document for:	Discussion

- Introduction

In RAN#104 meeting, the revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved in [1]. As one of objective is NTN_testing_NGSO_channel_model with following

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel mode

According to the meeting agenda, and work plan in [2], this is the 1st meeting to discuss the UE demodulation performance requirements for NGSO for NR-NTN and IoT NTN in FR1-NTN bands and corresponding LTE bands based on the above channel model

In the last RAN4 meeting, the demodulation requirement for NTN testing was under discussion, the related agreements are captured in the WF [1] as following

Issue 1-2-1 UE assumption on propagator model for satellite motion for RAN4 simulation - Agreement - Following the agreement in the last meeting - Consider Keplerian model and equation of motion models in UE side as baseline assumption for RAN4 requirement - Note: There is no limitation for UE implementation to estimate the doppler shift and propagation delay based on the signalled ephemeris information Issue 1-2-2 Simulation Assumption for channel model - Agreement - Only the Rel-17 NTN fading channel models with timing-varying Doppler Shift and propagation delay is considered for this WI Issue 1-2-3 Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models - Proposals - Option 1: - Keep the same NR NTN PDSCH requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied. - Keep the same IoT NTN UE demodulation requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied. - Option 2: Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models - Option 3: - RAN4 shall start to agree parameters for using the NGSO channel model in order to understand whether the margin produced is acceptable to define requirements with. - RAN4 shall use the new NGSO channel model for all performance requirements, in it’s current state. - Option 4: After concluding discussions on the time-varying Doppler shift and propagation delay, initiate simulation campaigns to analyse the impact on demodulation requirements. Issue 1-2-4 FFS to revisit the UE assumption of residual doppler is 0.1ppm - Agreement - Keep the UE assumption of residual doppler is 0.1ppm Issue 1-2-5 Test cases for NR NTN with applying time varying Doppler Shift and propagation delay model - Agreement - Keep the UE assumption of residual doppler is 0.1ppm - Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2. - FFS on test 1-3 and 1-4 - Note: test 1-3 is related with UE capability to support 32 HARQ process, and test 1-4 is related with UE capability with HARQ disable Issue 1-2-6 Test Scope for IoT NTN with applying TE-emulated time vary doppler shift and propagation delay models - Agreement - No requirement for IoT NTN PUSCH with TE-emulated time vary doppler shift and propagation delay channel model Issue 1-2-7 Test case for NB-IoT NTN with applying the time-varying Doppler shift and propagation delay model - Agreement - Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.3.1.1 Test 1 - FFS If the LEO-600 model is not enough to cover the test time of Test 1, RAN4 should consider using the LEO-1200 model Issue 1-2-8 Test case for eMTC NTN with applying the time-varying Doppler shift and propagation delay model - Agreement - FFS to consider Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model - If considering Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model - Cover test 1 and test 2 according to TS 36.102 Clause 8.2.1.1 - FFS to cover test 3 Issue 1-2-9 Simulation Assumption for cellSpecificKoffset-r17 with applying the time-varying Doppler shift and propagation delay model - Agreement - Adopt the selected Rel-17 NTN test cases and their associated simulation assumptions as the baseline for this WI Issue 1-2-10 Simulation Assumption for IoT NTN PDSCH requirement - Agreement - Apply the time-varying Doppler shift and propagation delay model with LEO-600 for agreed test cases using the same parameters Issue 1-2-11 Simulation Assumption for IoT NTN NPUSCH requirement - Agreement - No requirement for IoT NTN PUSCH with TE-emulated time vary doppler shift and propagation delay channel model Issue 1-2-12 Applicability rule - Agreement - Add new applicability description which test cases use the time-varying Doppler shift and propagation delay model in Rel-19 TS 36.102 and TS 38.101-5 - Details of applicability rule are pending on the agreed test cases

In this contribution, the view on the demodulation requirement impact for NR NTN and IoT NTN based on TE-emulated channel model with varying Doppler and delay shifts was provided.

# 2	RAN4 demodulation requirement impact

Regarding the demodulation requirement, the requirements for NR PDSCH, eMTC PDSCH and NB-IoT are also defined based on the assumption that UE capable of pre-compensation of doppler shift and time delay. As specified in the TS 38.508, the NTN tests defined, corresponding to zero Doppler and constant delay for the type of satellite under test, keeping such information constant throughout the duration of each measurement in the test. Meanwhile, the EphemerisInfo-r17 values in SIB19 shall be periodically signalled to the UE during test based on the Ephemeris information in clause 5.6.2 (kept constant throughout the duration of each measurement in the test).

Observation 1: NTN tests defined, corresponding to zero Doppler and constant delay for the type of satellite under test, and EphemerisInfo-r17 values in SIB19 kept constant throughput the duration of each measurement in the test.

The UE Doppler and propagation delay estimation, and pre-compensation will be verified before the baseband processing, where the estimation is not reference signal based. After the baseband, only residual frequency offset error or time offset can be handled by reference signal, such as TRS for NR and CRS for LTE eMTC/NB-IoT. So, to check whether the existing NR NTN and IoT NTN can be applied, we see one possible way is to add some residual frequency offset and time offset with considering the estimation/pre-compensation errors. Similar as IoT over NTN for SAN requirement, we suggest to consider the fixed frequency offset and time offset during the test.

Regarding the value of frequency error and timing error, the worst case of 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s can be considered to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply, compared to ideally pre-compensated, according to core requirement. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

Table 7.1C.2-1: Te_NTN Timing Error Limit Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. FR1-NTN; 15; 15; 29*64*Tc. 30; 24*64*Tc. 60; N/A. 30; 15; 24*64*Tc. 30; 22*64*Tc. 60; N/A. Note 1: Tc is the basic timing unit defined in TS 38.211 [6].

Proposal 1: Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

Test cases for NR NTN

For NR NTN, the four cases were defined as following in Rel-17 considering different UE capability related with number of HARQ process, and HARQ disable.

Table 8.2.1.2.2.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 0.3 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | 7.6 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | -0.4 |
| 1-4 | R.PDSCH.1-1.1 FDD(1) | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 1.1 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

For test 1-3 is related with UE capability to support 32 HARQ process, and test 1-4 is related with UE capability with HARQ disable.  According to the WID, the number of demodulation performance requirements should be minimized based on specified TE emulate time varying delay and doppler shift. Considering test 1-3 and test 1-4 required UE additional capability. Therefore, we are fine to only consider apply the time-varying Doppler shift and propagation delay model for test 1-1 and test 1-2.

Proposal 2: Only apply the time-varying Doppler Shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2.

Test cases for IoT NTN

And for IoT NTN, RAN4 introduced the demodulation performance requirements for UE category M1, and UE category NB1 and NB2, as following

Table 8.2.1.1.1.1-2: Minimum performance for single antenna port (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 1.4MHz 16QAM 1/2 | R.1 FDD | OP.1 FDD | NTN-TDLC5-30 | 1x1 | 70 | 10.4 | M1 |
| 2 | 1.4MHz QPSK 1/3 | R.2 FDD | OP.1 FDD | NTN-TDLA100-200 | 1x1 | 70 | -4.2 | M1 |
| 3 | 1.4MHz QPSK 1/10 | R.3 FDD | OP.1 FDD | NTN-TDLA100-10 | 1x1 | 70 | -11.5 | M1 |

Table 8.3.1.1.1.1-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -4.7 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -10.6 | NB1, NB2 |

For NB-IoT, there are two cases. For test 1, based on RAN4 conformance test specification, the test time requires around 244second. According to the simulation time for LEO600 with 30 degrees elevation angle condition, it is around 250second. Considering 244 second is the minimum test time and to guarantee the coverage of simulation results under fading channel condition, the test time may exceed the 250 second. Considering that, we think it is necessary to use LEO-1200 model, if the LEO-600 model is not enough to cover the test time of Test 1, although the demodulation requirement is specified based on LEO-600 model.

Proposal 3: Only applying time varying channel model for NB-IoT test case 1, and if f the LEO-600 model is not enough to cover the test time of Test 1, RAN4 should consider the LEO-1200 for testing

For eMTC, although there is no specific test from RAN5, consider the test coverage in future for eMTC devices, we think the time varying Doppler shift and propagation delay model shall also applied for eMTC demodulation requirement. Regarding the applicable test cases. Considering the test 1-3 is targeting CE mode B and 64 repetition number required, we prefer to only consider test 1-1 and 1-2 for applying time varying Doppler shift and propagation delay model

Proposal 4: Only applying time varying channel model for eMTC case 1 and case 2, If the LEO-600 model is not enough to cover the test time of Test 1, RAN4 should consider the LEO-1200 for testing

# 3	Conclusion

In this contribution, the view on the demodulation requirement impact with varying Doppler and delay shifts

is provided.

Proposal 1: Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

Proposal 2: Only apply the time-varying Doppler Shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2.

Proposal 3: Only applying time varying channel model for NB-IoT test case 1, and if f the LEO-600 model is not enough to cover the test time of Test 1, RAN4 should consider the LEO-1200 for testing

Proposal 4: Only applying time varying channel model for eMTC case 1 and case 2, If the LEO-600 model is not enough to cover the test time of Test 1, RAN4 should consider the LEO-1200 for testing

# References

R4-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

R4- 2412554, Workplan for enhanced requirements and conductive test methodology, Samsung

R4-2504700, Way Forward for [114bis][323] NTN_testing_NGSO_channel_model_and_demod