3GPP TSG-RAN WG4 Meeting #115            	R4-2505973

Malta, MT, 19th-23th, May, 2025

Agenda item:	7.21.7

Source:	Samsung

Title:	View on demodulation requirement for AI based CSI prediction

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised Rel-19 WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved as following objective related with CSI prediction part [1]

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model) - Functionality-based LCM leveraged from other use cases, when necessary and applicable - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 - Core requirements for above two use cases for AI/ML LCM procedures and UE features [RAN4] - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. - Testability and interoperability [RAN4] - Finalize the testing framework and procedure for one-sided models and further analysis the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In the last RAN4#114bis meeting, CSI reporting requirement framework for CSI prediction was under discussed. The related agreed agreements are captured in the WF [1] as following

- CSI reporting requirement framework for CSI prediction - Codebook to be used in the tests - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions - Requirement baseline - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed - Test metric - Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/MLis 90 % of the maximum throughput obtained at SNRue_AI/ML using the precoders configured according to the UE reports - trnd is the throughput measured at SNRue_AI/ML with random precoding., - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1, follow2 in section 6.3 of 38.101-4

Meanwhile, in order to facilitate the AI/ML model evaluation and alignment, the initial simulation assumption is agreed and captured in the [2]

According to agenda, this is 1st meeting to discuss the performance part of AI/ML based prediction. In this contribution, the view on the test scope and general setup for demodulation requirement of AI/ML based prediction is provided.

# 2	Test scope

In general, the purpose of using the AI/ML based prediction is to acquire more accuracy channel state information. Based on the predicted CSI, the performance of CSI reporting is expected to be improved, including PMI reporting, RI reporting and CQI reporting. Considering the main targeting in RAN1 for AI/ML based CSI prediction design is for PMI reporting compared with existing non-AI based eType II Doppler codebook, and meanwhile, in the last RAN4 meeting, it was agreed that in Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined. Therefore, for UE demodulation and CSI reporting requirement, only PMI requirements based on AI/ML CSI prediction will be defined in Rel-19

# 3	Test setup for requirement

In this section, we provide our view on the general test setup for specifying the PMI requirement based on AI/ML CSI prediction.  In Rel-18, RAN4 defined the Rel-18 eType II doppler codebook requirement. The test parameters can be leveraged to specify PMI requirement based on AI/ML CSI prediction.  Meanwhile, currently RAN4 has agreed the initial simulation assumption for AI/ML CSI prediction model feasibility evaluation, proceeded step by step involving SGCS before and after PMI feedback, and Throughput. Based on output of AI/ML model evaluation and alignment, the test parameters can be further discussed.

Antenna configuration

In Rel-18 eType II doppler codebook requirement, considering the test complexity, only 16Tx performance was specified. In current feasibility evaluation, both 16Tx and 32Tx are involved for model alignment in terms of three steps. For requirement definition, similar legacy PMI test, we prefer to only select one of antenna configuration for requirement, pending on the feasibility evaluation of AI/ML model. For detail value of N1,N2, the following combination can be considered

- 32Tx port: (N1, N2)=(4,4)

- 16 Tx port: (N1, N2)=(4,2)

Then, based on existing PMI test for type I and Rel-18 eType II doppler codebook, the following CSI-RS configuration for each CSI-RS resource with 16TX port and 32TX port can be considered as starting point for requirement

For 32Tx

| NZP CSI-RS for CSI acquisition | Number of CSI-RS ports (X) | 32 |
| --- | --- | --- |
|  | CDM Type | CDM4 (FD2, TD2) |
|  | Density (ρ) | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) | Row 17,(5, 12) |

For 16Tx

|  | Number of CSI-RS ports (X) |  | 16 |
| --- | --- | --- | --- |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (5) |

Proposal 1:   Only one Tx antenna configuration will be selected for specifying the PMI requirement based on AI/ML prediction. The following Tx antenna configuration and associated CSI-RS configuration can be considered

For 32Tx

| NZP CSI-RS for CSI acquisition | Number of CSI-RS ports (X) | 32 |
| --- | --- | --- |
|  | CDM Type | CDM4 (FD2, TD2) |
|  | Density (ρ) | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) | Row 17,(5, 12) |

For 16Tx

|  | Number of CSI-RS ports (X) |  | 16 |
| --- | --- | --- | --- |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (5) |

Number of layers

For legacy PMI reporting requirement, considering the larger number of antennas deployed at gNB side, it is more typical to support multi-layer transmission. In existing PMI test cases, rank2 was considered

Proposal 2:   Consider rank 2 for specifying the PMI requirement based on AI/ML prediction

CSI-RS Configuration

The following is the RAN1 agreement as following

Agreement in RAN1#120 meeting For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused. - For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported. - For inference report, - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

For RAN1 evaluation for AI CSI, both periodic and Aperiodic CSI-RS configuration are considered.

Agreement in RAN1#116 For the evaluation of the AI/ML based CSI prediction, consider the following CSI-RS configuration - Periodic: 5ms periodicity (baseline), 20 ms periodicity (encouraged) - Aperiodic: Optional, CSI-RS burst with K resources and time interval m slots (based on R18 MIMO eType -II ) - Note: Companies to report observation window (number/distance) and prediction window Agreement in RAN1#116bis For the AI/ML based CSI prediction, adopt the following assumption as a baseline for evaluation purpose - UE speed: 30km/h, 60km/h - Others can be additionally submitted, e.g., 10km/h, 120km/h - Observation Window (number/distance): 5/5ms, 10/5ms - Others can be additionally submitted, e.g., 4/5ms, 15/5ms - Prediction window (number/distance between prediction instance/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms, 4/5ms/5ms - Others can be additionally submitted, e.g., 2/5ms/5ms, 3/5ms/5ms, 1/5ms/10ms - For other assumption, reuse Rel-18 baseline Agreement in RAN1#116bis For the evaluation of the AI/ML based CSI prediction, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted - Note: The same selected parameter combination shall be applied for benchmarks - CSI report periodicity: 5ms, 20ms (encouraged) - Other can be additionally submitted, e.g., 10ms Agreement For the UE-sided model based CSI prediction, for optional evaluation using AP CSI-RS, consider following assumption on observation window (number/distance) - Observation window: 12/2ms, 8/2ms, 4/2ms - Others can be additionally submitted

For AI/ML CSI prediction inference, both periodic and aperiodic are supported. Since the baseline is to use Rel-18 etype II doppler codebook for prediction, in case of aperiodic CSI-RS resources configured, the number of CSI-RS is expected to be configured as K= {4, 8,12}.  In currently Rel-18 etype II doppler codebook requirement, the aperiodic CSI-RS resources with K=4 is configured. Based on RAN1 evaluation, the baseline assumption is periodic configuration, and aperiodic configuration is optional. Therefore, to align with RAN1, we prefer to prioritize the PMI requirement with periodic configuration, FFS on aperiodic configuration for specifying PMI requirement

Proposal 3:   Prioritize to specify PMI requirement with periodic configuration.

CSI feedback overhead

In last meeting, some parameter combination (PC) values were proposed to facilitate the simulation for step 2 with following value

Agreement in RAN4#114bis CSI feedback overhead; Option1: Parameter combination = 2 (PC2) Option2: PC4 Option3: PC6 Option4: PC7 Note: this part relates to step2, may be down selected later.

For PC value, this parameter is related with parameter combination represented by by (L, pυ, β) for Rel-18 TypeII Doppler codebook as following table, where (*) indicates legacy Rel-16 eType II codebook combination. For these parameter combination,  L is the number of othogonal beams for each polarization, selected from a set of oversampled 2D DFT beams in spatial domain, pυ is configured to determine the number of DFT basis vectors in frequency domain, and  β is configured to determine the number of maximum linear combination coefficients.

Table 1: Codebook parameter configurations for L,  β and pυ

| paramCombination-Doppler-r18 | L | pυ |  | β |
| --- | --- | --- | --- | --- |
|  |  | υ ∈1,2 | υ ∈3,4 |  |
| 1 | 2 | 1/8 | 1/16 | ¼ |
| 2 (*) | 2 | ¼ | 1/8 | ½ |
| 3 (*) | 4 | ¼ | 1/8 | ¼ |
| 4 | 4 | ¼ | ¼ | ¼ |
| 5 | 4 | ¼ | ¼ | ½ |
| 6 (*) | 4 | ¼ | ¼ | ¾ |
| 7 (*) | 4 | ½ | ¼ | ½ |
| 8 (*) | 6 | ¼ | - | ½ |
| 9 (*) | 6 | ¼ | - | ¾ |

Based on RAN1 evaluation, for Rel-18 eType II Doppler codebook, PC 6 and PC 7 were considered as benchmarks value, and in existing RAN4 test, PC7 was used, where the PC7 is also same as the value when defining the requirements of Rel-16 Type II codebook.

Therefore, to align with RAN1 evaluation and existing RAN4 test, we would like to proposes set the PC7 (L=4, pυ=1/2, β=1/2) for specifying PMI requirement with AI/ML based CSI prediction

Proposal 4: RAN4 consider PC7 for specifying PMI requirement with AI/ML based CSI prediction

Other parameters

For other parameters, the parameters in the existing requirement for Rel-18 eTpye II Doppler Codebook can be reused as baseline

| Parameter | Value |
| --- | --- |
| Channel bandwidth and subcarrier spacing | For FDD, 10MHz/15kHz For TDD, 40MHz/30kHz |
| TDD DL-UL configuration | FR1.30-1 as specified in 38.101-4 Annex A. |
| Number of UE receiver antennas | 2 and 4 |
| R (numberOfPMI-SubbandsPerCQI-Subband-Doppler-r18) | 1 |
| Other Test parameters not mentioned above | For FDD 2Rx, Table 6.3.2.1.6-1 in 38.101-4 For TDD 2Rx, Table 6.3.2.2.6-1 in 38.101-4 For FDD 4Rx, Table 6.3.3.1.6-1 in 38.101-4 For TDD 4Rx, Table 6.3.3.2.6-1 in 38.101-4 |

Proposal 5: Reused the parameters in existing requirement for Rel-18 eTpye II Doppler Codebook can be reused as baseline for specifying PMI requirement with AI/ML CSI prediction.

| Parameter | Value |
| --- | --- |
| Channel bandwidth and subcarrier spacing | For FDD, 10MHz/15kHz For TDD, 40MHz/30kHz |
| TDD DL-UL configuration | FR1.30-1 as specified in 38.101-4 Annex A. |
| Number of UE receiver antennas | 2 and 4 |
| R (numberOfPMI-SubbandsPerCQI-Subband-Doppler-r18) | 1 |
| Other Test parameters not mentioned above | For FDD 2Rx, Table 6.3.2.1.6-1 in 38.101-4 For TDD 2Rx, Table 6.3.2.2.6-1 in 38.101-4 For FDD 4Rx, Table 6.3.3.1.6-1 in 38.101-4 For TDD 4Rx, Table 6.3.3.2.6-1 in 38.101-4 |

# 4	Conclusion

In this contribution, the view on test scope of demodulation requirement for AI/ML based CSI prediction is provided. Meanwhile, the general test setup for specifying PMI requirement is also provided

Proposal 1:   Only one Tx antenna configuration will be selected for specifying the PMI requirement based on AI/ML prediction. The following Tx antenna configuration and associated CSI-RS configuration can be considered

For 32Tx

| NZP CSI-RS for CSI acquisition | Number of CSI-RS ports (X) | 32 |
| --- | --- | --- |
|  | CDM Type | CDM4 (FD2, TD2) |
|  | Density (ρ) | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) | Row 17,(5, 12) |

For 16Tx

|  | Number of CSI-RS ports (X) |  | 16 |
| --- | --- | --- | --- |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (5) |

Proposal 2:  Consider rank 2 for specifying the PMI requirement based on AI/ML prediction

Proposal 3:  Prioritize to specify PMI requirement with periodic configuration.

Proposal 4: RAN4 consider PC7 for specifying PMI requirement with AI/ML based CSI prediction

Proposal 5: Reused the parameters in existing requirement for Rel-18 eTpye II Doppler Codebook can be reused as baseline for specifying PMI requirement with AI/ML CSI prediction.

| Parameter | Value |
| --- | --- |
| Channel bandwidth and subcarrier spacing | For FDD, 10MHz/15kHz For TDD, 40MHz/30kHz |
| TDD DL-UL configuration | FR1.30-1 as specified in 38.101-4 Annex A. |
| Number of UE receiver antennas | 2 and 4 |
| R (numberOfPMI-SubbandsPerCQI-Subband-Doppler-r18) | 1 |
| Other Test parameters not mentioned above | For FDD 2Rx, Table 6.3.2.1.6-1 in 38.101-4 For TDD 2Rx, Table 6.3.2.2.6-1 in 38.101-4 For FDD 4Rx, Table 6.3.3.1.6-1 in 38.101-4 For TDD 4Rx, Table 6.3.3.2.6-1 in 38.101-4 |

# References

R4-2502856, WF on AI/ML for NR air interface

R4-2505105, WF on AI/ML for NR Air Interface

R4-2505229, Updated simulation assumptions for AI/ML CSI prediction