3GPP TSG-RAN WG4 Meeting #115            	R4-2505975

Malta, MT, 19th-23th, May, 2024

Agenda item:	7.27.5

Source:	Samsung

Title:	View on BS demodulation requirement for NR NES

Document for:	Discussion

- Introduction

In the RAN#107 meeting, the revised WI was endorsed to support enhancements of network energy saving for NR with the following objectives in WID [1]

- Objective of Core part - Specify procedures and signalling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2 - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3 - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g., adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Objective of Performance part - Specify corresponding performance requirements and test cases for the network energy saving techniques

According to meeting agenda, this is the 1st meeting to discussion the performance part of enhancements of network energy savings for NR. In this contribution, the view on the test scope of BS demodulation requirement for NES is provided.

# 2	Test Scope

During the study in the SI phase [2], the network energy consumption model for the base station (BS) was defined including the reference configurations for FR1 TDD/FDD and FR2, the deep/light/micro sleep power states with corresponding relative power, transition time and energy consumption among different power states based on two types of BS categories, and the scaling rules for the active DL/UL power states considering BS power split by a static part of power and a dynamic part of power with the latter part reflecting the dynamic power consumption with respect to transmission/reception resource configurations in time, frequency, spatial and power domains.

As one of objective for Rel-19 NES, the adaptation of common signal/channel transmission was specified in RAN1. For PRACH adaptation in time domain, based on RAN1 design, the main targeting is to investigate the PRACH configuration with following adaptative mechanisms

Agreement in RAN1#116 - Adaptation based on configuration of additional/[different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - For additional PRACH resources - Adaptation of PRACH resource periodicity/PRACH occasion - Adaptation of PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle - Adaptation based on extending cell DRX operation for PRACH - Concentrating ROs in time domain - Other options are not precluded

Clearly, this objective is related with PRACH resources configuration for NES-capable UE, and signalling to trigger the adaptation for PRACH transmission. There is no new physical layer design for PRACH transmission. From the baseband perspective, there is no performance impact foreseen from BS side.

Observation 1: For adaptation of PRACH in time domain, the main impact is related as PRACH configuration and signalling design for NES-capable UE.

As the objective for UL WUS to support on-demand SIB1 for UEs in idle/inactive mode, from signal design perspective, the following was agreed

Agreement in RAN1#116 - For study of UL WUS design, consider at least PRACH as a starting point - FFS: Whether there is dedicated PRACH resource for SIB1 request - Other option(s) not precluded

And from UL WUS configuration perspective, the following was agreed

Agreement in RAN1#116bis - For UL WUS design for SIB1 request, at least dedicated PRACH resource is the assumption for further study in RAN1 - FFS: Details on time, frequency, and/or PRACH preamble resources for UL WUS - FFS: whether RACH resource for SIB1 request could be used for an initial access procedure and/or an on-demand SI procedure Agreement in RAN1#118 For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration: - rsrp-ThresholdSSB - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig Note: In legacy spec, the parameters above under the IE RACH-ConfigCommon

This objective is to relate with UL WUS design for SIB1 request for UEs in idle/inactive mode. UE will obtain the UL WUS configuration from Cell A, UE transmits UL WUS on NES cell, then UE receives on-demand SIB1 from NES Cell, the triggering method by UL WUS using PRACH. From signal design perspective for UL WUS, it is based on existing PRACH sequence. From UL WUS transmission configuration, the similar parameters of RACH configuration are included in the UL-WUS configuration. From the baseband perspective, existing PRACH requirement can meet for UL WUS transmission

Observation 2: From signal design perspective for UL WUS, it is based on existing PRACH sequence. From UL WUS transmission configuration, the similar parameters of RACH configuration are included in the UL-WUS configuration.

Based above analysis and observations, we think there is no BS demodulation requirement impact for supporting NES.

Proposal 1: No new BS demodulation requirements introduced with objective: “adaptation of common signal/channel transmissions”

Proposal 2: No new BS demodulation requirements introduced with objective: “support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA”

Proposal 3: No new BS demodulation requirements introduced with objective: “support for on-demand SIB1 for UEs in idle/inactive mode”

# 3	Conclusion

In this contribution, the view on the BS demodulation requirement with NES is provided.

Observation 1: For adaptation of PRACH in time domain, the main impact is related as PRACH configuration and signalling design for NES-capable UE.

Observation 2: From signal design perspective for UL WUS, it is based on existing PRACH sequence. From UL WUS transmission configuration, the similar parameters of RACH configuration are included in the UL-WUS configuration.

Proposal 1: No new BS demodulation requirements introduced with objective: “adaptation of common signal/channel transmissions”.

Proposal 2: No new BS demodulation requirements introduced with objective: “support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA”.

Proposal 3: No new BS demodulation requirements introduced with objective: “support for on-demand SIB1 for UEs in idle/inactive mode”.

# References

RP-250343, Revised WID on enhancements of network energy saving for NR, Ericsson, Apple