3GPP TSG-RAN WG4 Meeting #115           	R4-2505976

Malta, MT, 19th-23th, May, 2025

Agenda item:	7.25.6

Source:	Samsung

Title:	View on demodulation requirement for SBFD

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the WI was endorsed to support the evolution of NR duplex operation SBFD, which can be categorized as following

- Objective 1: semi-static indication of time location of SBFD subbands

- Objective 2: semi-static indication of frequency domain location of SBFD subbands mode

- Objective 3: SBFD operation to support random access in SBFD symbols

- Objective 4: UE transmission, reception and measurement behavior and procedures in SBFD symbols

- Objective 5: Enhancements for CLI handling

As stated in the WID,  RAN4 needs to specify the necessary performance requireemnts for enhancements listed above. According to meeting agenda, this is the 1st meeting to discuss the performance part of SBFD. In this contribution, the overall view on the test scope of demodulation requirement for SBFD is provided.

# 2	Overall RAN4 demodulation impact

2.1	Objective 1: semi-static indication of time location of SBFD subbands

The detailed objective 1 as following

- Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded

This objective is related with the semi-static indication of time location of SBFD subbands to UEs on DL BWP part. There is no dedicated demodulation requirement required for above objective and RAN4. And for specific demodulation requirement introduced in SBFD slots/symbols, RAN4 needs to follow the design from RAN1 for SBFD configuration and indication

Proposal 1: No new demodulation requirement required for objective “semi-static indication of time location of SBFD subbands”

2.2	Objective 2: semi-static indication of frequency location of SBFD subbands

The detailed objective 2 as following

- Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded

similar as objective 1, This objective is related with the semi-static indication of frequency domain location of SBFD subbands to UEs on DL BWP part. There is no dedicated demodulation requirement required for above objective and RAN4. And for specific demodulation requirement introduced in SBFD slots/symbols, RAN4 needs to follow the design from RAN1 for SBFD configuration and indication

Proposal 2: No new demodulation requirement required for objective “semi-static indication of frequency location of SBFD subbands”.

2.3	Objective 3: SBFD operation to support random access in SBFD symbols

The detailed objective 3 as following

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

RAN1 and RAN2 are discussing to support random access in SBFD symbols on UL sub-band for SBFD-aware UE with basic scope as following:

RRC state: support random access in SBFD symbols by UE in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode Support CBRA and CFRA in SBFD symbols. PRACH format: No new PRACH format is introduced RA type: 4 step RACH, 2 step FFS

RAN1 is introducing new RO configurations and resource mapping for SBFD slots and no RAN2 RACH procedure.

From RAN4 demodulation requirement for random access perspective, since there are no new formats introduced, and the existing PRACH requirements are generic enough which is agonistic to PRACH configurations specified in RAN1 TS 38.213. Therefore, the legacy PRACH test cases can be applied for SBFD operation at gNB.

Proposal 3: No new demodulation requirement required for objective “random access in SBFD symbols”. The legacy PRACH test cases will be applied SBFD operation at gNB.

2.4	Objective 4: UE transmission, reception and measurement behavior and procedures in SBFD symbols

The detailed objective 4 as following

- Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators

This objective can be further categorized with following areas:

- RAN1 is on discussion to specify the procedure and UE behaviour for DL reception and UL transmission scheduling across SBFD symbols/slot and non-SBFD symbols/slot including FH and repetition schemes.

This is major related to demodulation requirements from RAN4 perspective. Based on RAN1 design, the FH location, and actual repetition transmission will be impact across SBFD symbols and non-SBFD symbols.

- RAN1 is on discussion to specify CSI-RS configuration and CSI reporting for SBFD symbols/slot and non-SBFD symbols/slot.

It may have also impact on the CSI reporting requirement where the CSI-RS for SBFD symbols.

- RAN1 is on discussion on collision handling.

RAN1 conclusion is to reuse existing collision handling principles i.e., principles i.e. SSB is prioritized over than configured/scheduled UL transmission. There is no demodulation impact foreseen on the collision handling.

- RAN1 is on discussion on separate UL PC and/or spatial relation for SBFD slots and non-SBFD slots e.g. reusing multi-TRP repetition scheme framework (detailed framework will be decided by August RAN1 meeting)

This mainly has RF and RRM requirement impact, there is no demodulation requirement impact.

Proposal 4: For objective “UE transmission, reception and measurement behavior and procedures”, following potential demodulation impact foreseen:

- Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots

- FFS on Enhancement on resource allocation in frequency domain in SBFD symbols

2.5	Objective 5: Enhancements for CLI handling

The detailed objective 5 as following

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Note: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’ assessment of feasibility and specification impact - Note: Following are assumed for UL resource muting - No new DCI field/MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

For CLI handling, there are three subobjectives.

For the objective of Information exchange among gNBs, it can be categorized as following:

- Spatial schemes (FR1 gNB Tx beam nulling. FR2 gNB beam paring)

- RAN1 specify the required information exchange between gNBs e.g., measurement resource configuration

- Power domain schemes

- RAN1 concluded: UL Tx power control-based schemes are not considered in the down-selection of gNB-to-gNB CLI handling and UE-to-UE CLI handling schemes.

- gNB-to-gNB CLI measurement

- RAN1 specify the required information exchange between gNB

- RAN1 is discussion to support non-transparent UL resource muting for gNB-to-gNB CLI measurement and/or interference covariance matrix measurement

- Time/frequency coordination

- RAN1 specify the required information exchange between gNBs e.g., time/frequency location

Above enhancement is major related to RAN3 information exchange between gNBs. For non- transparent pant UL resource muting for gNB-to-gNB CLI, it may have performance impact for BS demodulation requirement

Similar as gNB side, UE to CLI schemes can be categorized as following:

- Spatial schemes for FR2

- RAN1 is on discussion to specify Rx beam configuration for UE-to-UE CLI measurement with separate optional UE capability

- Power domain schemes

- UL Tx power control-based schemes are not considered in the down-selection of gNB-to-gNB CLI handling and UE-to-UE CLI handling schemes

- UE-to-UE CLI measurement

- RAN1 is introducing L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework

- Measurement resource: Periodic, semi-persistent, or aperiodic i.e. SRS-RSRP resource or CLI-RSSI resources

- Measurement reporting: at least aperiodic reporting, new reporting quantities e.g. L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices; CSI framework update (timeline and priority rules)

- Measurement method still FFS with candidate options

- Time/frequency coordination

- RAN1 specify the required information exchange between gNBs e.g. time/frequency location

For UL to UE-CLI measurement, it is related with RRM requirement, there is no demodulation performance impact on the UE side.

For the objective of UL resource muting for PUSCH, it will impact the resource mapping and rate matching, as well as power boosting for REs in the symbol with UL resource muting. Clearly, the UL resource muting has the performance impact at the gNB side.

Proposal 5: For objective “Enhancements for CLI handling”, introduce PUSCH requirement for UL resource muting

# 3	Conclusion

In this contribution, the view on the demodulation scope for SBFD is provided.

Proposal 1: No new demodulation requirement required for objective “semi-static indication of time location of SBFD subbands”

Proposal 2: No new demodulation requirement required for objective “semi-static indication of frequency location of SBFD subbands”.

Proposal 3: No new demodulation requirement required for objective “random access in SBFD symbols”. The legacy PRACH test cases will be applied SBFD operation at gNB.

Proposal 4: For objective “UE transmission, reception and measurement behavior and procedures”, following potential demodulation impact foreseen:

- Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots

- FFS on Enhancement on resource allocation in frequency domain in SBFD symbols

Proposal 5: For objective “Enhancements for CLI handling”, introduce PUSCH requirement for UL resource muting

# References

RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, HiSilicon