3GPP TSG-RAN WG4 Meeting #115            	R4-2505977

Malta, MT, 19th-23th, May, 2024

Agenda item:	7.24.6

Source:	Samsung

Title:	View on BS demodulation requirement for MIMO with 3Tx

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WI was endorsed to support NR MIMO phase 5. One of objective related with uplink as following in WID [1]

- Objective of Core part - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource - Note: UL full power transmission mode 1 and mode 2 are not supported - Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching

According to meeting agenda, this is the 1st meeting to discussion the performance part of enhancements of network energy savings for NR. Based our analysis of accompanies paper for demodulation test scope for Rel-19 MIMO as following [2],

| Objectives | BS demodulation |
| --- | --- |
| Objective 1: UE-initiated/event-driven beam management | No impact |
| Objective 2: CSI enhancement up to 128 CSI-RS ports | No impact |
| Objective 3: UE reporting enhancement for CJT | No impact |
| Objective 4: 3-antenna-port codebook-based UL transmissions | 3Tx PUSCH demodulation requirements |
| Objective 5: Asymmetric DL sTRP/UL mTRP | No impact |

it is straightforward to introduce PUSCH demodulation performance requirements for UE which supporting 3Tx. In this contribution, the test setup for PUSCH requirement with 3Tx transmission is provided.

# 2	Test Setup

In Rel-15, RAN4 has introduced PUSCH requirements with UE supporting 1Tx and 2Tx scenarios. And PUSCH requirement with supporting 4Tx scenario targeting for FWA or CPE devices was introduced in Rel-18 NR FR1 WI. In general, the existing test configurations can be considered as reference for specifying PUSCH requirements with 3Tx transmission.

Duplex operation

From BS demodulation requirement aspect, there is no minor different between FDD and TDD. As agreed, the same requirements are applicable to FDD and TDD with different UL-DL patterns.

Proposal 1: RAN4 specify PUSCH requirement for both TDD and FDD. The same requirements can be applied for FDD and TDD with different UL-DL patterns

SCS and BW

In Rel-15, RAN4 introduced PUSCH with specifying most of SCS and CBW combination for FR1, where large effort for simulation results and results alignment are suffered. For 3Tx-3layer transmission verification, we prefer to select the minimum channel and maximum channel bandwidth per each SCS for specifying PUSCH requirement with 3Tx transmission.

Proposal 2: Consider the minimum channel bandwidth per each SCS for specifying PUSCH requirement with 3Tx transmission

- 15kHz SCS, 5MHz, 50MHz

- 30kHz SCS, 10MHz, 100MHz

Antenna configuration

For UL-2-layer transmission, different Rx configuration as 2Rx/4Rx/8Rx are considered with test applicable rule. For UL-3-layer transmission, it is straightforward to introduce PUSCH demodulation performance requirements with 4Rx and 8Rx.

Proposal 3: Consider 4Rx and 8Rx for specifying PUSCH requirement with 3Tx transmission

Number of layers

According to RAN1 agreement, to support non-codebook-based PUSCH transmission by a 3Tx UE, UE reports a capability of up to a maximum of 3	layers.  Although low layer can be supported by 3Tx transmission, how to select the proper non-codebook index is up to BS scheduling. In existing 4Tx transmission, only 4 layers transmission was considering when specifying the PUSCH requirement with 4Tx. Similar as 4Tx, only 3layers transmission is considered for requirement definition, the related TPMI index is configured as 0

Proposal 4: RAN4 only specify PUSCH requirement with UL 3 Layer transmission with 3Tx UE, and TPMI index is configured as 0

Number of DMRS

Since the 3Tx UE is not targeting for high-speed condition, similar as normal PUSCH demodulation requirement for 1Rx/2Rx/4Rx, only configured 2 DMRS should be enough.

Proposal 5: RAN4 only specify PUSCH requirement with 2 DMRS symbols

DMRS type

In Rel-15, only DMRS type 1 was considered. To facilitate the 3Tx transmission, both Rel-15 DMRS pattern and Rel-18 DMRS pattern can be supported. Rel-18 DMRS pattern is mainly targeting to support more UE Multiplexing for MU-MIMO scenarios. Since this WI is not targeting for MU-MIMO, we prefer to only consider Rel-15 DMRS pattern for requirement, the related DM-RS port is configured as {0,1,2}

Proposal 6: RAN4 only specify PUSCH requirement with Rel-15 DMRS pattern, the DM-RS port is {0,1,2}

Channel model and MCS

Similar as existing 4Tx requirement, we prefer to consider TDLB100-400, TDLA30-10 and TDLC300-100 for specifying the PUSCH requirement with 3Tx. For MCS, MCS 2, MCS 12 and MCS 17 can be considered as staring point.

Proposal 7: RAN4 only specify PUSCH requirement with covering TDLB100-400, TDLA30-10 and TDLC300-100 with following MCS combination

- TDLB100-400, MCS 2

- TDLA30-10, MCS 17

- TDLC300-100, MCS 12

Mapping type

For PUSCH mapping type, both mapping type A and mapping type B are supported. Both requirements are defined with test applicability for 1Rx/2Rx and 4Rx. Similar, we prefer to consider both mapping type A and mapping type B for specifying PUSCH requirement with 3Tx.

Proposal 8: RAN4 only specify PUSCH requirement with both mapping type A and type B

Other

For other parameters, the existing simulation assumption for 4Tx requirement can be reused as reference.

Proposal 9: The existing simulation assumption for 4Tx requirement can be reused as reference for specifying PUSCH requirements with 3Tx transmission

# 3	Conclusion

In this contribution, the view on the PUSCH demodulation requirement with 3Tx transmission is provided.

Proposal 1: RAN4 specify PUSCH requirement for both TDD and FDD. The same requirements can be applied for FDD and TDD with different UL-DL patterns

Proposal 2: Consider the minimum channel bandwidth per each SCS for specifying PUSCH requirement with 3Tx transmission

- 15kHz SCS, 5MHz, 50MHz

- 30kHz SCS, 10MHz, 100MHz

Proposal 3: Consider 4Rx and 8Rx for specifying PUSCH requirement with 3Tx transmission

Proposal 4: RAN4 only specify PUSCH requirement with UL 3 Layer transmission with 3Tx UE, and TPMI index is configured as 0

Proposal 5: RAN4 only specify PUSCH requirement with 2 DMRS symbols

Proposal 6: RAN4 only specify PUSCH requirement with Rel-15 DMRS pattern, the DM-RS port is {0,1,2}

Proposal 7: RAN4 only specify PUSCH requirement with covering TDLB100-400, TDLA30-10 and TDLC300-100 with following MCS combination

- TDLB100-400, MCS 2

- TDLA30-10, MCS 17

- TDLC300-100, MCS 12

Proposal 8: RAN4 only specify PUSCH requirement with both mapping type A and type B

Proposal 9: The existing simulation assumption for 4Tx requirement can be reused as reference for specifying PUSCH requirements with 3Tx transmission

# References

RP-242394, WID revision: NR MIMO Phase 5, Samsung

R4-2506681, RAN4 demodulation and CSI reporting performance work scope for NR MIMO phase 3, Samsung