3GPP RAN WG4 Meeting #115	R4-2505981

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title: 	Discussion on RRM core requirements testing framework for positioning

Source: 	CATT

Agenda item:	7.21.5

Document for:	Discussion

Introduction

Impacts of AI/ML based positioning on RRM requirements were discussed in last meeting and the agreements were captured in WF [1]. In this contribution, we will continue to discuss open issues summarized in [2] and analyse potential impacts on RRM core requirements.

Discussion

Prediction accuracy requirement for case 1

Whether to define prediction accuracy requirements for case 1 has been discussed for several meetings. Proposals brought up in last meeting for this issue are reproduced below:

Issue 3-1: Requirements for case 1 Proposals - Option 1: RAN4 will not define positioning accuracy requirements for case 1(direct UE positioning) - Why it is not feasible? - Option 2: RAN4 to further study how to define the requirements, companies are invited to bring concrete proposals on a requirements framework and next steps on how to define - Concrete proposals on how the requirements would be defined, how to determine the core/performance requirements and testability - At least reporting delay(including inference delay) requirements will be defined - Option 3: others - Recommended WF - Option 2 If no concrete proposals are shown in the next meeting, discussion should not continue.

The main issue is if it is feasible to define performance requirements like other performance requirements, i.e., is it possible to find a relation between the absolute position error and side conditions, e.g., SINR, bandwidth of reference signal, etc. Based on our simulation observations, the position error spans from less than 1 meter to a few meters in different InF scenarios with generalization considered. If we consider more diverse scenarios, such as Uma/Umi, range of position error may be wider. It seems not feasible to define a general performance requirement that can apply for diverse scenarios. Therefore we prefer to follow the legacy rule which is not to define performance requirements for UE-sided direct positioning.

Proposal 1: RAN4 not to define performance requirements for case 1.

Sample based timing measurements for case 3b

Another open issue is whether/how to define reporting requirement for sample-based timing measurement for case 3b.

Issue 3-5: Sample based timing measurements Proposals - Option 1: Introduce sample based timing measurement report mapping as follows: Proposal 3: Reporting range for sample-based timing measurements is from -985024´Tc to 985024´Tc. Proposal 4: Table 1 is used to define measurement report mapping table for sample-based timing measurement reporting for reporting granularity corresponding to k = 0. Proposal 5: Table 2 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 1. Proposal 6: Table 3 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 2. Proposal 7: Table 4 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 3. Proposal 8: Table 5 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 4. Proposal 9: Table 6 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 5. - Option 2: Introduce other report mapping - Recommended WF - To be further discussed

Till RAN1#120bis meeting, RAN1 have reached sufficient agreements [3][4][5][6]:

Agreement - RAN1#119: For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified Agreement – RAN1#120: For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0...5} Agreement – RAN1#120: For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting: - Nt' = {8, 16, 24} Agreement – RAN1#116bis: For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Agreement – RAN1#118: For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point.

Based on the above RAN1 agreements, RAN4 can define reporting requirements for sample-based timing measurement. Furthermore, considering that the timing information is represented relative to the existing UL RTOA reference time T0+tSRS and the timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc and k = {0...5} which is the same as that for UL RTOA reporting requirement. Therefore RAN4 can reuse the UL RTOA reporting requirement for sample-based timing measurement.

Regarding the power information associated to the Nt' values reported by gNB, RAN1 agreed to use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point in RAN1#118. Therefore RAN4 can wait for RAN1 progress before discussing how to define the reporting requirement.

Proposal 2: RAN4 to reuse the UL RTOA reporting requirement for sample-based timing measurement.

Proposal 3: RAN4 to wait for RAN1 progress before discussing how to define the reporting requirement for sample-based timing measurement.

Conclusions

This contribution discussed the potential impacts of AI/ML enabled positioning accuracy enhancement on RRM requirements and following proposals are provided:

Proposal 1: RAN4 not to define performance requirements for case 1.

Proposal 2: RAN4 to reuse the UL RTOA reporting requirement for sample-based timing measurement.

Proposal 3: RAN4 to wait for RAN1 progress before discussing how to define the reporting requirement for sample-based timing measurement.

References

[1] R4-2505105, WF on the requirements for AI/ML air interface_part1, Qualcomm, RAN4#114bis.

[2] R4-2504684, Topic summary for [114bis][126] NR_AIML_air_part1, Qualcomm, RAN4#114bis.

[3] Chair Notes RAN1#116bis, RAN1#116bis.

[4] Chair Notes RAN1#118, RAN1#118.

[5] Chair Notes RAN1#119, RAN1#119.

[6] Chair Notes RAN1#120, RAN1#120.